| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0287r1 |
| --- | --- |

| Title: | CE6: Compound Orthonormal Transform (Test 6.1.7 f/g) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the test results of CE6-1.7 f/g on a Compound Orthonormal Transform, which embeds 16-point and 32-point DST-7/DCT-8 into 64-point DCT-2. With the proposed method, it is reported that all the transform types used in VVC can be extracted from one single 64x64 matrix so that 33% transform core storage, i.e., 3.4 KBytes memory, can be saved when compared to VTM-2.0.1. It is reportedly shown that, on top of VTM-2.0.1, average -0.05%, 0.09% and 0.08% luma BD-Rate is achieved under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively. When Multiple Transform Selection (MTS) is also enabled for inter prediction residuals, average -0.05%, 0.03% and -0.07% luma BD-Rate is achieved under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively. There is almost no run-time impact by using the proposed method.

# Introduction

In MTS as described in VVC Draft 2 [1], besides DCT-II, two additional primary transform types, including DST-7 and DCT-8 are used. The transform size of DST7 and DCT-8 applicable for 4-point, 8-point, 16-point and 32-point. Separate transform cores require additional memory for storing the transform cores, and totally 3.4 Kbytes are needed. In JVET-K0291, a method of embedding 16-point and 32-point DST-7 and DCT-8 into 64-point DCT-2 are proposed. In this contribution, the methods proposed in JVET-K0292 are further tested on top of VTM-2.0.1.

# Proposed method

A 64-point primary transform core, which is called Compound Orthonormal Transform (COT) core, is proposed as shown below (only the left half of the transform matrix is shown, the right half can be derived according to the symmetry/anti- symmetry property of DCT-2 core), where

{aa, ab, ac, ad, ae, af, ag, ah, ai, aj, ak, al, am, an, ao, ap, aq, ar, as, at, au, av, aw, ax, ay, az, ba, bb, bc, bd, be, bf, bg, bh, bi, bj, bk, bl, bm, bn, bo, bp, bq, br, bs, bt, bu, bv, bw, bx, by, bz, ca, cb, cc, cd, ce, cf, cg, ch, ci, cj, ck} =

{ 256,  334,  139,  355,  301,  201,   71, 360,  346,  319,  280,  230,  171,  105,   35,  356,  353,  346,  337,  324,  309,  290,  269,  246,  220,  193,  163,  133,  100,   67,   34, 359,  358,  357,  354,  351,  347,  342,  336,  329,  322,  314,  305,  296,  285,  275,  263,  251,  238,  225,  211,  197,  182,  167,  151,  135,  119,  103,   86,   69,   52,   35,   17 }.

With the above proposed 64-point COT core, the 2-point, 4-point, 8-point 16-point DCT-2 and 32-point COT can be extracted from the even rows, and the 4-point, 8-point DST-4/DCT-4, 16-point and 32-point DST-7/DCT-8 can be extracted the odd rows.

# Results

## CE6-1.7f and CE6-1.7g

In this section, the results of two CE tests, including CE6-1.7f and CE6-1.7g, are reported. Based on the CE6 description, two tests are defined related to the proposed method, as described below table,

Table 1: Descriptions on CE6 Test 6.1.7f and 6.1.7g

| Test # | Description |
| --- | --- |
| 6.1.7-f | 32- and 64-point DCT-2 replaced by COT AMT intra: DST-7 / DCT-8 AMT inter: DCT-8 / DST-7 Max AMT size: 32 |
| 6.1.7-g | 32- and 64-point DCT-2 replaced by COT 16-point and 32-point: AMT intra: DST-7 / DCT-8 AMT inter: DCT-8 / DST-7 4-point and 8-point: AMT intra: DST-4 / DCT-4 AMT inter: DCT-4 / DST-4 Max AMT size: 32 |

For each of the above test, two experiments are conducted. The first test reports the coding performance under CTC condition, as shown in Table 2 and 4. The second test reports the coding performance under CTC condition with inter MTS enabled, as shown in Table 3 and 5.

Table 2: Coding performance of Test 6.1.7f

Table 3: Coding performance of Test 6.1.7f (Inter MTS is enabled)

Table 4: Coding performance of Test 6.1.7g

Table 5: Coding performance of Test 6.1.7g (Inter MTS is enabled)

## Combination of CE6-1.3 and CE6-1.7g

In this section, the results of combination of CE6-1.3 and CE6-1.7g are also reported. With this combined test, the transform cores of all primary transform can be derived from one single 64x64 matrix which is represented by 8-bit elements.

Table 4: Coding performance of combined test of CE6-1.3 and CE6-1.7g

Table 5: Coding performance of combined test of CE6-1.3 and CE6-1.7g (Inter MTS is enabled)

# Complexity Analysis

## Memory requirements for storing transform cores

Comparisons on the memory requirement for storing primary transform cores are described in Table 6. Detailed calculation on the storage cost of MTS is described in Table 7.

Table 6: Memory requirements for storing memory transform cores

|  | DCT-2 | MTS | Total |
| --- | --- | --- | --- |
| VTM-2.0.1 | 6.82 KB | 3.4 KB | 10.22 KB |
| CE6-1.7f | 6.82 KB | 0.2 KB | 7.02KB |
| CE6-1.7g | 6.82 KB | 0 KB | 6.82 KB |

Table 6: Memory requirements for storing memory transform cores of MTS

| Transform size | Number of elements | Number of transform types | Bits per element | Total |
| --- | --- | --- | --- | --- |
| 4-point | 16 | 2 | 10 bit | 320 bits |
| 8-point | 64 | 2 | 10 bit | 1280 bits |
| 16-point | 256 | 2 | 10 bit | 5120 bits |
| 32-point | 1024 | 2 | 10 bit | 20480 bits |
| All |  |  |  | 3.4 KB |

## Other metrics

| Metrics | Reported results |
| --- | --- |
| Operation counts | Unchanged |
| Minimum bit-precision | Unchanged |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output | No |
| Other operations and memory requirements relevant to the proposed tool | No |
| Encoder and decoder times obtained on the same computing platform for both reference and proposed tools should be provided | Reported in Section 3 |
| List of all combinations of transforms block size and transform type used for secondary transformation. | Same with VTM-2.0.1 |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms | Implementation and arithmetic commonality unchanged |
| If the proposal requires additional computations at the encoder or decoder | No |

# Proposed text changes

The following proposed text modifications are based on JVET-K1001-v5. The added texts are highlighted in green, and deleted texts are marked by red strikethrough.

8.4.4.2	Transformation process

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of scaled transform coefficients,

- a list of scaled transform coefficients x[ j ] with j = 0..nTbS − 1,

- a transform kernel type variable trType.

Output of this process is the list of transformed samples y[ i ] with i = 0..nTbS − 1.

The transformation matrix derivation process as specified in clause 8.4.4.3 in invoced with the transform size nTbS and the transform kernel Type trType as input, and the transformation maxtrix transMatrix as output.

Depending on the value of trType, the following applies:, the list of transformed samples y[ i ] with i = 0..nTbS − 1 is derived as follows:

- If trType is equal to 0, the following transform matrix multiplication applies:

y[i]= j=0nTbs-1transMatrix[i][j*26-Log2(nTbS) ]*x[j]  with i = 0..nTbS − 1	(8-390)

- Otherwise, if(trType is equal to 1 or trType is equal to 2), the following transform matrix multiplication applies:

y[i]= j=0nTbs-1transMatrix[i][(2*j+1)*25-Log2nTbS]*x[j]  with i = 0..nTbS − 1	(8-391)

- Otherwise (trType is equal to 2), the following transform matrix multiplication applies:

y[i]= j=0nTbs-1transMatrix[i+nTbs][(2*j+1)*25-Log2nTbS]*-1j+1*x[j]  with i = 0..nTbS − 1	(8-392)

8.4.4.3	Transformation matrix derivation process

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of scaled transform coefficients,

- the transformation kernel type trType.

Output of this process is the transformation matrix transMatrix.

The transformation matrix transMatrix is derived based on trType and nTbs as follows:

- If trType is equal to 0, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..63	(8-392)

transMatrixCol0to15 =	(8-393)

{

{ 256  256  256  256  256  256  256  256  256  256  256  256  256  256  256  256 }

{ 362  361  359  357  353  349  344  338  331  323  315  306  296  285  274  262 }

{ 362  358  351  341  327  311  291  268  243  216  186  155  122   88   53   18 }

{ 361  353  338  315  285  250  208  163  114   62    9  −44  −97 −147 −194 −236 }

{ 359  358  357  354  351  347  342  336  329  322  314  305  296  285  275  263 }

{ 356  353  346  337  324  309  290  269  246  220  193  163  133  100   67   34 }

{ 358  351  336  314  285  251  211  167  119   69   17  -35  -86 -135 -182 -225 }

{ 360  346  319  280  230  171  105   35  −35 −105 −171 −230 −280 −319 −346 −360 }

{ 359  338  296  236  163   79   −9  −97 −178 −250 −306 −344 −361 −357 −331 −285 }

{ 358  327  268  186   88  −18 −122 −216 −291 −341 −362 −351 −311 −243 −155  −53 }

{ 357  315  236  130    9 −114 −223 −306 −353 −359 −323 −250 −147  −27   97  208 }

{ 357  336  296  238  167   86    0  -86 -167 -238 -296 -336 -357 -357 -336 -296 }

{ 353  324  269  193  100    0 -100 -193 -269 -324 -353 -353 -324 -269 -193 -100 }

{ 354  314  238  135   17 -103 -211 -296 -347 -358 -329 -263 -167  -52   69  182 }

{ 355  301  201   71  −71 −201 −301 −355 −355 −301 −201  −71   71  201  301  355 }

{ 353  285  163    9 −147 −274 −349 −357 −296 −178  −27  130  262  344  359  306 }

{ 351  268  122  −53 −216 −327 −362 −311 −186  −18  155  291  358  341  243   88 }

{ 349  250   79 −114 −274 −357 −338 −223  −44  147  296  361  323  194    9 −178 }

{ 351  285  167   17 -135 -263 -342 -357 -305 -197  -52  103  238  329  359  322 }

{ 346  269  133  -34 -193 -309 -356 -324 -220  -67  100  246  337  353  290  163 }

{ 347  251   86 -103 -263 -351 -342 -238  -69  119  275  354  336  225   52 -135 }

{ 346  230   35 −171 −319 −360 −280 −105  105  280  360  319  171  −35 −230 −346 }

{ 344  208   −9 −223 −349 −338 −194   27  236  353  331  178  −44 −250 −357 −323 }

{ 341  186  −53 −268 −362 −291  −88  155  327  351  216  −18 −243 −358 −311 −122 }

{ 338  163  −97 −306 −357 −223   27  262  362  274   44 −208 −353 −315 −114  147 }

{ 342  211    0 -211 -342 -342 -211    0  211  342  342  211    0 -211 -342 -342 }

{ 337  193  -34 -246 -353 -309 -133  100  290  356  269   67 -163 -324 -346 -220 }

{ 336  167  -86 -296 -357 -238    0  238  357  296   86 -167 -336 -336 -167   86 }

{ 334  139 −139 −334 −334 −139  139  334  334  139 −139 −334 −334 −139  139  334 }

{ 331  114 −178 −353 −296  −44  236  362  250  −27 −285 −357 −194   97  323  338 }

{ 327   88 −216 −362 −243   53  311  341  122 −186 −358 −268   18  291  351  155 }

{ 323   62 −250 −359 −178  147  353  274  −27 −306 −338  −97  223  362  208 −114 }

{ 329  119 -167 -347 -305  -69  211  357  275   17 -251 -359 -238   35  285  354 }

{ 324  100 -193 -353 -269    0  269  353  193 -100 -324 -324 -100  193  353  269 }

{ 322   69 -238 -358 -197  119  342  296   17 -275 -351 -151  167  354  263  -35 }

{ 319   35 −280 −346 −105  230  360  171 −171 −360 −230  105  346  280  −35 −319 }

{ 315    9 −306 −323  −27  296  331   44 −285 −338  −62  274  344   79 −262 −349 }

{ 311  −18 −327 −291   53  341  268  −88 −351 −243  122  358  216 −155 −362 −186 }

{ 306  −44 −344 −250  130  361  178 −208 −357  −97  274  331    9 −323 −285   79 }

{ 314   17 -296 -329  -52  275  342   86 -251 -351 -119  225  357  151 -197 -359 }

{ 309    0 -309 -309    0  309  309    0 -309 -309    0  309  309    0 -309 -309 }

{ 305  -35 -336 -263  103  354  211 -167 -359 -151  225  351   86 -275 -329  -17 }

{ 301  −71 −355 −201  201  355   71 −301 −301   71  355  201 −201 −355  −71  301 }

{ 296  −97 −361 −147  262  323  −44 −353 −194  223  344    9 −338 −236  178  357 }

{ 291 −122 −362  −88  311  268 −155 −358  −53  327  243 −186 −351  −18  341  216 }

{ 285 −147 −357  −27  344  194 −250 −315   97  362   79 −323 −236  208  338  −44 }

{ 296  -86 -357 -167  238  336    0 -336 -238  167  357   86 -296 -296   86  357 }

{ 290 -100 -356 -133  269  309  -67 -353 -163  246  324  -34 -346 -193  220  337 }

{ 285 -135 -357  -52  329  225 -211 -336   35  354  151 -275 -296  119  358   69 }

{ 280 −171 −346   35  360  105 −319 −230  230  319 −105 −360  −35  346  171 −280 }

{ 274 −194 −331   97  359    9 −357 −114  323  208 −262 −285  178  338  −79 −361 }

{ 268 −216 −311  155  341  −88 −358   18  362   53 −351 −122  327  186 −291 −243 }

{ 262 −236 −285  208  306 −178 −323  147  338 −114 −349   79  357  −44 −361    9 }

{ 275 -182 -336   69  359   52 -342 -167  285  263 -197 -329   86  358   35 -347 }

{ 269 -193 -324  100  353    0 -353 -100  324  193 -269 -269  193  324 -100 -353 }

{ 263 -225 -296  182  322 -135 -342   86  354  -35 -359  -17  357   69 -347 -119 }

{ 256 −256 −256  256  256 −256 −256  256  256 −256 −256  256  256 −256 −256  256 }

{ 250 −274 −223  296  194 −315 −163  331  130 −344  −97  353   62 −359  −27  362 }

{ 243 −291 −186  327  122 −351  −53  362  −18 −358   88  341 −155 −311  216  268 }

{ 236 −306 −147  349   44 −362   62  344 −163 −296  250  223 −315 −130  353   27 }

{ 251 -263 -238  275  225 -285 -211  296  197 -305 -182  314  167 -322 -151  329 }

{ 246 -269 -220  290  193 -309 -163  324  133 -337 -100  346   67 -353  -34  356 }

{ 238 -296 -167  336   86 -357    0  357  -86 -336  167  296 -238 -238  296  167 }

{ 230 −319 −105  360  −35 −346  171  280 −280 −171  346   35 −360  105  319 −230 }

{ 223 −331  −62  361 −114 −306  262  178 −349   −9  353 −163 −274  296  130 −359 }

{ 216 −341  −18  351 −186 −243  327   53 −358  155  268 −311  −88  362 −122 −291 }

{ 208 −349   27  331 −250 −163  359  −79 −306  285  114 −362  130  274 −315  −62 }

{ 225 -322  -86  359  -69 -329  211  238 -314 -103  358  -52 -336  197  251 -305 }

{ 220 -324  -67  356 -100 -309  246  193 -337  -34  353 -133 -290  269  163 -346 }

{ 211 -342    0  342 -211 -211  342    0 -342  211  211 -342    0  342 -211 -211 }

{ 201 −355   71  301 −301  −71  355 −201 −201  355  −71 −301  301   71 −355  201 }

{ 194 −359  114  262 −338   27  315 −296  −62  349 −236 −147  362 −163 −223  353 }

{ 186 −362  155  216 −358  122  243 −351   88  268 −341   53  291 −327   18  311 }

{ 178 −362  194  163 −361  208  147 −359  223  130 −357  236  114 −353  250   97 }

{ 197 -354   86  285 -314  -35  342 -238 -151  359 -135 -251  336  -17 -322  275 }

{ 193 -353  100  269 -324    0  324 -269 -100  353 -193 -193  353 -100 -269  324 }

{ 182 -359  167  197 -358  151  211 -357  135  225 -354  119  238 -351  103  251 }

{ 171 −360  230  105 −346  280   35 −319  319  −35 −280  346 −105 −230  360 −171 }

{ 163 −357  262   44 −315  331  −79 −236  361 −194 −130  349 −285   −9  296 −344 }

{ 155 −351  291  −18 −268  358 −186 −122  341 −311   53  243 −362  216   88 −327 }

{ 147 −344  315  −79 −208  359 −274    9  262 −361  223   62 −306  349 −163 −130 }

{ 167 -357  238   86 -336  296    0 -296  336  -86 -238  357 -167 -167  357 -238 }

{ 163 -353  246   67 -324  309  -34 -269  346 -133 -193  356 -220 -100  337 -290 }

{ 151 -347  296  -35 -251  358 -211  -86  322 -329  103  197 -357  263   17 -285 }

{ 139 −334  334 −139 −139  334 −334  139  139 −334  334 −139 −139  334 −334  139 }

{ 130 −323  349 −194  −62  285 −361  250   −9 −236  359 −296   79  178 −344  331 }

{ 122 −311  358 −243   18  216 −351  327 −155  −88  291 −362  268  −53 −186  341 }

{ 114 −296  362 −285   97  130 −306  361 −274   79  147 −315  359 −262   62  163 }

{ 135 -329  336 -151 -119  322 -342  167  103 -314  347 -182  -86  305 -351  197 }

{ 133 -324  337 -163 -100  309 -346  193   67 -290  353 -220  -34  269 -356  246 }

{ 119 -305  357 -251   35  197 -342  336 -182  -52  263 -358  296 -103 -135  314 }

{ 105 −280  360 −319  171   35 −230  346 −346  230  −35 −171  319 −360  280 −105 }

{  97 −262  353 −344  236  −62 −130  285 −359  331 −208   27  163 −306  362 −315 }

{  88 −243  341 −358  291 −155  −18  186 −311  362 −327  216  −53 −122  268 −351 }

{  79 −223  323 −362  331 −236   97   62 −208  315 −361  338 −250  114   44 −194 }

{ 103 -275  357 -322  182   17 -211  336 -351  251  -69 -135  296 -359  305 -151 }

{ 100 -269  353 -324  193    0 -193  324 -353  269 -100 -100  269 -353  324 -193 }

{  86 -238  336 -357  296 -167    0  167 -296  357 -336  238  -86  -86  238 -336 }

{  71 −201  301 −355  355 −301  201  −71  −71  201 −301  355 −355  301 −201   71 }

{  62 −178  274 −338  362 −344  285 −194   79   44 −163  262 −331  361 −349  296 }

{  53 −155  243 −311  351 −362  341 −291  216 −122   18   88 −186  268 −327  358 }

{  44 −130  208 −274  323 −353  362 −349  315 −262  194 −114   27   62 −147  223 }

{  69 -197  296 -351  354 -305  211  -86  -52  182 -285  347 -357  314 -225  103 }

{  67 -193  290 -346  353 -309  220 -100  -34  163 -269  337 -356  324 -246  133 }

{  52 -151  238 -305  347 -359  342 -296  225 -135   35   69 -167  251 -314  351 }

{  35 −105  171 −230  280 −319  346 −360  360 −346  319 −280  230 −171  105  −35 }

{  27  −79  130 −178  223 −262  296 −323  344 −357  362 −359  349 −331  306 −274 }

{  18  −53   88 −122  155 −186  216 −243  268 −291  311 −327  341 −351  358 −362 }

{   9  −27   44  −62   79  −97  114 −130  147 −163  178 −194  208 −223  236 −250 }

{  35 -103  167 -225  275 -314  342 -357  358 -347  322 -285  238 -182  119  -52 }

{  34 -100  163 -220  269 -309  337 -353  356 -346  324 -290  246 -193  133  -67 }

{  17  -52   86 -119  151 -182  211 -238  263 -285  305 -322  336 -347  354 -358 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..63	(8-394)

transMatrixCol16to31 =	(8-395)

{

{ 256  256  256  256  256  256  256  256  256  256  256  256  256  256  256  256 }

{ 250  236  223  208  194  178  163  147  130  114   97   79   62   44   27    9 }

{ −18  −53  −88 −122 −155 −186 −216 −243 −268 −291 −311 −327 −341 −351 −358 −362 }

{−274 −306 −331 −349 −359 −362 −357 −344 −323 −296 −262 −223 −178 −130  −79  −27 }

{ 251  238  225  211  197  182  167  151  135  119  103   86   69   52   35   17 }

{ -34  -67 -100 -133 -163 -193 -220 -246 -269 -290 -309 -324 -337 -346 -353 -356 }

{-263 -296 -322 -342 -354 -359 -357 -347 -329 -305 -275 -238 -197 -151 -103  -52 }

{−360 −346 −319 −280 −230 −171 −105  −35   35  105  171  230  280  319  346  360 }

{−223 −147  −62   27  114  194  262  315  349  362  353  323  274  208  130   44 }

{  53  155  243  311  351  362  341  291  216  122   18  −88 −186 −268 −327 −358 }

{ 296  349  361  331  262  163   44  −79 −194 −285 −344 −362 −338 −274 −178  −62 }

{-238 -167  -86    0   86  167  238  296  336  357  357  336  296  238  167   86 }

{ 100  193  269  324  353  353  324  269  193  100    0 -100 -193 -269 -324 -353 }

{ 275  336  359  342  285  197   86  -35 -151 -251 -322 -357 -351 -305 -225 -119 }

{ 355  301  201   71  −71 −201 −301 −355 −355 −301 −201  −71   71  201  301  355 }

{ 194   44 −114 −250 −338 −361 −315 −208  −62   97  236  331  362  323  223   79 }

{ −88 −243 −341 −358 −291 −155   18  186  311  362  327  216   53 −122 −268 −351 }

{−315 −362 −306 −163   27  208  331  359  285  130  −62 −236 −344 −353 −262  −97 }

{ 225   86  -69 -211 -314 -358 -336 -251 -119   35  182  296  354  347  275  151 }

{-163 -290 -353 -337 -246 -100   67  220  324  356  309  193   34 -133 -269 -346 }

{-285 -357 -329 -211  -35  151  296  358  322  197   17 -167 -305 -359 -314 -182 }

{−346 −230  −35  171  319  360  280  105 −105 −280 −360 −319 −171   35  230  346 }

{−163   62  262  359  315  147  −79 −274 −361 −306 −130   97  285  362  296  114 }

{ 122  311  358  243   18 −216 −351 −327 −155   88  291  362  268   53 −186 −341 }

{ 331  344  178  −79 −296 −359 −236    9  250  361  285   62 −194 −349 −323 −130 }

{-211    0  211  342  342  211    0 -211 -342 -342 -211    0  211  342  342  211 }

{ 220  346  324  163  -67 -269 -356 -290 -100  133  309  353  246   34 -193 -337 }

{ 296  357  238    0 -238 -357 -296  -86  167  336  336  167  -86 -296 -357 -238 }

{ 334  139 −139 −334 −334 −139  139  334  334  139 −139 −334 −334 −139  139  334 }

{ 130 −163 −349 −306  −62  223  361  262   −9 −274 −359 −208   79  315  344  147 }

{−155 −351 −291  −18  268  358  186 −122 −341 −311  −53  243  362  216  −88 −327 }

{−344 −296   −9  285  349  130 −194 −361 −236   79  331  315   44 −262 −357 −163 }

{ 197  -86 -314 -342 -151  135  336  322  103 -182 -351 -296  -52  225  358  263 }

{-269 -353 -193  100  324  324  100 -193 -353 -269    0  269  353  193 -100 -324 }

{-305 -336 -103  211  359  225  -86 -329 -314  -52  251  357  182 -135 -347 -285 }

{−319  −35  280  346  105 −230 −360 −171  171  360  230 −105 −346 −280   35  319 }

{ −97  250  353  114 −236 −357 −130  223  359  147 −208 −361 −163  194  362  178 }

{ 186  362  155 −216 −358 −122  243  351   88 −268 −341  −53  291  327   18 −311 }

{ 353  223 −163 −362 −147  236  349   62 −296 −315   27  338  262 −114 −359 −194 }

{-182  167  358  211 -135 -354 -238  103  347  263  -69 -336 -285   35  322  305 }

{ 309  309    0 -309 -309    0  309  309    0 -309 -309    0  309  309    0 -309 }

{ 314  296  -52 -342 -251  119  357  197 -182 -358 -135  238  347   69 -285 -322 }

{ 301  −71 −355 −201  201  355   71 −301 −301   71  355  201 −201 −355  −71  301 }

{  62 −315 −274  130  362  114 −285 −306   79  359  163 −250 −331   27  349  208 }

{−216 −341   18  351  186 −243 −327   53  358  155 −268 −311   88  362  122 −291 }

{−359 −130  296  274 −163 −353   −9  349  178 −262 −306  114  361   62 −331 −223 }

{ 167 -238 -336    0  336  238 -167 -357  -86  296  296  -86 -357 -167  238  336 }

{-337 -220  193  346   34 -324 -246  163  353   67 -309 -269  133  356  100 -290 }

{-322 -238  197  342  -17 -351 -167  263  305 -103 -359  -86  314  251 -182 -347 }

{−280  171  346  −35 −360 −105  319  230 −230 −319  105  360   35 −346 −171  280 }

{ −27  353  130 −315 −223  250  296 −163 −344   62  362   44 −349 −147  306  236 }

{ 243  291 −186 −327  122  351  −53 −362  −18  358   88 −341 −155  311  216 −268 }

{ 362   27 −359  −62  353   97 −344 −130  331  163 −315 −194  296  223 −274 −250 }

{-151  296  251 -211 -322  103  357   17 -351 -135  305  238 -225 -314  119  354 }

{ 353  100 -324 -193  269  269 -193 -324  100  353    0 -353 -100  324  193 -269 }

{ 329  167 -305 -211  275  251 -238 -285  197  314 -151 -336  103  351  -52 -358 }

{ 256 −256 −256  256  256 −256 −256  256  256 −256 −256  256  256 −256 −256  256 }

{  −9 −361   44  357  −79 −349  114  338 −147 −323  178  306 −208 −285  236  262 }

{−268 −216  311  155 −341  −88  358   18 −362   53  351 −122 −327  186  291 −243 }

{−361   79  338 −178 −285  262  208 −323 −114  357    9 −359   97  331 −194 −274 }

{ 135 -336 -119  342  103 -347  -86  351   69 -354  -52  357   35 -358  -17  359 }

{-356   34  353  -67 -346  100  337 -133 -324  163  309 -193 -290  220  269 -246 }

{-336  -86  357    0 -357   86  336 -167 -296  238  238 -296 -167  336   86 -357 }

{−230  319  105 −360   35  346 −171 −280  280  171 −346  −35  360 −105 −319  230 }

{  44  338 −208 −236  323   79 −362   97  315 −250 −194  344   27 −357  147  285 }

{ 291  122 −362   88  311 −268 −155  358  −53 −327  243  186 −351   18  341 −216 }

{ 357 −178 −236  338    9 −344  223  194 −353   44  323 −262 −147  361  −97 −296 }

{-119  357  -35 -342  182  263 -296 -135  354  -17 -347  167  275 -285 -151  351 }

{ 346 -163 -269  290  133 -353   34  337 -193 -246  309  100 -356   67  324 -220 }

{ 342    0 -342  211  211 -342    0  342 -211 -211  342    0 -342  211  211 -342 }

{ 201 −355   71  301 −301  −71  355 −201 −201  355  −71 −301  301   71 −355  201 }

{ −79 −285  323    9 −331  274   97 −357  208  178 −361  130  250 −344   44  306 }

{−311  −18  327 −291  −53  341 −268  −88  351 −243 −122  358 −216 −155  362 −186 }

{−349  262   79 −344  274   62 −338  285   44 −331  296   27 −323  306    9 −315 }

{ 103 -357  182  211 -351   69  296 -305  -52  347 -225 -167  358 -119 -263  329 }

{-324  269  100 -353  193  193 -353  100  269 -324    0  324 -269 -100  353 -193 }

{-347   86  263 -342   69  275 -336   52  285 -329   35  296 -322   17  305 -314 }

{−171  360 −230 −105  346 −280  −35  319 −319   35  280 −346  105  230 −360  171 }

{ 114  208 −362  223   97 −338  306  −27 −274  353 −147 −178  359 −250  −62  323 }

{ 327  −88 −216  362 −243  −53  311 −341  122  186 −358  268   18 −291  351 −155 }

{ 338 −323   97  194 −357  285  −27 −250  362 −236  −44  296 −353  178  114 −331 }

{ -86  336 -296    0  296 -336   86  238 -357  167  167 -357  238   86 -336  296 }

{ 290 -337  100  220 -356  193  133 -346  269   34 -309  324  -67 -246  353 -163 }

{ 351 -167 -135  342 -305   52  238 -359  225   69 -314  336 -119 -182  354 -275 }

{ 139 −334  334 −139 −139  334 −334  139  139 −334  334 −139 −139  334 −334  139 }

{−147 −114  315 −353  208   44 −274  362 −262   27  223 −357  306  −97 −163  338 }

{−341  186   53 −268  362 −291   88  155 −327  351 −216  −18  243 −358  311 −122 }

{−323  357 −250   44  178 −331  353 −236   27  194 −338  349 −223    9  208 −344 }

{  69 -296  354 -211  -52  285 -357  225   35 -275  358 -238  -17  263 -359  251 }

{-246  356 -269   34  220 -353  290  -67 -193  346 -309  100  163 -337  324 -133 }

{-354  238  -17 -211  347 -329  167   69 -275  359 -285   86  151 -322  351 -225 }

{−105  280 −360  319 −171  −35  230 −346  346 −230   35  171 −319  360 −280  105 }

{ 178    9 −194  323 −361  296 −147  −44  223 −338  357 −274  114   79 −250  349 }

{ 351 −268  122   53 −216  327 −362  311 −186   18  155 −291  358 −341  243  −88 }

{ 306 −359  344 −262  130   27 −178  296 −357  349 −274  147    9 −163  285 −353 }

{ -52  238 -347  342 -225   35  167 -314  358 -285  119   86 -263  354 -329  197 }

{ 193 -324  353 -269  100  100 -269  353 -324  193    0 -193  324 -353  269 -100 }

{ 357 -296  167    0 -167  296 -357  336 -238   86   86 -238  336 -357  296 -167 }

{  71 −201  301 −355  355 −301  201  −71  −71  201 −301  355 −355  301 −201   71 }

{−208   97   27 −147  250 −323  359 −353  306 −223  114    9 −130  236 −315  357 }

{−358  327 −268  186  −88  −18  122 −216  291 −341  362 −351  311 −243  155  −53 }

{−285  331 −357  361 −344  306 −250  178  −97    9   79 −163  236 −296  338 −359 }

{  35 -167  275 -342  358 -322  238 -119  -17  151 -263  336 -359  329 -251  135 }

{-133  246 -324  356 -337  269 -163   34  100 -220  309 -353  346 -290  193  -67 }

{-358  336 -285  211 -119   17   86 -182  263 -322  354 -357  329 -275  197 -103 }

{ −35  105 −171  230 −280  319 −346  360 −360  346 −319  280 −230  171 −105   35 }

{ 236 −194  147  −97   44    9  −62  114 −163  208 −250  285 −315  338 −353  361 }

{ 362 −358  351 −341  327 −311  291 −268  243 −216  186 −155  122  −88   53  −18 }

{ 262 −274  285 −296  306 −315  323 −331  338 −344  349 −353  357 −359  361 −362 }

{ -17   86 -151  211 -263  305 -336  354 -359  351 -329  296 -251  197 -135   69 }

{  67 -133  193 -246  290 -324  346 -356  353 -337  309 -269  220 -163  100  -34 }

{ 359 -357  351 -342  329 -314  296 -275  251 -225  197 -167  135 -103   69  -35 }

},

transMatrix[ m ][ n ] = ( n & 1 ? −1 : 1 ) * transMatrixCol16to31[47 −m ][ n ]	(8-396)
		 with m =32..47, n = 0..63

transMatrix[ m ][ n ] = ( n & 1 ? −1 : 1 ) * transMatrixCol0to15[63 −m ][ n ]	(8-397)
		with m =48..63, n = 0..63

- Otherwise, if trType is equal to 1 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-398)

{

{ 117  219  296  336 }

{ 296  296    0 −296 }

{ 336 −117 −296  219 }

{ 219 −336  296 −117 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] = 	(8-399)

{

{  65  127  185  237  280  314  338  350 }

{ 185  314  350  280  127  −65 −237 −338 }

{ 280  338  127 −185 −350 −237   65  314 }

{ 338  185 −237 −314   65  350  127 −280 }

{ 350  −65 −338  127  314 −185 −280  237 }

{ 314 −280  −65  338 −237 −127  350 −185 }

{ 237 −350  280  −65 −185  338 −314  127 }

{ 127 −237  314 −350  338 −280  185  −65 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] = 	(8-400)

{

{  34   67  100  133  163  193  220  246  269  290  309  324  337  346  353  356 }

{ 100  193  269  324  353  353  324  269  193  100    0 −100 −193 −269 −324 −353 }

{ 163  290  353  337  246  100  −67 −220 −324 −356 −309 −193  −34  133  269  346 }

{ 220  346  324  163  −67 −269 −356 −290 −100  133  309  353  246   34 −193 −337 }

{ 269  353  193 −100 −324 −324 −100  193  353  269    0 −269 −353 −193  100  324 }

{ 309  309    0 −309 −309    0  309  309    0 −309 −309    0  309  309    0 −309 }

{ 337  220 −193 −346  −34  324  246 −163 −353  −67  309  269 −133 −356 −100  290 }

{ 353  100 −324 −193  269  269 −193 −324  100  353    0 −353 −100  324  193 −269 }

{ 356  −34 −353   67  346 −100 −337  133  324 −163 −309  193  290 −220 −269  246 }

{ 346 −163 −269  290  133 −353   34  337 −193 −246  309  100 −356   67  324 −220 }

{ 324 −269 −100  353 −193 −193  353 −100 −269  324    0 −324  269  100 −353  193 }

{ 290 −337  100  220 −356  193  133 −346  269   34 −309  324  −67 −246  353 −163 }

{ 246 −356  269  −34 −220  353 −290   67  193 −346  309 −100 −163  337 −324  133 }

{ 193 −324  353 −269  100  100 −269  353 −324  193    0 −193  324 −353  269 −100 }

{ 133 −246  324 −356  337 −269  163  −34 −100  220 −309  353 −346  290 −193   67 }

{  67 −133  193 −246  290 −324  346 −356  353 −337  309 −269  220 −163  100  −34 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..31	(8-401)

transMatrixCol0to15 =	(8-402)

{

{  17   35   52   69   86  103  119  135  151  167  182  197  211  225  238  251 }

{  52  103  151  197  238  275  305  329  347  357  359  354  342  322  296  263 }

{  86  167  238  296  336  357  357  336  296  238  167   86    0  −86 −167 −238 }

{ 119  225  305  351  357  322  251  151   35  −86 −197 −285 −342 −359 −336 −275 }

{ 151  275  347  354  296  182   35 −119 −251 −336 −358 −314 −211  −69   86  225 }

{ 182  314  359  305  167  −17 −197 −322 −358 −296 −151   35  211  329  357  285 }

{ 211  342  342  211    0 −211 −342 −342 −211    0  211  342  342  211    0 −211 }

{ 238  357  296   86 −167 −336 −336 −167   86  296  357  238    0 −238 −357 −296 }

{ 263  358  225  −52 −296 −351 −182  103  322  336  135 −151 −342 −314  −86  197 }

{ 285  347  135 −182 −357 −251   52  314  329   86 −225 −359 −211  103  336  305 }

{ 305  322   35 −285 −336  −69  263  347  103 −238 −354 −135  211  358  167 −182 }

{ 322  285  −69 −347 −238  135  358  182 −197 −357 −119  251  342   52 −296 −314 }

{ 336  238 −167 −357  −86  296  296  −86 −357 −167  238  336    0 −336 −238  167 }

{ 347  182 −251 −314   86  359  103 −305 −263  167  351   17 −342 −197  238  322 }

{ 354  119 −314 −225  238  305 −135 −351   17  357  103 −322 −211  251  296 −151 }

{ 358   52 −351 −103  336  151 −314 −197  285  238 −251 −275  211  305 −167 −329 }

{ 359  −17 −358   35  357  −52 −354   69  351  −86 −347  103  342 −119 −336  135 }

{ 357  −86 −336  167  296 −238 −238  296  167 −336  −86  357    0 −357   86  336 }

{ 351 −151 −285  275  167 −347  −17  354 −135 −296  263  182 −342  −35  357 −119 }

{ 342 −211 −211  342    0 −342  211  211 −342    0  342 −211 −211  342    0 −342 }

{ 329 −263 −119  358 −167 −225  347  −52 −305  296   69 −351  211  182 −357  103 }

{ 314 −305  −17  322 −296  −35  329 −285  −52  336 −275  −69  342 −263  −86  347 }

{ 296 −336   86  238 −357  167  167 −357  238   86 −336  296    0 −296  336  −86 }

{ 275 −354  182  119 −336  314  −69 −225  359 −238  −52  305 −342  135  167 −351 }

{ 251 −359  263  −17 −238  358 −275   35  225 −357  285  −52 −211  354 −296   69 }

{ 225 −351  322 −151  −86  285 −359  275  −69 −167  329 −347  211   17 −238  354 }

{ 197 −329  354 −263   86  119 −285  358 −314  167   35 −225  342 −347  238  −52 }

{ 167 −296  357 −336  238  −86  −86  238 −336  357 −296  167    0 −167  296 −357 }

{ 135 −251  329 −359  336 −263  151  −17 −119  238 −322  358 −342  275 −167   35 }

{ 103 −197  275 −329  357 −354  322 −263  182  −86  −17  119 −211  285 −336  358 }

{  69 −135  197 −251  296 −329  351 −359  354 −336  305 −263  211 −151   86  −17 }

{  35  −69  103 −135  167 −197  225 −251  275 −296  314 −329  342 −351  357 −359 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..31	(8-403)

transMatrixCol16to31 =	(8-404)

{

{ 263  275  285  296  305  314  322  329  336  342  347  351  354  357  358  359 }

{ 225  182  135   86   35  −17  −69 −119 −167 −211 −251 −285 −314 −336 −351 −358 }

{−296 −336 −357 −357 −336 −296 −238 −167  −86    0   86  167  238  296  336  357 }

{−182  −69   52  167  263  329  358  347  296  211  103  −17 −135 −238 −314 −354 }

{ 322  359  329  238  103  −52 −197 −305 −357 −342 −263 −135   17  167  285  351 }

{ 135  −52 −225 −336 −354 −275 −119   69  238  342  351  263  103  −86 −251 −347 }

{−342 −342 −211    0  211  342  342  211    0 −211 −342 −342 −211    0  211  342 }

{ −86  167  336  336  167  −86 −296 −357 −238    0  238  357  296   86 −167 −336 }

{ 354  285   35 −238 −359 −251   17  275  357  211  −69 −305 −347 −167  119  329 }

{  35 −263 −354 −167  151  351  275  −17 −296 −342 −119  197  358  238  −69 −322 }

{−359 −197  151  357  225 −119 −351 −251   86  342  275  −52 −329 −296   17  314 }

{  17  329  275  −86 −351 −225  151  359  167 −211 −354 −103  263  336   35 −305 }

{ 357   86 −296 −296   86  357  167 −238 −336    0  336  238 −167 −357  −86  296 }

{ −69 −358 −119  296  275 −151 −354  −35  336  211 −225 −329   52  357  135 −285 }

{−347   35  358   86 −329 −197  263  285 −167 −342   52  359   69 −336 −182  275 }

{ 119  347  −69 −357   17  359   35 −354  −86  342  135 −322 −182  296  225 −263 }

{ 329 −151 −322  167  314 −182 −305  197  296 −211 −285  225  275 −238 −263  251 }

{−167 −296  238  238 −296 −167  336   86 −357    0  357  −86 −336  167  296 −238 }

{−305  251  197 −336  −52  358 −103 −314  238  211 −329  −69  359  −86 −322  225 }

{ 211  211 −342    0  342 −211 −211  342    0 −342  211  211 −342    0  342 −211 }

{ 275 −322  −17  336 −251 −135  359 −151 −238  342  −35 −314  285   86 −354  197 }

{−251 −103  351 −238 −119  354 −225 −135  357 −211 −151  358 −197 −167  359 −182 }

{−238  357 −167 −167  357 −238  −86  336 −296    0  296 −336   86  238 −357  167 }

{ 285  −17 −263  357 −197 −103  329 −322   86  211 −358  251   35 −296  347 −151 }

{ 197 −351  305  −86 −182  347 −314  103  167 −342  322 −119 −151  336 −329  135 }

{−314  135  103 −296  358 −263   52  182 −336  342 −197  −35  251 −357  305 −119 }

{−151  305 −359  296 −135  −69  251 −351  336 −211   17  182 −322  357 −275  103 }

{ 336 −238   86   86 −238  336 −357  296 −167    0  167 −296  357 −336  238  −86 }

{ 103 −225  314 −357  347 −285  182  −52  −86  211 −305  354 −351  296 −197   69 }

{−351  314 −251  167  −69  −35  135 −225  296 −342  359 −347  305 −238  151  −52 }

{ −52  119 −182  238 −285  322 −347  358 −357  342 −314  275 −225  167 −103   35 }

{ 358 −354  347 −336  322 −305  285 −263  238 −211  182 −151  119  −86   52  −17 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-405)

{

{ 336  296  219  117 }

{ 296    0 −296 −296 }

{ 219 −296 −117  336 }

{ 117 −296  336 −219 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] = 	(8-406)

{

{ 350  338  314  280  237  185  127   65 }

{ 338  237   65 −127 −280 −350 −314 −185 }

{ 314   65 −237 −350 −185  127  338  280 }

{ 280 −127 −350  −65  314  237 −185 −338 }

{ 237 −280 −185  314  127 −338  −65  350 }

{ 185 −350  127  237 −338   65  280 −314 }

{ 127 −314  338 −185  −65  280 −350  237 }

{  65 −185  280 −338  350 −314  237 −127 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] = 	(8-407)

{

{ 356  353  346  337  324  309  290  269  246  220  193  163  133  100   67   34 }

{ 353  324  269  193  100    0 −100 −193 −269 −324 −353 −353 −324 −269 −193 −100 }

{ 346  269  133  −34 −193 −309 −356 −324 −220  −67  100  246  337  353  290  163 }

{ 337  193  −34 −246 −353 −309 −133  100  290  356  269   67 −163 −324 −346 −220 }

{ 324  100 −193 −353 −269    0  269  353  193 −100 −324 −324 −100  193  353  269 }

{ 309    0 −309 −309    0  309  309    0 −309 −309    0  309  309    0 −309 −309 }

{ 290 −100 −356 −133  269  309  −67 −353 −163  246  324  −34 −346 −193  220  337 }

{ 269 −193 −324  100  353    0 −353 −100  324  193 −269 −269  193  324 −100 −353 }

{ 246 −269 −220  290  193 −309 −163  324  133 −337 −100  346   67 −353  −34  356 }

{ 220 −324  −67  356 −100 −309  246  193 −337  −34  353 −133 −290  269  163 −346 }

{ 193 −353  100  269 −324    0  324 −269 −100  353 −193 −193  353 −100 −269  324 }

{ 163 −353  246   67 −324  309  −34 −269  346 −133 −193  356 −220 −100  337 −290 }

{ 133 −324  337 −163 −100  309 −346  193   67 −290  353 −220  −34  269 −356  246 }

{ 100 −269  353 −324  193    0 −193  324 −353  269 −100 −100  269 −353  324 −193 }

{  67 −193  290 −346  353 −309  220 −100  −34  163 −269  337 −356  324 −246  133 }

{  34 −100  163 −220  269 −309  337 −353  356 −346  324 −290  246 −193  133  −67 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..31	(8-408)

transMatrixCol0to15 =	(8-409)

{

{ 359  358  357  354  351  347  342  336  329  322  314  305  296  285  275  263 }

{ 358  351  336  314  285  251  211  167  119   69   17  −35  −86 −135 −182 −225 }

{ 357  336  296  238  167   86    0  −86 −167 −238 −296 −336 −357 −357 −336 −296 }

{ 354  314  238  135   17 −103 −211 −296 −347 −358 −329 −263 −167  −52   69  182 }

{ 351  285  167   17 −135 −263 −342 −357 −305 −197  −52  103  238  329  359  322 }

{ 347  251   86 −103 −263 −351 −342 −238  −69  119  275  354  336  225   52 −135 }

{ 342  211    0 −211 −342 −342 −211    0  211  342  342  211    0 −211 −342 −342 }

{ 336  167  −86 −296 −357 −238    0  238  357  296   86 −167 −336 −336 −167   86 }

{ 329  119 −167 −347 −305  −69  211  357  275   17 −251 −359 −238   35  285  354 }

{ 322   69 −238 −358 −197  119  342  296   17 −275 −351 −151  167  354  263  −35 }

{ 314   17 −296 −329  −52  275  342   86 −251 −351 −119  225  357  151 −197 −359 }

{ 305  −35 −336 −263  103  354  211 −167 −359 −151  225  351   86 −275 −329  −17 }

{ 296  −86 −357 −167  238  336    0 −336 −238  167  357   86 −296 −296   86  357 }

{ 285 −135 −357  −52  329  225 −211 −336   35  354  151 −275 −296  119  358   69 }

{ 275 −182 −336   69  359   52 −342 −167  285  263 −197 −329   86  358   35 −347 }

{ 263 −225 −296  182  322 −135 −342   86  354  −35 −359  −17  357   69 −347 −119 }

{ 251 −263 −238  275  225 −285 −211  296  197 −305 −182  314  167 −322 −151  329 }

{ 238 −296 −167  336   86 −357    0  357  −86 −336  167  296 −238 −238  296  167 }

{ 225 −322  −86  359  −69 −329  211  238 −314 −103  358  −52 −336  197  251 −305 }

{ 211 −342    0  342 −211 −211  342    0 −342  211  211 −342    0  342 −211 −211 }

{ 197 −354   86  285 −314  −35  342 −238 −151  359 −135 −251  336  −17 −322  275 }

{ 182 −359  167  197 −358  151  211 −357  135  225 −354  119  238 −351  103  251 }

{ 167 −357  238   86 −336  296    0 −296  336  −86 −238  357 −167 −167  357 −238 }

{ 151 −347  296  −35 −251  358 −211  −86  322 −329  103  197 −357  263   17 −285 }

{ 135 −329  336 −151 −119  322 −342  167  103 −314  347 −182  −86  305 −351  197 }

{ 119 −305  357 −251   35  197 −342  336 −182  −52  263 −358  296 −103 −135  314 }

{ 103 −275  357 −322  182   17 −211  336 −351  251  −69 −135  296 −359  305 −151 }

{  86 −238  336 −357  296 −167    0  167 −296  357 −336  238  −86  −86  238 −336 }

{  69 −197  296 −351  354 −305  211  −86  −52  182 −285  347 −357  314 −225  103 }

{  52 −151  238 −305  347 −359  342 −296  225 −135   35   69 −167  251 −314  351 }

{  35 −103  167 −225  275 −314  342 −357  358 −347  322 −285  238 −182  119  −52 }

{  17  −52   86 −119  151 −182  211 −238  263 −285  305 −322  336 −347  354 −358 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..31	(8-410)

transMatrixCol16to31 =	(8-411)

{

{ 251  238  225  211  197  182  167  151  135  119  103   86   69   52   35   17 }

{−263 −296 −322 −342 −354 −359 −357 −347 −329 −305 −275 −238 −197 −151 −103  −52 }

{−238 −167  −86    0   86  167  238  296  336  357  357  336  296  238  167   86 }

{ 275  336  359  342  285  197   86  −35 −151 −251 −322 −357 −351 −305 −225 −119 }

{ 225   86  −69 −211 −314 −358 −336 −251 −119   35  182  296  354  347  275  151 }

{−285 −357 −329 −211  −35  151  296  358  322  197   17 −167 −305 −359 −314 −182 }

{−211    0  211  342  342  211    0 −211 −342 −342 −211    0  211  342  342  211 }

{ 296  357  238    0 −238 −357 −296  −86  167  336  336  167  −86 −296 −357 −238 }

{ 197  −86 −314 −342 −151  135  336  322  103 −182 −351 −296  −52  225  358  263 }

{−305 −336 −103  211  359  225  −86 −329 −314  −52  251  357  182 −135 −347 −285 }

{−182  167  358  211 −135 −354 −238  103  347  263  −69 −336 −285   35  322  305 }

{ 314  296  −52 −342 −251  119  357  197 −182 −358 −135  238  347   69 −285 −322 }

{ 167 −238 −336    0  336  238 −167 −357  −86  296  296  −86 −357 −167  238  336 }

{−322 −238  197  342  −17 −351 −167  263  305 −103 −359  −86  314  251 −182 −347 }

{−151  296  251 −211 −322  103  357   17 −351 −135  305  238 −225 −314  119  354 }

{ 329  167 −305 −211  275  251 −238 −285  197  314 −151 −336  103  351  −52 −358 }

{ 135 −336 −119  342  103 −347  −86  351   69 −354  −52  357   35 −358  −17  359 }

{−336  −86  357    0 −357   86  336 −167 −296  238  238 −296 −167  336   86 −357 }

{−119  357  −35 −342  182  263 −296 −135  354  −17 −347  167  275 −285 −151  351 }

{ 342    0 −342  211  211 −342    0  342 −211 −211  342    0 −342  211  211 −342 }

{ 103 −357  182  211 −351   69  296 −305  −52  347 −225 −167  358 −119 −263  329 }

{−347   86  263 −342   69  275 −336   52  285 −329   35  296 −322   17  305 −314 }

{ −86  336 −296    0  296 −336   86  238 −357  167  167 −357  238   86 −336  296 }

{ 351 −167 −135  342 −305   52  238 −359  225   69 −314  336 −119 −182  354 −275 }

{  69 −296  354 −211  −52  285 −357  225   35 −275  358 −238  −17  263 −359  251 }

{−354  238  −17 −211  347 −329  167   69 −275  359 −285   86  151 −322  351 −225 }

{ −52  238 −347  342 −225   35  167 −314  358 −285  119   86 −263  354 −329  197 }

{ 357 −296  167    0 −167  296 −357  336 −238   86   86 −238  336 −357  296 −167 }

{  35 −167  275 −342  358 −322  238 −119  −17  151 −263  336 −359  329 −251  135 }

{−358  336 −285  211 −119   17   86 −182  263 −322  354 −357  329 −275  197 −103 }

{ −17   86 −151  211 −263  305 −336  354 −359  351 −329  296 −251  197 −135   69 }

{ 359 −357  351 −342  329 −314  296 −275  251 −225  197 −167  135 −103   69  −35 }

},

# Conclusions

This proposal provides a method which embeds DST-7 and DCT-8 bases into DCT-2. With the proposed fast method, the additional memory requirement of MTS for storing primary transform cores becomes 0. The experimental results show that, when MTS is enabled for inter prediction residuals, average -0.05%, 0.03% and -0.07% luma BD-Rate is achieved under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 2),” JVET-K1001, 11th meeting, Ljubljana, SI, 10–18 July 2018.

- X. Zhao, X. Li, S. Liu, “CE6-related: Compound orthonormal transform,” JVET-K0292, 11th meeting, Ljubljana, SI, 10–18 July 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, 11th meeting, Ljubljana, SI, 10–18 July 2018.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).