| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0292-V3 |
| --- | --- |

| Title: | CE 6-1.1 (a,b): Selection of MTS Candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar Moonmo Koo Jaehyun Lim Seung Hwan Kim 10225 Willow Creek Rd. San Diego, CA | Tel: Email: | +1-858-635-5226 mehdi.salehifar@lge.com moonmo.koo@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

In JVET-K0096[1] AMT replacement and restriction, a simplified MTS was introduced. This contribution, reports test results of CE 6-1.1 (a,b), and explain details of algorithm and architecture applied in the proposed MTS selection and simplification, which is composed of three tests: 1) Using 1 MTS candidate for all modes for Intra CUs, 2) Using 2 MTS candidates for Angular mode and 3 candidates for non-angular mode for Intra CUs, and 3) Using 3 MTS candidates for all modes for Intra CUs. The first test uses only DST7 as the MTS candidate, however second and third tests use both DST7 and DCT8 as MTS candidates.

As first test (CE 6-1.1b) results, with 61% (AI), 91% (RA), and 95% (LD) encoding time compared to VTM anchor, BD-rate degradation is 0.30% (AI), 0.13% (RA), and 0.05% (LD).

As second test (CE 6-1.1a) results, with 78% (AI), 96% (RA), and 97% (LD) encoding time compared to VTM anchor, BD-rate degradation is 0.11% (AI), 0.06% (RA), and -0.05% (LD).

As third test results, with 88% (AI), 97% (RA), and 99% (LD) encoding time compared to VTM anchor, BD-rate difference is 0.01% (AI), 0.00% (RA), and -0.03% (LD).

# Proposed Method

1.1) MTS Candidates Selection Simplification 1:

In this simplification (Simplified_1) for MTS it uses 1 candidate for all intra direction as follows:

0) DST7 for both horizontal and vertical transform

Considering in this test only one option exists for MTS, so there will be no MTS index coding, and using MTS CU flag the selected transform would be determined.

1.2) MTS Candidates Selection Simplification 2:

In this simplification (Simplified_2) for MTS it uses 2 candidates for Angular and 3 candidates for non-angular in the following order:

A) Non-Angular mode:

0) DST7 for both horizontal and vertical transform

1) DST7 for Vertical and DCT8 for horizontal transform

2) DCT8 for Vertical and DST7 for horizontal transform

B) Horizontal Modes

0) DST7 for both horizontal and vertical transform

1) DCT8 for Vertical and DST7 for horizontal transform

C) Vertical Modes

0) DST7 for both horizontal and vertical transform

1) DST7 for Vertical and DCT8 for horizontal transform

For non-angular mode MTS index is binarized using unary code. For angular mode, a flag represents the selected MTS

1.3) MTS Candidates Selection Simplification 3:

In this simplification (Simplified_3) for MTS it uses 3 candidates for all intra direction as follows:

0) DST7 for both horizontal and vertical transform

1) DST7 for Vertical and DCT8 for horizontal transform

2) DCT8 for Vertical and DST7 for horizontal transform

For all modes MTS index is binarized using unary code, with context modeling depending on the direction.

# Experimental results

The proposed method has been implemented based on BMS 2.0.1 reference software and experimentally evaluated under VTM configuration according to common test conditions defined in JVET-K1010 [1] and Core Experiment description in JVET-K1026 [2].

2.1) Simplified_1

Anchor is VTM 2.0.1 (Inter MTS Off):

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.32% | -0.19% | -0.13% | 62% | 98% |
| Class A2 | 0.35% | -0.13% | -0.14% | 64% | 98% |
| Class B | 0.30% | -0.18% | -0.19% | 60% | 96% |
| Class C | 0.29% | -0.19% | -0.21% | 59% | 96% |
| Class E | 0.27% | -0.17% | -0.16% | 61% | 96% |
| Overall | 0.30% | -0.17% | -0.17% | 61% | 97% |
| Class D | 0.24% | -0.09% | -0.57% | 58% | 93% |
| Class F (optional) | 0.19% | -0.18% | -0.06% | 60% | 95% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.11% | 0.00% | 93% | 101% |
| Class A2 | 0.08% | -0.12% | -0.17% | 92% | 100% |
| Class B | 0.16% | -0.09% | 0.01% | 91% | 102% |
| Class C | 0.20% | 0.12% | -0.12% | 90% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.13% | -0.05% | -0.06% | 91% | 101% |
| Class D | 0.12% | -0.54% | -0.16% | 90% | 93% |
| Class F (optional) | 0.21% | -0.10% | -0.17% | 85% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.15% | 0.02% | 95% | 100% |
| Class C | 0.05% | -0.15% | -0.15% | 95% | 99% |
| Class E | 0.10% | -0.21% | -0.66% | 98% | 100% |
| Overall | 0.05% | -0.17% | -0.21% | 95% | 100% |
| Class D | 0.02% | 0.26% | -0.22% | 95% | 97% |
| Class F (optional) | -0.10% | -0.20% | -0.35% | 92% | 97% |

Anchor is VTM 2.0.1 (Inter MTS On):

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.20% | 0.02% | 93% | 99% |
| Class A2 | 0.05% | -0.15% | -0.10% | 94% | 100% |
| Class B | 0.15% | -0.14% | 0.00% | 93% | 101% |
| Class C | 0.14% | 0.10% | -0.05% | 93% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.12% | -0.09% | -0.03% | 93% | 100% |
| Class D | 0.09% | -0.28% | 0.26% | 93% | 102% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.15% | 0.10% | 96% | 98% |
| Class C | 0.02% | 0.02% | -0.08% | 96% | 99% |
| Class E | 0.10% | 0.22% | -0.53% | 98% | 99% |
| Overall | 0.03% | 0.00% | -0.12% | 97% | 99% |
| Class D | -0.09% | -0.20% | 0.31% | 96% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

2.2) Simplified_2

Anchor is VTM 2.0.1(Inter MTS Off):

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.26% | 0.22% | 80% | 98% |
| Class A2 | 0.09% | 0.16% | 0.13% | 81% | 98% |
| Class B | 0.11% | 0.26% | 0.19% | 78% | 99% |
| Class C | 0.10% | 0.10% | 0.26% | 76% | 99% |
| Class E | 0.10% | 0.28% | 0.27% | 77% | 98% |
| Overall | 0.11% | 0.21% | 0.22% | 78% | 99% |
| Class D | 0.07% | 0.16% | -0.07% | 75% | 93% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.13% | 97% | 102% |
| Class A2 | 0.03% | -0.03% | -0.03% | 97% | 101% |
| Class B | 0.09% | 0.19% | 0.19% | 95% | 101% |
| Class C | 0.11% | 0.10% | 0.12% | 94% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.09% | 0.12% | 96% | 100% |
| Class D | 0.08% | -0.03% | 0.08% | 93% | 91% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.09% | 0.21% | 97% | 100% |
| Class C | 0.03% | -0.10% | -0.06% | 96% | 99% |
| Class E | -0.27% | 0.32% | 0.13% | 100% | 105% |
| Overall | -0.05% | 0.01% | 0.10% | 97% | 101% |
| Class D | 0.04% | 0.39% | -0.13% | 98% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Anchor is VTM 2.0.1 (Inter MTS On):

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.05% | 0.24% | 96% | 100% |
| Class A2 | 0.00% | 0.11% | 0.04% | 96% | 100% |
| Class B | 0.04% | 0.10% | 0.17% | 96% | 100% |
| Class C | 0.06% | 0.05% | 0.15% | 96% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.08% | 0.15% | 96% | 100% |
| Class D | 0.01% | -0.02% | 0.12% | 96% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.16% | 0.45% | 98% | 100% |
| Class C | -0.01% | 0.07% | 0.07% | 98% | 99% |
| Class E | -0.07% | -0.07% | -0.01% | 100% | 100% |
| Overall | -0.02% | -0.06% | 0.21% | 99% | 100% |
| Class D | -0.06% | 0.36% | 0.56% | 98% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

2.3) Simplified_3

Anchor is VTM 2.0.1:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.04% | -0.03% | 88% | 100% |
| Class A2 | 0.02% | -0.02% | -0.01% | 89% | 100% |
| Class B | 0.02% | -0.02% | 0.00% | 87% | 99% |
| Class C | -0.01% | -0.08% | 0.11% | 87% | 101% |
| Class E | -0.01% | 0.05% | -0.06% | 88% | 100% |
| Overall | 0.01% | -0.03% | 0.01% | 88% | 100% |
| Class D | -0.02% | -0.01% | -0.21% | 86% | 95% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.13% | 0.04% | 98% | 101% |
| Class A2 | 0.01% | -0.13% | -0.06% | 97% | 100% |
| Class B | 0.00% | -0.03% | -0.02% | 97% | 101% |
| Class C | 0.00% | 0.09% | 0.26% | 97% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | 0.06% | 97% | 101% |
| Class D | -0.01% | -0.10% | 0.19% | 96% | 94% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.01% | -0.34% | 98% | 101% |
| Class C | 0.00% | 0.02% | -0.17% | 98% | 101% |
| Class E | -0.14% | -0.75% | -0.23% | 100% | 102% |
| Overall | -0.03% | -0.18% | -0.25% | 99% | 101% |
| Class D | -0.04% | 0.04% | 0.12% | 98% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusion

This contribution, reports test results of CE 6-1.1 (a,b), which is for MTS simplification. The proposed technique is tested in three tests: 1) Using 1 MTS candidate for all modes for Intra CUs, 2) Using 2 MTS candidates for Angular mode and 3 candidates for non-angular mode for Intra CUs, and 3) Using 3 MTS candidates for all modes for Intra CUs.  As it is reported in this contribution, the simplified MTS has significant encoding time reduction with a small performance loss. It is strongly recommended that the proposed method be adopted into the next VTM.

# Reference

- M. Koo, M. Salehifar, “CE 6.1.11: AMT replacement and restriction”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0096, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- J. Boyce et al., “JVET common test conditions and software reference configurations”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1010, 11th meeting, Ljubljana, SI, 10-18 July 2018.

- A. Said and X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signaling”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K1026, 11th meeting, Ljubljana, SI, 10-18 July 2018.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).