| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0293-v2 |
| --- | --- |

| Title: | CE8: CPR mode with local search ranges (Test CE8.3.1 and CE8.3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu, Xiang Li and Shan Liu | Tel: Email: | xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the CE test results for CE8.3.1 and CE8.3.2. Specifically, in CE8.3.1a, search range that uses only current CTU is allowed; in CE8.3.2a, search range that uses current CTU and 1 left CTU is allowed. In addition to these, in CE 8.3.1b and CE8.3.2b, chroma block vectors will be clipped to integer positions.

It is reported that with these constraints, the BD rate changes over VTM2 are:

For CE8.3.1a:

- In AI, -0.16%/-11.61%/-36.51% for CTC/Class F/SCC 1080p classes, separately.

- In RA, -0.07%/-9.63%/-22.76% for CTC/Class F/SCC 1080p classes, separately.

For CE8.3.1b:

- In AI, -0.18%/-11.87%/-36.60% for CTC/Class F/SCC 1080p classes, separately.

- In RA, -0.07%/-9.73%/-22.36% for CTC/Class F/SCC 1080p classes, separately.

For CE8.3.2a:

- In AI, -0.29%/-15.49%/-46.10 % for CTC/Class F/SCC 1080p classes, separately.

- In RA, -0.10%/-12.67%/-29.42% for CTC/Class F/SCC 1080p classes, separately.

For CE8.3.2b:

- In AI, -0.31%/-15.60%/-45.73% for CTC/Class F/SCC 1080p classes, separately.

- In RA, -0.09%/-12.69%/-28.69% for CTC/Class F/SCC 1080p classes, separately.

# Proposed CE tests

In last JVET meeting, CPR mode [1] was adopted into BMS software for further evaluation. In CE8, a few methods are proposed to constrain the search range of CPR mode. In CE8.3.1, search range is constrained to be within the current CTU. In CE8.3.2, search range is constrained to be within the current CTU and the left CTU. In addition to these, in CE 8.3.1b and CE8.3.2b, chroma interpolation is disabled.

This document reports coding efficiency of the CE tests in comparison to VTM-2 and also the CPR in BMS (BMS-CPR).

# Simulation results

The simulation is performed using CE8-3 software (based on BMS-2.1rc1), under VTM configuration. CPR mode is turned on. Common test conditions in CE8 [2] are followed. The following two tests are performed:

- Test 1: CE8.3.1a (1 CTU)

- Test 2: CE8.3.1b (1 CTU, no chroma interpolation)

- Test 3: CE8.3.2a (2 CTU)

- Test 4: CE8.3.2b (2 CTU, no chroma interpolation)

Results are shown in the following two tables:

Table I: coding performance comparison of test 1 on top of VTM-2.0.1 and BMS-CPR (full frame)

Table II: coding performance comparison of test 2 on top of VTM-2.0.1 and CE8.3.6b BMS-CPR (full frame)

Table III: coding performance comparison of test3 on top of VTM-2.0.1 and BMS-CPR (full frame)

Table IV: coding performance comparison of test 4 on top of VTM-2.0.1 and BMS-CPR (full frame)

# Conclusion

In this contribution, constraints on CPR mode with variations in CE8.3.1 and CE8.3.2 are tested. Simulation results show that the search range with current and left CTU is a good tradeoff between coding efficiency and encoding runtime.

# References

[1] Xiaozhong Xu, Xiang Li and Shan Liu, “CE8-2.2: Current picture referencing using reference index signaling”, JVET-K0076, Ljubljana, SL, July 2018

[2] X. Xu, K. Müller, L. Wang, “Description of Core Experiment 8 (CE8): Current Picture Referencing”, JVET-K1028, Ljubljana, SL, July 2018

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).