| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0304-V1 |
| --- | --- |

| Title: | CE 6-Related : Multiplication Free Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar Moonmo Koo Seethal Paluri Jaehyun Lim Seung Hwan Kim 10225 Willow Creek Rd. San Diego, CA | Tel: Email: | +1-858-635-5226 mehdi.salehifar@lge.com moonmo.koo@lge.com seethal.paluri@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution proposes multiplication free transform which consists of only additions (subtraction) and shifts, which could be applied for any transform including MTS candidates. The proposed technique is tested with different maximum number of the allowable terms for the shifts and additions. In particular it is tested with: 1) Allowing 5 terms of shifts, 2) Allowing 4 terms of shifts, and 3) Allowing 3 terms of shifts.

As first test results with allowing maximum 5 terms of shifts compare to VTM anchor, has identical performance.

As second test results with allowing maximum 4 terms of shifts compare to VTM anchor, BD-rate difference of 0.00% (AI) is observed.

As third test results with allowing maximum 3 terms of shifts compare to VTM anchor, BD-rate difference of 0.11% (AI) is observed.

# Proposed Method

Any multiplication could be written as several shifts and additions.

As an example N multiply by 12 could be written as N<<3 + N<<2. Similarly any transform multiplication could be replaced and approximated by several shifts and addition. For the current MTS candidate in VTM2, two possible transforms are DST7 and DCT8. DST7 and DCT8 of size N, have N+ 1 distinguished element. This proposal approximates all the transform elements for MTS from size 4x4 to 32x32 with shifts and addition (subtraction). It has been tested for different maximum allowable number of shifts and additions. Particularly it is tested for max terms of 5,4 and 3.

Max Allowable Terms: 5

| 4x4 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Original Coefficient | Final Coefficient | Difference |  |
|  | 0 | 0 | 0 |  |
|  | 117 | 117 | 0 | 1<<7 - 1<<3 - 1<<2 + 1<<0 |
|  | 219 | 219 | 0 | 1<<8 - 1<<5 -1<<2 - 1<<0 |
|  | 296 | 296 | 0 | 1<<8 + 1<<5 +1<<3 |
|  | 336 | 336 | 0 | 1<<8 + 1<<6 +1<<4 |
| 8x8 |  |  |  |  |
|  | Original Coefficient | Final Coefficient | Difference |  |
|  | 0 | 0 | 0 |  |
|  | 65 | 65 | 0 | 1<<6 + 1<<0 |
|  | 127 | 127 | 0 | 1<<7 - 1<<0 |
|  | 185 | 185 | 0 | 1<<7 + 1<<6 - 1<<3 + 1<<0 |
|  | 237 | 237 | 0 | 1<<8 - 1<<4 - 1<<1 - 1<<0 |
|  | 280 | 280 | 0 | 1<<8 + 1<<5 - 1<<3 |
|  | 314 | 314 | 0 | 1<<8 + 1<<6 - 1<<3 + 1<<1 |
|  | 338 | 338 | 0 | 1<<8 + 1<<6 + 1<<4 + 1<<1 |
|  | 350 | 350 | 0 | 1<<8 + 1<<6 + 1<<5 - 1<<1 |
| 16x16 | Original Coefficient | Final Coefficient | Difference |  |
|  | 0 | 0 | 0 |  |
|  | 34 | 34 | 0 | 1<<5 + 1<<1 |
|  | 67 | 67 | 0 | 1<<6 + 1<<1 + 1<<0 |
|  | 100 | 100 | 0 | 1<<6 + 1<<5 + 1<<2 |
|  | 133 | 133 | 0 | 1<<7 + 1<<2 + 1<<0 |
|  | 163 | 163 | 0 | 1<<7 + 1<<5 + 1<<1 + 1<<0 |
|  | 193 | 193 | 0 | 1<<7 + 1<<6 + 1<<0 |
|  | 220 | 220 | 0 | 1<<8 - 1<<5 - 1<<2 |
|  | 246 | 246 | 0 | 1<<8 - 1<<3 - 1<<1 |
|  | 269 | 269 | 0 | 1<<8 + 1<<3 + 1<<2 + 1<<0 |
|  | 290 | 290 | 0 | 1<<8 + 1<<5 + 1<<1 |
|  | 309 | 309 | 0 | 1<<8 + 1<<6 - 1<<3 - 1<<2+ 1<<0 |
|  | 324 | 324 | 0 | 1<<8 + 1<<6 + 1<<2 |
|  | 337 | 337 | 0 | 1<<8 +1<<6 + 1<<4+ 1<<0 |
|  | 346 | 346 | 0 | 1<<8 + 1<<6 + 1<<5 -1<<2-1<<1 |
|  | 353 | 353 | 0 | 1<<9 - 1<<7 - 1<<5 + 1<<0 |
|  | 356 | 356 | 0 | 1<<8 + 1<<6 + 1<<5 + 1<<2 |
| 32x32 | Original Coefficient | Final Coefficient | Difference |  |
|  | 0 | 0 | 0 |  |
|  | 17 | 17 | 0 | 1<<4 + 1<<0 |
|  | 35 | 35 | 0 | 1<<5 + 1<<1 + 1<<0 |
|  | 52 | 52 | 0 | 1<<6 - 1<<3 -1<<2 |
|  | 69 | 69 | 0 | 1<<6 + 1<<2 + 1<<0 |
|  | 86 | 86 | 0 | 1<<6 + 1<<5 - 1<<3 - 1<<1 |
|  | 103 | 103 | 0 | 1<<6 + 1<<5 + 1<<3-1<<0 |
|  | 119 | 119 | 0 | 1<<7 - 1<<3 - 1<<0 |
|  | 135 | 135 | 0 | 1<<7 + 1<<2 + 1<<1 + 1<<0 |
|  | 151 | 151 | 0 | 1<<7 + 1<<4 + 1<<3- 1<<0 |
|  | 167 | 167 | 0 | 1<7 + 1<<5 + 1<<3 - 1<<0 |
|  | 182 | 182 | 0 | 1<<7 + 1<<6 - 1<<3 - 1<<1 |
|  | 197 | 197 | 0 | 1<<7 + 1<<6 + 1<<2 + 1<<0 |
|  | 211 | 211 | 0 | 1<<8 - 1<<5 - 1<<3 - 1<<2 - 1<<0 |
|  | 225 | 225 | 0 | 1<<8 - 1<<5 + 1<<0 |
|  | 238 | 238 | 0 | 1<<8 - 1<<4 - 1<<1 |
|  | 251 | 251 | 0 | 1<<8 - 1<<2 - 1<<0 |
|  | 263 | 263 | 0 | 1<<8 + 1<<3 - 1<<0 |
|  | 275 | 275 | 0 | 1<<8 + 1<<4 + 1<<2 - 1<<0 |
|  | 285 | 285 | 0 | 1<<8 + 1<<5 - 1<<2 + 1<<0 |
|  | 296 | 296 | 0 | 1<<8 + 1<<5 + 1<<3 |
|  | 305 | 305 | 0 | 1<<8 + 1<<5 + 1<<4 + 1<<0 |
|  | 314 | 314 | 0 | 1<<8 + 1<<6 - 1<<2 - 1<<1 |
|  | 322 | 322 | 0 | 1<<8 + 1<<6 + 1<<1 |
|  | 329 | 329 | 0 | 1<<8 + 1<<6 + 1<<3 + 1<<0 |
|  | 336 | 336 | 0 | 1<<8 + 1<<6 + 1<<4 |
|  | 342 | 342 | 0 | 1<<8 + 1<<6 + 1<<4 + 1<<2 + 1<<1 |
|  | 347 | 347 | 0 | 1<<8 + 1<<6 + 1<<5 - 1<<2 - 1<<0 |
|  | 351 | 351 | 0 | 1<<9 - 1<<7 - 1<<5 - 1<<0 |
|  | 354 | 354 | 0 | 1<<9 - 1<<7 - 1<<5 + 1<<1 |
|  | 357 | 357 | 0 | 1<<8 + 1<<6 + 1<<5 + 1<<2 + 1<<0 |
|  | 358 | 358 | 0 | 1<<8 + 1<<6 + 1<<5 + 1<<2 + 1<<1 |
|  | 359 | 359 | 0 | 1<<8 + 1<<6 + 1<<5 + 1<<3 - 1<<0 |

Max Allowable Terms: 4

| 4x4 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Original Coefficient | Final Coefficient | Difference |  |
|  | 0 | 0 | 0 |  |
|  | 117 | 117 | 0 | 1<<7 - 1<<3 - 1<<2 + 1<<0 |
|  | 219 | 219 | 0 | 1<<8 - 1<<5 -1<<2 - 1<<0 |
|  | 296 | 296 | 0 | 1<<8 + 1<<5 +1<<3 |
|  | 336 | 336 | 0 | 1<<8 + 1<<6 +1<<4 |
| 8x8 |  |  |  |  |
|  | Original Coefficient | Final Coefficient | Difference |  |
|  | 0 | 0 | 0 |  |
|  | 65 | 65 | 0 | 1<<6 + 1<<0 |
|  | 127 | 127 | 0 | 1<<7 - 1<<0 |
|  | 185 | 185 | 0 | 1<<7 + 1<<6 - 1<<3 + 1<<0 |
|  | 237 | 237 | 0 | 1<<8 - 1<<4 - 1<<1 - 1<<0 |
|  | 280 | 280 | 0 | 1<<8 + 1<<5 - 1<<3 |
|  | 314 | 314 | 0 | 1<<8 + 1<<6 - 1<<3 + 1<<1 |
|  | 338 | 338 | 0 | 1<<8 + 1<<6 + 1<<4 + 1<<1 |
|  | 350 | 350 | 0 | 1<<8 + 1<<6 + 1<<5 - 1<<1 |
| 16x16 | Original Coefficient | Final Coefficient | Difference |  |
|  | 0 | 0 | 0 |  |
|  | 34 | 34 | 0 | 1<<5 + 1<<1 |
|  | 67 | 67 | 0 | 1<<6 + 1<<1 + 1<<0 |
|  | 100 | 100 | 0 | 1<<6 + 1<<5 + 1<<2 |
|  | 133 | 133 | 0 | 1<<7 + 1<<2 + 1<<0 |
|  | 163 | 163 | 0 | 1<<7 + 1<<5 + 1<<1 + 1<<0 |
|  | 193 | 193 | 0 | 1<<7 + 1<<6 + 1<<0 |
|  | 220 | 220 | 0 | 1<<8 - 1<<5 - 1<<2 |
|  | 246 | 246 | 0 | 1<<8 - 1<<3 - 1<<1 |
|  | 269 | 269 | 0 | 1<<8 + 1<<3 + 1<<2 + 1<<0 |
|  | 290 | 290 | 0 | 1<<8 + 1<<5 + 1<<1 |
|  | 309 | 308 | 1 | 1<<8 + 1<<6 - 1<<3 - 1<<2 |
|  | 324 | 324 | 0 | 1<<8 + 1<<6 + 1<<2 |
|  | 337 | 337 | 0 | 1<<8 +1<<6 + 1<<4+ 1<<0 |
|  | 346 | 348 | 2 | 1<<8 + 1<<6 + 1<<5 -1<<2 |
|  | 353 | 353 | 0 | 1<<9 - 1<<7 - 1<<5 + 1<<0 |
|  | 356 | 356 | 0 | 1<<8 + 1<<6 + 1<<5 + 1<<2 |
| 32x32 | Original Coefficient | Final Coefficient | Difference |  |
|  | 0 | 0 | 0 |  |
|  | 17 | 17 | 0 | 1<<4 + 1<<0 |
|  | 35 | 35 | 0 | 1<<5 + 1<<1 + 1<<0 |
|  | 52 | 52 | 0 | 1<<6 - 1<<3 -1<<2 |
|  | 69 | 69 | 0 | 1<<6 + 1<<2 + 1<<0 |
|  | 86 | 86 | 0 | 1<<6 + 1<<5 - 1<<3 - 1<<1 |
|  | 103 | 103 | 0 | 1<<6 + 1<<5 + 1<<3-1<<0 |
|  | 119 | 119 | 0 | 1<<7 - 1<<3 - 1<<0 |
|  | 135 | 135 | 0 | 1<<7 + 1<<2 + 1<<1 + 1<<0 |
|  | 151 | 151 | 0 | 1<<7 + 1<<4 + 1<<3- 1<<0 |
|  | 167 | 167 | 0 | 1<7 + 1<<5 + 1<<3 - 1<<0 |
|  | 182 | 182 | 0 | 1<<7 + 1<<6 - 1<<3 - 1<<1 |
|  | 197 | 197 | 0 | 1<<7 + 1<<6 + 1<<2 + 1<<0 |
|  | 211 | 212 | 1 | 1<<8 - 1<<5 - 1<<3 - 1<<2 |
|  | 225 | 225 | 0 | 1<<8 - 1<<5 + 1<<0 |
|  | 238 | 238 | 0 | 1<<8 - 1<<4 - 1<<1 |
|  | 251 | 251 | 0 | 1<<8 - 1<<2 - 1<<0 |
|  | 263 | 263 | 0 | 1<<8 + 1<<3 - 1<<0 |
|  | 275 | 275 | 0 | 1<<8 + 1<<4 + 1<<2 - 1<<0 |
|  | 285 | 285 | 0 | 1<<8 + 1<<5 - 1<<2 + 1<<0 |
|  | 296 | 296 | 0 | 1<<8 + 1<<5 + 1<<3 |
|  | 305 | 305 | 0 | 1<<8 + 1<<5 + 1<<4 + 1<<0 |
|  | 314 | 314 | 0 | 1<<8 + 1<<6 - 1<<2 - 1<<1 |
|  | 322 | 322 | 0 | 1<<8 + 1<<6 + 1<<1 |
|  | 329 | 329 | 0 | 1<<8 + 1<<6 + 1<<3 + 1<<0 |
|  | 336 | 336 | 0 | 1<<8 + 1<<6 + 1<<4 |
|  | 342 | 340 | 2 | 1<<8 + 1<<6 + 1<<4 + 1<<2 |
|  | 347 | 348 | 1 | 1<<8 + 1<<6 + 1<<5 - 1<<2 |
|  | 351 | 351 | 0 | 1<<9 - 1<<7 - 1<<5 - 1<<0 |
|  | 354 | 354 | 0 | 1<<9 - 1<<7 - 1<<5 + 1<<1 |
|  | 357 | 356 | 1 | 1<<8 + 1<<6 + 1<<5 + 1<<2 |
|  | 358 | 356 | 2 | 1<<8 + 1<<6 + 1<<5 + 1<<2 |
|  | 359 | 360 | 1 | 1<<8 + 1<<6 + 1<<5 + 1<<3 |

Max Allowable Terms: 3

| 4x4 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Original Coefficient | Final Coefficient | Difference |  |
|  | 0 | 0 | 0 |  |
|  | 117 | 116 | 1 | 1<<7 - 1<<3 - 1<<2 |
|  | 219 | 220 | 1 | 1<<8 - 1<<5 -1<<2 |
|  | 296 | 296 | 0 | 1<<8 + 1<<5 +1<<3 |
|  | 336 | 336 | 0 | 1<<8 + 1<<6 +1<<4 |
| 8x8 |  |  |  |  |
|  | Original Coefficient | Final Coefficient | Difference |  |
|  | 0 | 0 | 0 |  |
|  | 65 | 65 | 0 | 1<<6 + 1<<0 |
|  | 127 | 127 | 0 | 1<<7 - 1<<0 |
|  | 185 | 184 | 1 | 1<<7 + 1<<6 - 1<<3 |
|  | 237 | 236 | 1 | 1<<8 - 1<<4 - 1<<1 |
|  | 280 | 280 | 0 | 1<<8 + 1<<5 - 1<<3 |
|  | 314 | 312 | 2 | 1<<8 + 1<<6 - 1<<3 |
|  | 338 | 336 | 2 | 1<<8 + 1<<6 + 1<<4 |
|  | 350 | 351 | 1 | 1<<8 + 1<<6 + 1<<5 |
| 16x16 | Original Coefficient | Final Coefficient | Difference |  |
|  | 0 | 0 | 0 |  |
|  | 34 | 34 | 0 | 1<<5 + 1<<1 |
|  | 67 | 67 | 0 | 1<<6 + 1<<1 + 1<<0 |
|  | 100 | 100 | 0 | 1<<6 + 1<<5 + 1<<2 |
|  | 133 | 133 | 0 | 1<<7 + 1<<2 + 1<<0 |
|  | 163 | 162 | 1 | 1<<7 + 1<<5 + 1<<1 |
|  | 193 | 193 | 0 | 1<<7 + 1<<6 + 1<<0 |
|  | 220 | 220 | 0 | 1<<8 - 1<<5 - 1<<2 |
|  | 246 | 246 | 0 | 1<<8 - 1<<3 - 1<<1 |
|  | 269 | 268 | 1 | 1<<8 + 1<<3 + 1<<2 |
|  | 290 | 290 | 0 | 1<<8 + 1<<5 + 1<<1 |
|  | 309 | 312 | 3 | 1<<8 + 1<<6 - 1<<3 |
|  | 324 | 324 | 0 | 1<<8 + 1<<6 + 1<<2 |
|  | 337 | 336 | 1 | 1<<8 +1<<6 + 1<<4 |
|  | 346 | 352 | 6 | 1<<8 + 1<<6 + 1<<5 |
|  | 353 | 352 | 1 | 1<<9 - 1<<7 - 1<<5 |
|  | 356 | 352 | 4 | 1<<8 + 1<<6 + 1<<5 |
| 32x32 | Original Coefficient | Final Coefficient | Difference |  |
|  | 0 | 0 | 0 |  |
|  | 17 | 17 | 0 | 1<<4 + 1<<0 |
|  | 35 | 35 | 0 | 1<<5 + 1<<1 + 1<<0 |
|  | 52 | 52 | 0 | 1<<6 - 1<<3 -1<<2 |
|  | 69 | 69 | 0 | 1<<6 + 1<<2 + 1<<0 |
|  | 86 | 88 | 2 | 1<<6 + 1<<5 - 1<<3 |
|  | 103 | 104 | 1 | 1<<6 + 1<<5 + 1<<3 |
|  | 119 | 119 | 0 | 1<<7 - 1<<3 - 1<<0 |
|  | 135 | 134 | 1 | 1<<7 + 1<<2 + 1<<1 |
|  | 151 | 152 | 1 | 1<<7 + 1<<4 + 1<<3 |
|  | 167 | 168 | 1 | 1<7 + 1<<5 + 1<<3 |
|  | 182 | 184 | 2 | 1<<7 + 1<<6 - 1<<3 |
|  | 197 | 196 | 1 | 1<<7 + 1<<6 + 1<<2 |
|  | 211 | 208 | 3 | 1<<8 - 1<<5 - 1<<4 |
|  | 225 | 225 | 0 | 1<<8 - 1<<5 + 1<<0 |
|  | 238 | 238 | 0 | 1<<8 - 1<<4 - 1<<1 |
|  | 251 | 251 | 0 | 1<<8 - 1<<2 - 1<<0 |
|  | 263 | 263 | 0 | 1<<8 + 1<<3 - 1<<0 |
|  | 275 | 276 | 1 | 1<<8 + 1<<4 + 1<<2 |
|  | 285 | 284 | 1 | 1<<8 + 1<<5 - 1<<2 |
|  | 296 | 296 | 0 | 1<<8 + 1<<5 + 1<<3 |
|  | 305 | 304 | 1 | 1<<8 + 1<<5 + 1<<4 |
|  | 314 | 316 | 2 | 1<<8 + 1<<6 - 1<<2 |
|  | 322 | 322 | 0 | 1<<8 + 1<<6 + 1<<1 |
|  | 329 | 328 | 1 | 1<<8 + 1<<6 + 1<<3 |
|  | 336 | 336 | 0 | 1<<8 + 1<<6 + 1<<4 |
|  | 342 | 336 | 6 | 1<<8 + 1<<6 + 1<<4 |
|  | 347 | 352 | 5 | 1<<8 + 1<<6 + 1<<5 |
|  | 351 | 352 | 1 | 1<<9 - 1<<7 - 1<<5 |
|  | 354 | 352 | 2 | 1<<9 - 1<<7 - 1<<5 |
|  | 357 | 352 | 5 | 1<<8 + 1<<6 + 1<<5 |
|  | 358 | 352 | 6 | 1<<8 + 1<<6 + 1<<5 |
|  | 359 | 352 | 7 | 1<<8 + 1<<6 + 1<<5 |

# Experimental results

The proposed method has been implemented based on VTM2.0.1 reference software and experimentally evaluated for VTM test according to common test conditions defined in JVET-K---- [2] and Core Experiment description in JVET-K---- [3].

2.1) 5 Terms Results:

This is identical to original one, so it has identical result to VTM/BMS.

2.2) 4 Terms Results:

Anchor is VTM 2.0.1:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.04% | -0.03% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | -0.04% | 100% | 101% |
| Class B | 0.01% | 0.03% | 0.00% | 100% | 100% |
| Class C | -0.01% | 0.01% | 0.13% | 100% | 99% |
| Class E | -0.01% | 0.01% | -0.10% | 101% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.01% | 0.18% | -0.01% | 99% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

2.3) 3 Terms Results:

Anchor is VTM 2.0.1:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | -0.03% | 0.02% | 95% | 94% |
| Class A2 | 0.18% | -0.04% | -0.06% | 101% | 102% |
| Class B | 0.11% | 0.02% | 0.06% | 101% | 101% |
| Class C | 0.02% | -0.01% | 0.17% | 100% | 100% |
| Class E | 0.09% | -0.03% | -0.16% | 101% | 100% |
| Overall | 0.11% | -0.01% | 0.02% | 100% | 100% |
| Class D | 0.03% | 0.24% | -0.11% | 100% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).