| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0313-v2 |
| --- | --- |

| Title: | CE1-related: Non-square virtual pipeline data unit |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu, Xiang Li, Shan Liu | Tel: Email: | {mengxxu, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Virtual pipeline data units (VPDUs) are defined in JVET-K0556 as non-overlapping MxM-luma(L)/NxN-chroma(C) units in a picture. In hardware decoders, successive VPDUs are processed by multiple pipeline stages at the same time; different stages process different VPDUs simultaneously. The VPDU size is roughly proportional to the buffer size in most pipeline stages. In this contribution, the VPDU is extended to cover non-square regions. It is reported that 0.00%, 0.22%, 0.14%, and 0.17% luma BD-rate reductions are achieved on average over BMS-2.1 with VTM configurations for AI, RA, LB, and LP configurations.

# Introduction

Virtual pipeline data units (VPDUs) are defined in JVET-K0556 [3] as non-overlapping MxM-luma(L)/NxN-chroma(C) units in a picture. In hardware decoders, successive VPDUs are processed by multiple pipeline stages at the same time; different stages process different VPDUs simultaneously. The VPDU size is roughly proportional to the buffer size in most pipeline stages. Several constraints in JVET-K0556 [3] are tested in CE1 sub test 2.

In VTM-2.0.1 [4], the maximum TT size for inter frames is set to 64, which may degrade the coding performance.

# Proposed method

In this contribution, non-square VPDU is proposed. Each VPDU contains the same number of pixels so that can be processed by the pipeline engines, but it does not necessarily to have a square shape. A non-square VPDU can either be part of a CU, or can contain one or multiple CUs. It is proposed that a conformant bitstream shall meet the following constraints for the non-square VPDUs.

- For each VPDU containing one or multiple CUs, the CUs are completely contained in the VPDU.

- For each CU containing one or more VPDUs, the VPDUs are completely contained in the CU.

With these conditions, the TT split for the 128x128 block may be implemented in VPDU, thus the maximum TT size for inter frames can be larger than 64. In this contribution, 128 is used as the maximum TT size for inter frames.

In VTM-2.0.1 and BMS-2.1, the max TT size for intra frames is 32. Thus the proposed method has no effect on intra frames in current implementation. The proposed method has been implemented on the encoder to produce the bitstreams conformant to those constraints.

1

2

3

5

4

6

Figure 1 Example of a CTU containing 6 sub CUs (solid lines) and 4 VPDUs (dashed lines). The numbers associated with the CUs indicate the encoding/decoding order.

Figure 1 shows an example where CTU size is 128x128 and VPDU size is 4096. The CTU is split by vertical TT, then the center partition is further split by horizontal TT, finally the center partition is split by vertical BT. In this example, the first CU (32x128) falls into the first VPDU; the second CU (64x32) and the third CU (32x64) together fall into the second VPDU; the fourth CU (32x64) and the fifth CU (64x32) together fall into the third VPDU; the sixth CU (32x128) falls into the fourth VPDU. This CTU can be covered by four non-overlapping VPDUs. Therefore, this partition is valid.

1

5

2

3

4

Figure 2 Illustration of a CTU partition that cannot be covered by non-overlapping VPDUs. The numbers associated with the CUs indicate the encoding/decoding order.

Figure 2 shows another example where the CTU is 128x128 and VPDU size is 4096. It is split by vertical TT, then the center partition is further split by horizontal TT. The first CU is 32x128, the second is 64x32, the third is 64x64, the fourth is 64x32, the fifth is 32x128. As the size of VPDU is 4096, the second CU also starts the second VPDU. As the second CU size is 2048, while the third CU size is 4096, the TU of the third CU crosses VPDU boundary, which shall not be allowed in a conforming bitstream.

# Simulation results

During the development of BMS-2.1, a bug regarding TT split is identified and fixed. To evaluate the result without influenced by the bug, BMS-2.1 with VTM configuration is used as the anchor.

Reference #1: BMS-2.1 (with VTM configuration)

Reference #2: BMS-2.1 (with VTM configuration) + MAX_TT_SIZE_INTER=128

Test: BMS-2.1 (with VTM configuration) + proposed method

Compared with BMS-2.1 with VTM configuration, the result shows 0.00%, 0.22%, 0.14%, and 0.17% coding gain on average for luma in AI, RA, LB, and LP configurations, respectively. It is also reported up to 15% and 3% complexity increase for encoding and decoding, respectively. When compared with BMS-2.1 with MAX_TT_SIZE_INTER equal to 128, the proposed VPDU constraints show 0.00%, 0.07%, 0.08%, and 0.08% loss on average for luma in AI, RA, LB, and LP configurations, respectively.

We would like to thank HHI for the crosscheck.

# Conclusions

In this contribution, non-square VPDU is proposed. It is reported that 0.00%, 0.22%, 0.14%, and 0.17% luma BD-rate reductions are achieved on average over BMS-2.1 with VTM configuration, for AI, RA, LB, and LP configurations.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “JVET common test conditions and software reference configurations for SDR video,” F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-K1010.

- “Description of Core Experiment 1 (CE1): Partitioning,” J. Ma, F. Le Léannec, M. W. Park, JVET-K1021.

- “CE1-related: CE1-related: Constraint for binary and ternary partitions,” C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, JVET-K0556.

- “Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu, JVET-K1001.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS