| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0317-v1 |
| --- | --- |

| Title: | CE4-related: Sub-block MV clipping in affine prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Min Gao, Xiang Li, Meng Xu, and Shan Liu | Tel: Email: | maxwellgao@tencent.com xlxiangli@tencent.com mengxxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

For affine prediction modes, affine motion compensation is performed on a sub-block basis and the size of sub-block is 4x4. The per-pixel memory bandwidth requirement for 4x4 inter prediction block is quite high, especially when 4x4 bi-directional inter prediction is enabled. To reduce the memory bandwidth in affine prediction, this contribution proposes to constrain the MVs of four 4x4 sub-blocks within one 8x8 block such that the max difference between integer parts of the four 4x4 sub-block MVs is no more than 1 pixel. It is reported that the worst case memory bandwidth requirement by affine is reduced at the cost of minor BD-rate increase (0.08% and -0.01% for RA and LDB configurations, respectively).

Introduction

Affine prediction is used to capture the non-translational motions e.g. zoom in/out and rotation in VVC [1]. In affine prediction models, a simplified 4-parameter and/or 6-parameter models are applied to the coding block to predict the motion information for all sub-blocks within the current block. To derive motion information of each sub-block, the motion vector of the center sample of each sub-block is calculated with 4-parameter or 6-parameter models using two or three control points. Then the motion compensation interpolation filters are applied to generate the prediction of each sub-block with derived motion vector. In the current implementation of affine prediction, the size of sub-block is 4x4.

In VVC, the uni-directional inter prediction for 4x4 block needs to load up to (4+7)*(4+7)=121 luma samples due to the interpolation requirement. That means the per-pixel memory bandwidth requirement for 4x4 uni-directional inter prediction is 7.6 sample per pixel. If the 4x4 bi-directional inter prediction is allowed, the requirement is up to 15.125 sample/pixel. Therefore, the memory bandwidth requirement in affine prediction is quite high.

Sub-block MV clipping in affine prediction

To reduce the memory bandwidth requirement in affine prediction, each 8x8 block within the block is regarded as the basic unit. The MVs of all four 4x4 sub-blocks inside the 8x8 block are constrained such that the max difference between integer parts of the four 4x4 sub-block MVs is no more than 1 pixel. So that the bandwidth is (8+7+1)*(8+7+1)/(8*8)=4 sample/pixel.

In the proposed method, after the MVs of all sub-blocks inside the current block are calculated with affine model, the MV of the sub-blocks containing the control points are firstly replaced with the corresponding control point MV. This means that, the MV of the top-left, top-right and bottom-left sub-blocks are replaced by the top-left, top-right and bottom-left control points MV, respectively. Then, for each 8x8 block within the current block, the MVs of all four 4x4 sub-blocks are clipped to guarantee the max difference between integer parts of the four MVs no more than 1 pixel. Here it should be noted that the sub-blocks containing the control points (top-left, top-right and bottom-left sub-blocks) use the corresponding control point MV to involve in the MV clipping process. During the clipping process, the MV of the top-right control point is kept un-changed.

The clipping process applied to each 8x8 block is described as follows:

- the minimal and maximal values for the MV components, MVminx, MVminy, MVmaxx, MVmaxy are firstly determined for each 8x8 block as follows:

  - Get the minimal MV component among the four 4x4 sub-block MVs

MVminx = min(MVx0, MVx1, MVx2, MVx3)

MVminy = min(MVy0, MVy1, MVy2, MVy3)

  - Use the integer part of MVminx and MVminy as the minimal MV component

MVminx = MVminx >> MV_precision << MV_precision

MVminy = MVminy >> MV_precision << MV_precision

  - if the top-right control point is in current 8x8 block

if (MV1x > MVmaxx)

MVminx = (MV1x >> MV_precision << MV_precision) – (1 << MV_precision)

MVmaxx = MVminx + (2 << MV_precision) – 1

if (MV1y > MVmaxy)

MVminy = (MV1y >> MV_precision << MV_precision) – (1 << MV_precision)

MVmaxy = MVminy + (2 << MV_precision) – 1

  - The maximal MV component is calculated as follows:

MVmaxx = MVminx + (2 << MV_precision) – 1

MVmaxy = MVminy + (2 << MV_precision) – 1

- The MV components of each 4x4 block inside this 8x8 block are clipped as follows:

MVxi = max(MVminx, min(MVmaxx, MVxi))

MVyi = max(MVminy, min(MVmaxy, MVyi))

where (MVxi, MVyi) is the MV of ith sub-block within one 8x8 block, where i is 0, 1, 2, 3; (MV1x, MV1y) is the MV of the top-right control point; MV_precision is equal to 4 corresponding to 1/16 motion vector fraction accuracy. Since the difference between integer parts of MVminx and MVmaxx (MVminy and MVmaxy) is 1 pixel, the max difference between integer parts of the four 4x4 sub-block MVs is no more than 1 pixel.

Simulation results

The proposed method is implemented on the top of VTM2.0.1. The common test conditions defined in [2] is used in the simulation. Table 1 shows the BD-Rate of the proposed method relative to VTM2.0.1.

Table. 1. The BD-Rate of the proposed method relative to VTM2.0.1.

Conclusion

This contribution proposes to constrain the MVs of four 4x4 sub-blocks within one 8x8 block such that the max difference between integer parts of the four 4x4 sub-block MVs is no more than 1 pixel to keep the worst-case memory bandwidth requirement low. It is proposed to adopt this method.

Reference

[1] B. Bross, J. Chen, S. Liu, JVET-K1001, “Versatile video coding”, July. 2018.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, JVET-K1010, “JVET common test conditions and software reference configurations for SDR video”, July. 2018.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).