| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0322 |
| --- | --- |

| Title: | CE4-related: constrained model-based affine merge |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, Xiaozhong Xu, Xiang Li, and Shan Liu 661 Bryant St, Palo Alto, CA 94301 USA | Tel: Email: | +1-650-798-3300 guichunli@tencent.com xiaozhongxu@tencent.com xlxiangli@tencent.com shanl@tencent.com |
| Source: | Tencent America |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to apply a constraint on model-based affine merge candidates to avoid storing extra affine related information in motion data line buffer for an entire CTU row. It was reported that with this restriction, 0.14% and 0.03% BD-rate changes on random access and low delay B configurations are observed, respectively.

# Introduction

In current Versatile Video Coding Test Model (VTM-2.0.1), there are 5 positions of spatial candidate for model based affine merge candidates and AMVP candidates are as depicted in Figure 1.

Figure 1 – Model Based Merge Candidates in VTM-2.0.1

A0 is the left-bottom candidate, A1 is the left candidate, B0 is the above-right candidate, B1 is the above candidate, and B2 is the above-left candidate.

In model-based affine merge mode in VTM-2.0.1, the order of model-based spatial merge candidates is {A1, B1, B0, A0, B2}. The first available candidate will be used to derive affine model from the candidate for the current block.

In Affine Merge Common Base software provided in CE4.2.1, the order and positions of model-based spatial merge candidates are also {A1, B1, B0, A0, B2}, and all available spatial candidates are put in an affine merge candidate list to derive the affine model of the current block. Constructed control point based affine merge candidates are added after model-based candidates in the list, if available.

Comparing to regular translational inter merge mode, there’s additional information required from the model-based affine merge candidates when deriving the affine model from them. It is required to have the affine flag of the candidate block, the motion vector values of all control points of the candidate block, as well as width and height of the block. For 4-parameter affine, MV of 2 control points (top-left and top-right) and the width are required. For 6-parameter model, MV of 3 control points (top-left, top-right, and left-bottom), width and height are required.

When the above neighboring candidates are located outside of the current CTU, as candidate B2, B1, and B0 depicted in Figure-2, the additional required information must be stored in the motion date line buffer for the entire CTU row. It requires new motion data line buffer design and increase in size in addition to the line buffer used for regular inter merge and AMVP.

Figure 2 - Model-based merge candidate derivation

# Proposed Method

In order to avoid the additional line buffer storage, the proposed method applies one restriction on model-based affine merge candidate derivation.

In this method, when a neighboring affine candidate block is above the current CTU boundary, it is marked as unavailable, so that it won’t be used in model-based affine candidate derivation. Available neighboring candidates from other positions can still be used for the derivation.

When constructed control-point based method is used, such as in the Affine Merge Common Base software, the control-point based candidates are not affected by the proposed method.

When the proposed method is used, no modification to the motion data line buffer is necessary.

# Simulation Results

The proposed method is implemented on top of VTM-2.0.1 plus affine merge common base software, as provided in CE4.2.1 [1][4]. Simulation tests were performed with common test condition [2].

VTM-2.0.1 affine merge common base software with common test condition is used as anchor. VTM-2.0.1 software [3] with common test condition is used as a second reference.

## Simulation result

Simulation results are provided in the following tables. It was reported that there are 0.14% and 0.03% BD-rate loss on random access and low delay B configuration, respectively.

Table 1 -  Simulation results of Random Access Configuration

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 Affine Merge Common Base |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.14% | 0.07% | 97% | 98% | -0.86% | -0.45% | -0.83% | 100% | 103% |
| Class A2 | 0.40% | 0.33% | 0.35% | 97% | 97% | -0.61% | -0.65% | -0.79% | 102% | 102% |
| Class B | 0.10% | 0.12% | 0.07% | 96% | 95% | -0.57% | -0.45% | -0.63% | 104% | 101% |
| Class C | 0.03% | 0.12% | 0.09% | 94% | 89% | -0.46% | -0.34% | -0.45% | 105% | 102% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.14% | 0.16% | 0.13% | 96% | 94% | -0.61% | -0.46% | -0.65% | 103% | 102% |
| Class D | 0.04% | 0.02% | 0.07% | 93% | 86% | -0.64% | -0.65% | -0.70% | 105% | 105% |
| Class F (optional) | 0.10% | 0.14% | 0.07% | 94% | 91% | -0.04% | -0.04% | -0.07% | 102% | 96% |

Table 2 - Simulation results of Low Delay B Configuration

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 Affine Merge Common Base |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.05% | 0.04% | 0.32% | 99% | 103% | -0.33% | -0.24% | -0.32% | 104% | 101% |
| Class C | 0.01% | 0.23% | 0.00% | 95% | 94% | -0.34% | -0.16% | -0.26% | 103% | 96% |
| Class E | 0.00% | 0.04% | -0.51% | 95% | 94% | -0.64% | -0.60% | -0.70% | 104% | 100% |
| Overall | 0.03% | 0.10% | 0.00% | 97% | 98% | -0.41% | -0.30% | -0.40% | 103% | 99% |
| Class D | 0.08% | 0.49% | 0.71% | 95% | 96% | -0.52% | 0.05% | 0.10% | 105% | 106% |
| Class F (optional) | 0.00% | 0.20% | 0.40% | 96% | 94% | -0.09% | 0.40% | 0.38% | 103% | 96% |

# Conclusion

It was reported that the proposed constraint on affine merge candidate can completely avoid additional line buffer storage for implementing affine merge mode. The impact on BD rate brought by this change is minor.

# References

[1] H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-K1024, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

[3] VTM-2.0.1 software: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-2.0.1

[4] VTM-2.0.1 with Affine Merge Common Base: https://vcgit.hhi.fraunhofer.de/JVET-K-CE4/VVCSoftware_VTM/tree/CE4.2.1

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).