| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L0324-v1 |
| --- | --- |

| Title: | CE3: Intra reference sample interpolation filter selection using MDIS conditions (Test 3.1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Adarsh Krishnan Ramasubramonian Vadim Seregin Ted Hsieh Marta Karczewicz | Email: | geertv@qti.qualcomm.com aramasub@qti.qualcomm.com vseregin@qti.qualcomm.com thsieh@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The MDIS (Mode Dependent Intra Smoothing) filter in VTM2 is applied to the intra reference samples depending on the MDIS conditions. This requires storing unfiltered and filtered lines of intra reference samples. In JEM7, 4-tap cubic and 4-tap Gaussian interpolation filters for prediction of directional intra modes are included. However, depending on the MDIS conditions and interpolation filter switching conditions it may occur that two smoothing filtering operations, MDIS and Gaussian interpolation, are applied consecutively. Therefore, it is proposed to merge the two smoothing filtering operations into a single 4-tap Gaussian interpolation filter, while the MDIS conditions are employed to decide between the cubic and Gaussian interpolation filters. This way only the unfiltered reference samples needs to be stored for prediction. The reported BD-rate results over the VTM-2.0.1 anchor are:

- AI: −0.46% (Y), −0.58% (U), −0.61% (V) with runtimes 103% (Enc) and 102% (Dec)

- RA: −0.19% (Y), −0.14% (U), −0.13% (V) with runtimes 103% (Enc) and 103% (Dec)

- LB: −0.10% (Y), −0.32% (U), −0.42% (V) with runtimes 101% (Enc) and 100% (Dec)

Additional results obtained with a reduced number of bits per filter tap for the 4-tap cubic and 4-tap Gaussian filters are also reported:

- AI: −0.45% (Y), −0.60% (U), −0.62% (V) with runtimes 102% (Enc) and 102% (Dec)

- RA: −0.17% (Y), −0.09% (U), −0.11% (V) with runtimes 102% (Enc) and 102% (Dec)

- LB: −0.10% (Y), −0.14% (U), −0.50% (V) with runtimes 101% (Enc) and 100% (Dec)

It is observed that the latter BD-rate results are comparable to the former results and, therefore, the second set of simplified filters may be preferable in an implementation.

If the MDIS tool in the VTM-2.0.1 anchor is disabled, because the proposed tool overlaps with the VTM2 tool, then the following BD-rates are reported for the simplified filters:

- AI: −0.66% (Y), −0.73% (U), −0.75% (V) with runtimes 104% (Enc) and 100% (Dec)

- RA: −0.35% (Y), −0.39% (U), −0.39% (V) with runtimes 101% (Enc) and 99% (Dec)

- LB: −0.11% (Y), −0.37% (U), −0.18% (V) with runtimes 101% (Enc) and 100% (Dec)

Three cross-check reports are included in the CE3 summary report JVET-L0023.

# Problem Description

The MDIS (Mode Dependent Intra Smoothing) filter in VTM2 [1][2], i.e., ([1 2 1] + 2)/4, is applied to the intra reference samples depending on the MDIS conditions. This requires storing unfiltered and filtered lines of intra reference samples for intra prediction. Subsequently, linear interpolation is applied to the reference sample line in case of fractional pixel prediction for directional modes.

JEM7 [3] includes the ‘Intra4Tap’ tool that replaces the linear interpolation with a combination of two 4-tap, 32-phase interpolation filters: cubic and Gaussian. The main objective of this tool is to improve compression efficiency. These interpolation filters are applied to either the unfiltered or filtered reference samples depending on the separate conditions associated with the ‘Intra4Tap’ tool in JEM7. The problem is that in case Gaussian interpolation filtering is applied on top of the MDIS filtered reference samples, two smoothing filtering operations are applied consecutively. This means that complexity is increased, and additional storage is required for the unfiltered and filtered intra reference sample lines.

# Proposal

As in JEM7, the test CE3-3.1.2 [4] described in this document proposes to replace the linear interpolation with a combination of two 4-tap, 32-phase interpolation filters: cubic and Gaussian. The 4-tap cubic filter is the filter from JEM7, while the proposed 4-tap Gaussian filter has increased smoothing strength compared with the filter utilized in JEM7. This way it is feasible to merge the two smoothing filtering operations for intra prediction of directional modes: MDIS filtering and Gaussian interpolation filtering. The second difference from the JEM7 tool is that the MDIS conditions are employed to decide between the two interpolation filters. This way only the unfiltered reference sample line needs to be stored for MDIS, which reduces memory requirements in certain implementations. A third difference is that the ([1 2 1] + 2)/4 filtering in case of non-fractional sample prediction is avoided by using the zero-phase interpolation filter taps. The details of the proposed algorithm are described next.

In VTM2 the following MDIS table is included:

Table 1: MDIS thresholds (VTM2)

| sizeIndex | MDISthreshold[sizeIndex] |
| --- | --- |
| 0 | 20 |
| 1 | 20 |
| 2 | 20 |
| 3 | 14 |
| 4 | 2 |
| 5 | 0 |
| 6 | 20 → 0 (*) |
| 7 | 0 |

(*) In this proposal the MDISthreshold corresponding with sizeIndex=6 is modified to value 0.

The block size index is defined as follows in VTM2:

sizeIndex = ( log2( BlockWidth ) + log2( BlockHeight ) ) / 2

VTM2 defines the minimum index difference between the directional intra mode and respectively the indices of horizontal and vertical modes as follows:

IntraModeDiff = min( abs( IntraModeIdx - HOR_IDX ), abs( IntraModeIdx – VER_IDX ) )

The proposed condition for switching the interpolation filters in directional intra prediction is the following:

IF ( IntraModeDiff > MDISthreshold[sizeIndex] )

THEN “apply cubic interpolation filter”

ELSE “apply Gaussian interpolation filter”

In this algorithm, the Gaussian interpolation filter, which has smoothing properties, is applied as the only smoothing filtering step. The cubic and Gaussian interpolation filters are applied for luma prediction only, while for chroma prediction the linear interpolation is utilized as in VTM2.

In test CE3-3.1.2, the cubic interpolation filter is the 32-phase 4-tap filter from JEM7 with filter taps enumerated in Table 2(a).

Table 2: 4-tap cubic interpolation filters

| - CE3-3.1.2 cubic filter: { 0, 256, 0, 0 }, // 0 { -3, 252, 8, -1 }, // 1 { -5, 247, 17, -3 }, // 2 { -7, 242, 25, -4 }, // 3 { -9, 236, 34, -5 }, // 4 { -10, 230, 43, -7 }, // 5 { -12, 224, 52, -8 }, // 6 { -13, 217, 61, -9 }, // 7 { -14, 210, 70, -10 }, // 8 { -15, 203, 79, -11 }, // 9 { -16, 195, 89, -12 }, // 10 { -16, 187, 98, -13 }, // 11 { -16, 179, 107, -14 }, // 12 { -16, 170, 116, -14 }, // 13 { -17, 162, 126, -15 }, // 14 { -16, 153, 135, -16 }, // 15 { -16, 144, 144, -16 }, // 16 { -16, 135, 153, -16 }, // 17 { -15, 126, 162, -17 }, // 18 { -14, 116, 170, -16 }, // 19 { -14, 107, 179, -16 }, // 20 { -13, 98, 187, -16 }, // 21 { -12, 89, 195, -16 }, // 22 { -11, 79, 203, -15 }, // 23 { -10, 70, 210, -14 }, // 24 { -9, 61, 217, -13 }, // 25 { -8, 52, 224, -12 }, // 26 { -7, 43, 230, -10 }, // 27 { -5, 34, 236, -9 }, // 28 { -4, 25, 242, -7 }, // 29 { -3, 17, 247, -5 }, // 30 { -1, 8, 252, -3 }, // 31 | - CE3-3.1.2 simplified cubic filter: { 0, 64, 0, 0 }, { -1, 63, 2, 0 }, { -1, 62, 4, -1 }, { -2, 61, 6, -1 }, { -3, 59, 9, -1 }, { -3, 58, 11, -2 }, { -3, 56, 13, -2 }, { -3, 54, 15, -2 }, { -4, 53, 18, -3 }, { -4, 51, 20, -3 }, { -4, 49, 22, -3 }, { -5, 47, 25, -3 }, { -4, 45, 27, -4 }, { -4, 43, 29, -4 }, { -5, 41, 32, -4 }, { -4, 38, 34, -4 }, { -4, 36, 36, -4 }, { -4, 34, 38, -4 }, { -5, 32, 41, -4 }, { -4, 29, 43, -4 }, { -4, 27, 45, -4 }, { -4, 25, 47, -4 }, { -3, 22, 49, -4 }, { -3, 20, 51, -4 }, { -3, 18, 53, -4 }, { -2, 15, 54, -3 }, { -2, 13, 56, -3 }, { -2, 11, 58, -3 }, { -2, 9, 59, -2 }, { -1, 6, 61, -2 }, { -1, 4, 62, -1 }, { 0, 2, 63, -1 } | - CE3-3.1.4 cubic filter: { 0, 64, 0, 0 }, { -1, 63, 2, 0 }, { -2, 62, 4, 0 }, { -2, 60, 7, -1 }, { -2, 58, 10, -2 }, { -3, 57, 12, -2 }, { -4, 56, 14, -2 }, { -4, 55, 15, -2 }, { -4, 54, 16, -2 }, { -5, 53, 18, -2 }, { -6, 52, 20, -2 }, { -6, 49, 24, -3 }, { -6, 46, 28, -4 }, { -5, 44, 29, -4 }, { -4, 42, 30, -4 }, { -4, 39, 33, -4 }, { -4, 36, 36, -4 }, { -4, 33, 39, -4 }, { -4, 30, 42, -4 }, { -4, 29, 44, -5 }, { -4, 28, 46, -6 }, { -3, 24, 49, -6 }, { -2, 20, 52, -6 }, { -2, 18, 53, -5 }, { -2, 16, 54, -4 }, { -2, 15, 55, -4 }, { -2, 14, 56, -4 }, { -2, 12, 57, -3 }, { -2, 10, 58, -2 }, { -1, 7, 60, -2 }, { 0, 4, 62, -2 }, { 0, 2, 63, -1 } |
| --- | --- | --- |

The proposed 32-phase 4-tap Gaussian filter with increased smoothing strength compared with the JEM7 filter is included in Table 3(a).

Table 3: 4-tap Gaussian interpolation filters

| - CE3-3.1.2 Gaussian filter: { 64, 128, 64, 0 }, { 61, 116, 67, 12 }, { 59, 115, 69, 13 }, { 57, 115, 71, 13 }, { 55, 114, 73, 14 }, { 53, 113, 75, 15 }, { 50, 112, 78, 16 }, { 47, 111, 80, 18 }, { 45, 110, 82, 19 }, { 43, 109, 84, 20 }, { 41, 108, 86, 21 }, { 40, 106, 88, 22 }, { 37, 105, 90, 24 }, { 36, 103, 92, 25 }, { 34, 101, 94, 27 }, { 32, 100, 96, 28 }, { 30, 98, 98, 30 }, { 28, 96, 100, 32 }, { 27, 94, 101, 34 }, { 25, 92, 103, 36 }, { 24, 90, 105, 37 }, { 22, 88, 106, 40 }, { 21, 86, 108, 41 }, { 20, 84, 109, 43 }, { 19, 82, 110, 45 }, { 18, 80, 111, 47 }, { 16, 78, 112, 50 }, { 15, 75, 113, 53 }, { 14, 73, 114, 55 }, { 13, 71, 115, 57 }, { 13, 69, 115, 59 }, { 12, 67, 116, 61 }, | - CE3-3.1.2 simplified Gaussian filter: { 16, 32, 16, 0 }, { 15, 29, 17, 3 }, { 15, 29, 17, 3 }, { 14, 29, 18, 3 }, { 13, 29, 18, 4 }, { 13, 28, 19, 4 }, { 13, 28, 19, 4 }, { 12, 28, 20, 4 }, { 11, 28, 20, 5 }, { 11, 27, 21, 5 }, { 10, 27, 22, 5 }, { 9, 27, 22, 6 }, { 9, 26, 23, 6 }, { 9, 26, 23, 6 }, { 8, 25, 24, 7 }, { 8, 25, 24, 7 }, { 8, 24, 24, 8 }, { 7, 24, 25, 8 }, { 7, 24, 25, 8 }, { 6, 23, 26, 9 }, { 6, 23, 26, 9 }, { 6, 22, 27, 9 }, { 5, 22, 27, 10 }, { 5, 21, 27, 11 }, { 5, 20, 28, 11 }, { 4, 20, 28, 12 }, { 4, 19, 28, 13 }, { 4, 19, 28, 13 }, { 4, 18, 29, 13 }, { 3, 18, 29, 14 }, { 3, 17, 29, 15 }, { 3, 17, 29, 15 } | - CE3-3.1.4 Gaussian filter: { 16, 32, 16, 0 }, { 15, 29, 17, 3 }, { 14, 29, 18, 3 }, { 14, 29, 18, 3 }, { 14, 28, 18, 4 }, { 13, 28, 19, 4 }, { 12, 28, 20, 4 }, { 12, 27, 20, 5 }, { 11, 27, 21, 5 }, { 11, 27, 21, 5 }, { 10, 26, 22, 6 }, { 10, 26, 22, 6 }, { 9, 26, 23, 6 }, { 9, 26, 23, 6 }, { 8, 25, 24, 7 }, { 8, 25, 24, 7 }, { 7, 25, 25, 7 }, { 7, 24, 25, 8 }, { 7, 24, 25, 8 }, { 6, 23, 26, 9 }, { 6, 23, 26, 9 }, { 6, 22, 26, 10 }, { 6, 22, 26, 10 }, { 5, 21, 27, 11 }, { 5, 21, 27, 11 }, { 5, 20, 27, 12 }, { 4, 20, 28, 12 }, { 4, 19, 28, 13 }, { 4, 18, 28, 14 }, { 3, 18, 29, 14 }, { 3, 18, 29, 14 }, { 3, 17, 29, 15 } |
| --- | --- | --- |

Note that in case integer reference sample locations are computed, the zero-phase of both filters is used. Note also that the MDIS condition in VTM2 enables smoothing of reference samples for planar mode. For planar mode, the ([1 2 1] + 2) / 4 filtering can be implemented together with the planar prediction algorithm, hence, no additional storage of an additional filtered reference samples is required.

In addition to the cubic filter in Table 2(a) and the Gaussian filter in Table 3(a), the number of bits required to store the filters is reduced with minimal impact on the measured BD-rates as is reported in Section 3. For the cubic filter, the number of bits per filter tap is reduced from 9 bits to 7 bits in Table 2(b), and for the Gaussian filter, the number of bits is reduced from 7 bits to 5 bits in Table 3(b). For comparison, the cubic and Gaussian filters utilized in the test CE3-3.1.4 [6] are enumerated in Table 2(c) and Table 3(c). Section 3 also reports the results obtained with the filters from CE3-3.1.4.

# Results

The following results are obtained according to the JVET common test conditions [5] and the CE3 test guidelines [4]. The BMS2.0.1 software is used for the VTM2.0.1 results. The VTM2.0.1 anchors are also generated using the BMS2.0.1 software.

## CE3-3.1.2: AI, RA, LB test conditions

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.14% | -0.19% | 103% | 101% |
| Class A2 | -0.17% | -0.19% | -0.24% | 104% | 101% |
| Class B | -0.43% | -0.60% | -0.56% | 103% | 103% |
| Class C | -0.74% | -0.86% | -0.94% | 102% | 104% |
| Class E | -0.85% | -1.01% | -1.06% | 101% | 102% |
| Overall | -0.46% | -0.58% | -0.61% | 103% | 102% |
| Class D | -0.55% | -0.49% | -0.67% | 101% | 101% |
| Class F (optional) | -0.28% | -0.59% | -0.44% | 102% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.11% | -0.07% | 101% | 100% |
| Class A2 | -0.10% | -0.13% | -0.12% | 103% | 102% |
| Class B | -0.22% | -0.16% | -0.13% | 104% | 104% |
| Class C | -0.29% | -0.15% | -0.19% | 103% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | -0.14% | -0.13% | 103% | 103% |
| Class D | -0.24% | -0.30% | -0.01% | 101% | 103% |
| Class F (optional) | 0.04% | 0.04% | 0.13% | 102% | 104% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.17% | -0.19% | 100% | 101% |
| Class C | -0.08% | -0.19% | 0.01% | 102% | 99% |
| Class E | -0.20% | -0.75% | -1.39% | 100% | 101% |
| Overall | -0.10% | -0.32% | -0.42% | 101% | 100% |
| Class D | -0.08% | 0.44% | 0.14% | 100% | 98% |
| Class F (optional) | -0.20% | -0.12% | 0.02% | 100% | 99% |

## CE3-3.1.2 with reduced bit/tap filters from Table 2(b) and Table 3(b): AI, RA, LB test conditions

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.13% | -0.21% | 104% | 101% |
| Class A2 | -0.15% | -0.21% | -0.25% | 103% | 102% |
| Class B | -0.42% | -0.58% | -0.53% | 102% | 103% |
| Class C | -0.75% | -0.94% | -0.97% | 102% | 104% |
| Class E | -0.83% | -1.02% | -1.05% | 101% | 102% |
| Overall | -0.45% | -0.60% | -0.62% | 102% | 102% |
| Class D | -0.52% | -0.49% | -0.70% | 101% | 100% |
| Class F (optional) | -0.24% | -0.59% | -0.63% | 102% | 104% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.13% | -0.02% | 100% | 99% |
| Class A2 | -0.08% | -0.07% | -0.17% | 102% | 101% |
| Class B | -0.20% | 0.00% | -0.10% | 103% | 103% |
| Class C | -0.27% | -0.19% | -0.15% | 103% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.17% | -0.09% | -0.11% | 102% | 102% |
| Class D | -0.23% | -0.11% | -0.05% | 102% | 104% |
| Class F (optional) | 0.10% | 0.07% | 0.05% | 101% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.03% | -0.30% | 100% | 101% |
| Class C | -0.09% | -0.25% | -0.35% | 102% | 99% |
| Class E | -0.21% | -0.28% | -1.02% | 100% | 100% |
| Overall | -0.10% | -0.14% | -0.50% | 101% | 100% |
| Class D | -0.11% | 0.15% | -0.25% | 100% | 98% |
| Class F (optional) | -0.02% | 0.13% | 0.34% | 100% | 100% |

## CE3-3.1.2 with reduced bit/tap filters from Table 2(b) and Table 3(b): AI, RA, LB test conditions (anchor with MDIS disabled)

For these results, the anchor is modified by disabling the MDIS tool in VTM2.0.1. This way the full gain of the proposed tool is revealed.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 (MDIS disabled) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.72% | -0.66% | -0.79% | 105% | 100% |
| Class A2 | -0.44% | -0.49% | -0.43% | 104% | 102% |
| Class B | -0.68% | -0.75% | -0.71% | 105% | 99% |
| Class C | -0.58% | -0.73% | -0.87% | 103% | 100% |
| Class E | -0.91% | -1.01% | -0.93% | 104% | 99% |
| Overall | -0.66% | -0.73% | -0.75% | 104% | 100% |
| Class D | -0.54% | -0.68% | -0.58% | 104% | 101% |
| Class F (optional) | -0.04% | -0.28% | -0.47% | 104% | 101% |

|  | Random Access Main 10 (MDIS disabled) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.41% | -0.86% | -0.60% | 100% | 99% |
| Class A2 | -0.24% | -0.36% | -0.35% | 101% | 99% |
| Class B | -0.40% | -0.17% | -0.41% | 101% | 99% |
| Class C | -0.33% | -0.33% | -0.25% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | -0.39% | -0.39% | 101% | 99% |
| Class D | -0.29% | 0.08% | -0.43% | 101% | 99% |
| Class F (optional) | 0.15% | 0.24% | -0.11% | 101% | 100% |

|  | Low delay B Main10 (MDIS disabled) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.13% | -0.17% | -0.16% | 101% | 99% |
| Class C | -0.14% | -0.37% | -0.28% | 102% | 102% |
| Class E | -0.05% | -0.68% | -0.09% | 100% | 100% |
| Overall | -0.11% | -0.37% | -0.18% | 101% | 100% |
| Class D | -0.10% | -0.54% | -0.41% | 100% | 99% |
| Class F (optional) | 0.23% | -0.36% | 0.06% | 100% | 101% |

## CE3-3.1.2 with filters from CE3-3.1.4 (Table 2(c) and Table 3(c)): AI, RA, LB test conditions

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.11% | -0.19% | 103% | 100% |
| Class A2 | -0.15% | -0.16% | -0.21% | 103% | 99% |
| Class B | -0.39% | -0.55% | -0.54% | 102% | 102% |
| Class C | -0.76% | -1.05% | -0.92% | 102% | 106% |
| Class E | -0.72% | -0.95% | -1.07% | 101% | 101% |
| Overall | -0.43% | -0.59% | -0.60% | 102% | 102% |
| Class D | -0.54% | -0.44% | -0.81% | 100% | 100% |
| Class F (optional) | -0.21% | -0.57% | -0.56% | 101% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.09% | 0.03% | 100% | 99% |
| Class A2 | -0.05% | 0.03% | -0.08% | 102% | 101% |
| Class B | -0.17% | -0.04% | -0.15% | 103% | 103% |
| Class C | -0.25% | -0.11% | -0.25% | 104% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.15% | -0.02% | -0.13% | 103% | 103% |
| Class D | -0.25% | -0.57% | -0.12% | 102% | 105% |
| Class F (optional) | 0.13% | 0.07% | -0.01% | 102% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.10% | 0.25% | 100% | 99% |
| Class C | -0.04% | -0.10% | -0.19% | 102% | 98% |
| Class E | -0.09% | -0.46% | 0.22% | 99% | 98% |
| Overall | -0.05% | -0.10% | 0.10% | 101% | 99% |
| Class D | -0.06% | 0.52% | 0.21% | 100% | 101% |
| Class F (optional) | -0.14% | -0.10% | -0.13% | 100% | 100% |

# Conclusion

It is suggested to adopt the proposed 4-tap, 32-phase cubic and Gaussian interpolation filtering method using MDIS conditions into the VVC (Draft 3) and VTM3 software. The test results for the filters with reduced number of bits per filter tap over the VTM-2.0.1 anchor are:

- AI: −0.45% (Y), −0.60% (U), −0.62% (V) with runtimes 102% (Enc) and 102% (Dec)

- RA: −0.17% (Y), −0.09% (U), −0.11% (V) with runtimes 102% (Enc) and 102% (Dec)

- LB: −0.10% (Y), −0.14% (U), −0.50% (V) with runtimes 101% (Enc) and 100% (Dec)

If the MDIS tool in the VTM-2.0.1 anchor is disabled, because the proposed tool overlaps with the VTM2 tool, then the following BD-rates are reported for the simplified filters:

- AI: −0.66% (Y), −0.73% (U), −0.75% (V) with runtimes 104% (Enc) and 100% (Dec)

- RA: −0.35% (Y), −0.39% (U), −0.39% (V) with runtimes 101% (Enc) and 99% (Dec)

- LB: −0.11% (Y), −0.37% (U), −0.18% (V) with runtimes 101% (Enc) and 100% (Dec)

# Acknowledgements

The proponents are grateful to HHI, Panasonic, Huawei for performing a cross-check.

# References

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 2 (VTM2)”, 11th JVET Meeting, Ljubljana, SI, July 2018, JVET-K1002

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 2),” 11th JVET Meeting, Ljubljana, SI, July 2018, JVET-K1001

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of Joint Exploration Test Model 7,” 7th JVET Meeting, Torino, Italy, July 2017, JVET-G1001

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” 11th JVET Meeting, Ljubljana, SI, July 2018, JVET-K1023

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” 11th JVET Meeting, Ljubljana, SI, July 2018, JVET-K1010

- A. Filippov, V. Rufitskiy, J. Chen, “CE3: Intra reference sample interpolation filter (Test 3.1.4),” 12th JVET Meeting, Macau, CN, October 2018, JVET-L0275

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).