| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0338_r1 |
| --- | --- |

| Title: | CE3: Multi-directional LM (MDLM) (Test 5.4.1 and 5.4.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Ma, Haitao Yang, Jianle Chen | Email: | maxiang6@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the test results of multi-directional LM (MDLM) related, including MDLM and MDLM with line buffer constraint. In MDLM, the linear model parameters are derived only based on the top or left neighboring samples. On top of VTM2.0.1, the following two tests are conducted:

- CCLM+MDLM

- CCLM+MDLM with line buffer constraint.

Simulation results reportedly show BD rate on Y, Cb and Cr components over VTM2.0.1:

- CCLM+MDLM:

  - For AI configuration : -0.06%, -2.71%, and -3.13%, with 101% EncT, 99% DecT;

  - For RA configuration : -0.03%, -2.65%, and -2.84%, with 100% EncT, 99% DecT;

- CCLM+MDLM with line buffer constraint:

  - For AI configuration : 0.07%, -1.52%, and -1.81%, with 101% EncT, 99% DecT;

  - For RA configuration : 0.04%, -1.55%, and -1.77%, with 100% EncT, 99% DecT;

Tests description

MDLM description

Besides the above template and left template can be used to calculate the linear model coefficients together, they also can be used alternatively in the other 2 LM modes, called LM_A, and LM_L modes.

In LM_A mode, only the above template are used to calculate the linear model coefficients. To get more samples, the above template are extended to (W+H).

Figure 1: LM_A mode

In LM_L mode, only left template are used to calculate the linear model coefficients. To get more samples, the left template are extended to (H+W).

Figure 2: LM_L mode

For a non-square block, the above template are extended to W+W, the left template are extended to H+H.

If the above/left template is not available, then the LM_A/LM_L mode will not be checked or signaled. If the available samples not enough, the template will be padded by copying the right-most (for top template) sample or the below-most (for left template) sample to the nearest log2 number.

MDLM with line buffer constraint

In current LM coefficients derivation process, 2 luma line buffer will be used for down-sampling to get the top template of LM mode (CCLM, or MDLM), while only 1 luma line buffer will be used in luma component intra prediction. To reduce the line buffer, for the CU along with the CTU top boundary, only LM_L mode will be used. In this case, no additional line buffer will be needed.

All tests

There are 2 MDLM related tests totally:

Test 5.4.1: CCLM+MDLM

Test 5.4.2: CCLM+MDLM with line buffer constraint

Experimental results

The proposed tests are implemented based on BMS2.0.1, and tested under VVC CTC [1] with VTM configuration. The results are summarized in Tabel1, Table2, Tabel3, and Table4.

Test 5.4.1: CCLM+MDLM

Table 1: test results of CCLM+MDLM (AI configuration)

Table 2: test results of CCLM+MDLM (RA configuration)

Test 5.4.2: CCLM+MDLM with line buffer constraint

Table 3: test results of CCLM+MDLM with line buffer constraint (AI configuration)

Table 4: test results of CCLM+MDLM with line buffer constraint (RA configuration)

Conclusions

This proposal presents the results of MDLM related tests. It can be seen that, at AI configuration over VTM2.0.1, MDLM test results is -0.06%, -2.71%, and -3.13% on Y, Cb and Cr components. And MDLM with line buffer constraint test results is 0.07%, -1.52%, and -1.81% on Y, Cb and Cr components.

References

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET- K1010, Jul. 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).