| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0339_r1 |
| --- | --- |

| Title: | CE3: CCLM/MDLM coefficients derivation method using one luma line buffer (Test 5.5.1 and 5.5.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Ma, Haitao Yang, Jianle Chen | Email: | maxiang6@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the test results of CCLM and multi-directional LM (MDLM) using one luma line buffer. In which, only 1 luma line will be used to get the top template samples in CCLM mode, or MDLM mode. The tests including:

- CCLM, using 1 luma line buffer

- CCLM + MDLM, both using 1 luma line buffer

Simulation results reportedly show BD rate on Y, Cb and Cr components over VTM2.0.1:

- CCLM, using 1 luma line buffer:

  - For AI configuration: 0.02%, 0.24%, and 0.23%, with 100% EncT, 98% DecT;

  - For RA configuration: 0.03%, 0.32%, and 0.35%, with 100% EncT, 100% DecT;

- CCLM + MDLM, both using 1 luma line buffer:

  - For AI configuration: -0.04%, -2.62%, and -2.98%, with 101% EncT, 99% DecT;

  - For RA configuration: -0.02%, -2.45%, and -2.69%, with 101% EncT, 99% DecT;

Tests description

MDLM description

Besides the above template and left template can be used to calculate the linear model coefficients together, they also can be used alternatively in the other 2 LM modes, called LM_A, and LM_L modes.

In LM_A mode, only the above template are used to calculate the linear model coefficients. To get more samples, the above template are extended to (W+H).

Figure 1: LM_A mode

In LM_L mode, only left template are used to calculate the linear model coefficients. To get more samples, the left template are extended to (H+W).

Figure 2: LM_L mode

For a non-square block, the above template are extended to W+W, the left template are extended to H+H.

If the above/left template is not available, then the LM_A/LM_L mode will not be checked or signaled. If the available samples not enough, the template will be padded by copying the right-most (for top template) sample or the below-most (for left template) sample to the nearest log2 number.

CCLM/MDLM using 1 luma line buffer

In current LM coefficients derivation process, 2 luma line buffer will be used for down-sampling to get the top template of LM-related mode (CCLM, or MDLM), while only 1 luma line buffer will be used in luma component intra prediction.

To reduce the line buffer, for the CUs using CCLM/MDLM mode, only 1 luma line will be used to get the top template samples.

All tests

There are 2 related tests totally:

Test 5.5.1: CCLM, using 1 luma line buffer

Test 5.5.2: CCLM + MDLM, both using 1 luma line buffer

Experimental results

The proposed tests are implemented based on BMS2.0.1, and tested under VVC CTC [1] with VTM configuration. The results are summarized in Tabel1, Table2, Tabel3, and Table4.

Test 5.5.1: CCLM, using 1 luma line buffer

Table 1: test results of CCLM, using 1 luma line buffer (AI configuration)

Table 2: test results of CCLM, using 1 luma line buffer (RA configuration)

Test 5.5.2: CCLM + MDLM, both using 1 luma line buffer

Table 3: test results of CCLM + MDLM, both using 1 luma line buffer (AI configuration)

Table 4: test results of CCLM + MDLM, both using 1 luma line buffer (RA configuration)

Conclusions

This proposal presents the results of CCLM/MDLM using 1 luma line buffer tests. It can be seen that, at AI configuration over VTM2.0.1, CCLM using 1 luma line buffer only introduce 0.02%, 0.24%, and 0.23% coding loss on Y, Cb and Cr components. And CCLM+MDLM using 1 luma line buffer with line buffer constraint can provide 0.04%, 2.62%, and 2.98% coding gain on Y, Cb and Cr components.

References

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET- K1010, Jul. 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).