| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0344 |
| --- | --- |

| Title: | CE15-1: Palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Jicheng An, Jian Lou Yung-Hsuan Chao, Hongtao Wang, Vadim Seregin, Marta Karczewicz | Tel: Email: | +1-858-349-2818 yuchen.s@alibaba-inc.com +1-858-845-2163 yunghsua@qti.qualcomm.com |
| Source: | Alibaba Cloud Computing Ltd. Qualcomm |  |  |

_____________________________

- Abstract

This document reports the results of CE test 15.1 on palette mode. A simplified palette mode in JVET-K0411 is tested on top of BMS-2.0.1. For Class F sequences, in AI/RA/LDB configuration, the results show -10.3%/-8.0%/-4.8% BD-rate luma gain with 122%/117%/117% encoding time and 92%/99%/105% decoding time; for 4:2:0 TGM sequences in CE15, the results show -30.4%/-14.6%/-6.4% BD-rate luma gain with 138%/124%/123% encoding time and 70%/92%/100% decoding time in AI/RA/LDB configuration.

Introduction

This document reports the results of CE15 sub-tests 1 (CE15.1) on palette mode. A simplified palette mode in JVET-K0411 is tested on top of BMS2.0.1.

Description of techniques

A flag is transmitted for each CU to signal whether the palette mode is used in the current CU. If the palette mode is utilized, the pixels with pixel values close to the palette colors are represented by the selected palette colors denoted by the palette indices, where the corresponding palette colors are the representative colors in the current CU. The remaining pixels are presented by a special palette index value, denoted as an escape index, and their pixel values are signalled directly

To decode a palette encoded block, the decoder needs to decode palette colors and indices. Palette colors are described by a palette table and encoded by palette table coding tools. Based on the palette table, a pixel of three color components is represented by a palette index. Palette indices of all pixels in a CU form a palette index map and are encoded by palette index map coding tools.

Predictive coding is applied for the palette table. The decoder constructs a palette table predictor to predict the palette table of the current CU. The palettes of the previous palette coded CUs are used as the predictor (palette predictor stuffing). After the palette table is encoded, the pixels in a CU are represented by palette indices corresponding to the colors in the palette. The indices form an index map and are divided into several runs and then encoded in horizontal raster scan order. Figure 1 shows an example of the palette index map coding. In the figure, an index map of 16 indices is encoded by three runs with horizontal raster scan. There are two run modes: copy index mode and copy above mode. For each starting position of a run, a flag is transmitted to indicate which run mode is used.

If the copy above mode is used, a run of pixels will copy the palette indices from the pixels above. In the example in Figure 1, a run, R2, is a copy above run with a run value of 8. If the copy index mode is used, a palette_index_idc syntax element is first signalled, followed by a piece of PaletteIndexRun information. A run of pixels shares the same palette index, where the palette index and the run value are derived from the palette_index_idc and PaletteIndexRun, respectively. For instance, in Fig.1, R0 and R1 are two copy index runs, whose (palette index, run value) are (1, 6) and (2, 2), respectively.

# Figure 1. An example of run-based index map coding.

Simulation results

The proposed method is evaluated according to the CE15 description for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations with the BMS-2.0.1 software.

Conclusion

This document reports the results of CE15 sub-tests 1 on palette mode. A simplified palette mode in JVET-K0411 is tested on top of BMS2.0.1. For Class F sequences, the results show -10.3%/-8.0%/-4.8% BD-rate luma gain in AI/RA/LDB configuration; for 4:2:0 TGM sequences in CE15, the results show -30.4%/-14.6%/-6.4% BD-rate luma gain in AI/RA/LDB configuration.

Reference

- Y.-C. Sun, Y.-H. Chao and X. Xu, "Description of Core Experiment 15: Palette Mode”, JVET-K1035, 2018.

- Y.-C. Sun, J. An, J. Lou, "AHG11: Palette mode ", JVET-K0411, 11th JVET meeting, July 2018.

Patent rights declaration(s)

Alibaba Cloud Computing Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).