| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0346-v1 |
| --- | --- |

| Title: | CE13: Face row based geometry padding of reference pictures (Test 3.1.a and Test 3.1.c) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheng-Hsuan Shih, Jian-Liang Lin, Hung-Chih Lin, Shen-Kai Chang, Yung-Chang Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {ch.shih, jl.lin, hungchih.lin, shenkai.chang, yc.zhang, cc.ju}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of Test 3.1.a and Test 3.1.c in CE13: HEC with face row based geometry padding of reference pictures. The simulation results reportedly show that, in terms of end-to-end WS-PSNR-Y/U/V, HEC with face row based geometry padding of reference pictures using Lanczos 3/2 interpolation (Test 3.1.a) achieves the average -1.61%/-1.10%/-1.06%, and -1.30%/-1.55%/-1.66% BD-rate saving compared to PHEC and HEC, respectively. For HEC with face row based geometry padding of reference pictures using unfolding-based padding without interpolation (Test 3.1.c), the results reportedly show the average -0.97%/-0.52%/-0.46% and -0.66%/-0.97%/-1.06% BD-rate saving compared to PHEC and HEC, respectively. An additional testing of HEC with face row based geometry padding of reference pictures using nearest neighbor rounding is also tested, the results reportedly show the average -1.37%/-0.92%/-0.88% and -1.05%/-1.37%/-1.47% BD-rate saving compared to PHEC and HEC, respectively.

# Introduction

In the last meeting, Core Experiment 13 (CE13) [1] was established to study removing subjective artifacts at face boundaries for the 360° video category. This contribution reports the results of face row based geometry padding of reference pictures which has been proposed in JVET-E0057 [4], JVET-J0019 [2], and JVET-K0141 [3].

In conventional video coding systems, the replication padding for reference pixel data is used in inter prediction when the reference region is outside the frame boundary. Fig. 1 shows a typical implementation for replication padding which extends the boundary pixels in the border region.

Fig. 1 Replication padding in conventional video system

In this contribution, the spherical-based inter prediction for cubemap projection is proposed to use the spherical neighboring pixel data for efficient prediction based on the spherical continuity in 360° videos. In the proposed method, the inter prediction replaces the replication padding with the geometry padding when the reference pixels are located outside the frame boundary (layout boundary) or across the discontinuous edge.

# Proposed method

## HEC with face row based geometry padding of reference pictures using projection with Lanczos 3/2 interpolation (Test 3.1.a)

The geometry padding in Fig. 2 demonstrates a pixel position qb qboutside the Bottom face is represented by its spherical corresponding pixel qa, which is located at the Front face. This spherical corresponding pixel is obtained by using the HEC mapping functions. When the derived coordinate qa is fractional, 6-tap and 4-tap Lanczos interpolation filters [6][7] are applied for the luminance and chrominance components, respectively.

Fig. 2 Geometry padding process

To achieve the geometry padding [8][9] in inter prediction, the proposed implementation maximally utilizes face continuity by alternatively constructing the top sub-frame and the bottom sub-frame for each reference frame data, as illustrated in Fig. 3. Each sub-frame buffer has one 3x1 face region with an extended padding area. The padding size is the LCU size with additional 16 pixels, which yields the width of 144 pixels.

Fig. 3 Top and bottom sub-frame buffers: 3x1 face region with geometry padding area

## HEC with face row based geometry padding of reference pictures using unfolding-based padding without interpolation (Test 3.1.c)

In this test, the padding areas of sub-frame buffers are filled by duplicating spherical adjacent faces as illustrated in Fig. 4. An image example of the top sub-frame and the bottom sub-frame for reference frame data is illustrated in Fig. 5.

Fig. 4 Adjacent faces of target face

Fig. 5 Top and bottom sub-frame buffers: 3x1 face region with padding area by using unfolding-based padding without interpolation

## HEC with face row based geometry padding of reference pictures using projection with nearest neighbor rounding

The same geometric padding process in Section 2.1 is conducted in this test except that the interpolation method is replaced by nearest neighbor (NN) rounding [6].

# Experiment results

The experiments are conducted on 360Lib-7.0 [10] and BMS-2.0.1 (with fix BMS ticket #78 and #79) [11] followed by the JVET common test conditions (CTC) and evaluation procedures [12] under the VTM-2.0 [13] configuration setting. Following the CTC, the frame size is set to 3840x2560 with the face size of 1280x1280. Compared to PHEC and HEC, Table I demonstrates the average BD-rate performance in terms of end-to-end WS-PSNR for the proposed face row based geometry padding of reference pictures using projection with Lanczos 3/2 interpolation (Test 3.1.a). Table II demonstrates the average BD-rate performance in terms of end-to-end WS-PSNR for the proposed face row based geometry padding of reference pictures using unfolding-based padding without interpolation (Test 3.1.c). Table III demonstrates the average BD-rate performance in terms of end-to-end WS-PSNR for the proposed face row based geometry padding of reference pictures using projection with nearest neighbor (NN) rounding.

Table I BD-rate performance for HEC with face row based geometry padding of reference pictures using projection with Lanczos 3/2 interpolation compared to PHEC and HEC (Test 3.1.a)

|  | Anchor: PHEC |  |  | Anchor: HEC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -2.79% | -1.76% | -1.67% | -2.82% | -2.74% | -2.79% |
| ChairLift | -2.85% | -2.41% | -2.51% | -2.73% | -2.64% | -2.79% |
| KiteFlite | -0.47% | 0.06% | 0.31% | -0.02% | -0.06% | -0.06% |
| Harbor | 0.00% | 1.42% | 1.83% | -0.03% | -0.09% | -0.04% |
| Trolley | -0.85% | 0.14% | -0.28% | -0.19% | -0.34% | -0.37% |
| GasLamp | -0.54% | 0.27% | 0.41% | -0.06% | -0.10% | -0.05% |
| Balboa | -2.35% | -2.81% | -2.47% | -2.23% | -2.84% | -2.93% |
| Broadway | -1.66% | -1.97% | -1.91% | -1.59% | -2.07% | -2.46% |
| Landing2 | -1.90% | -0.99% | -1.23% | -1.41% | -2.00% | -2.06% |
| BranCastle2 | -2.72% | -2.95% | -3.09% | -1.92% | -2.60% | -3.05% |
| Class S1 | -1.25% | -0.38% | -0.32% | -0.98% | -0.99% | -1.02% |
| Class S2 | -2.16% | -2.18% | -2.18% | -1.79% | -2.38% | -2.63% |
| Overall | -1.61% | -1.10% | -1.06% | -1.30% | -1.55% | -1.66% |

Table II BD-rate performance for HEC with face row based geometry padding of reference pictures using unfolding-based padding without interpolation compared to PHEC and HEC (Test 3.1.c)

|  | Anchor: PHEC |  |  | Anchor: HEC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -1.26% | -1.07% | -1.04% | -1.30% | -2.06% | -2.16% |
| ChairLift | -1.64% | -1.49% | -1.31% | -1.52% | -1.72% | -1.59% |
| KiteFlite | -0.47% | 0.10% | 0.36% | -0.02% | -0.03% | -0.01% |
| Harbor | 0.00% | 1.43% | 1.83% | -0.03% | -0.08% | -0.04% |
| Trolley | -0.81% | 0.22% | -0.26% | -0.15% | -0.27% | -0.34% |
| GasLamp | -0.54% | 0.28% | 0.37% | -0.07% | -0.08% | -0.09% |
| Balboa | -1.32% | -1.74% | -1.76% | -1.20% | -1.77% | -2.22% |
| Broadway | -0.89% | -1.03% | -0.81% | -0.82% | -1.14% | -1.36% |
| Landing2 | -0.98% | 0.03% | -0.23% | -0.48% | -1.00% | -1.07% |
| BranCastle2 | -1.77% | -1.91% | -1.79% | -0.97% | -1.55% | -1.76% |
| Class S1 | -0.79% | -0.09% | -0.01% | -0.51% | -0.71% | -0.71% |
| Class S2 | -1.24% | -1.16% | -1.15% | -0.87% | -1.37% | -1.60% |
| Overall | -0.97% | -0.52% | -0.46% | -0.66% | -0.97% | -1.06% |

Table III BD-rate performance for HEC with face row based geometry padding of reference pictures using projection with nearest neighbor (NN) rounding compared to PHEC and HEC

|  | Anchor: PHEC |  |  | Anchor: HEC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -2.37% | -1.77% | -1.76% | -2.40% | -2.75% | -2.87% |
| ChairLift | -2.39% | -2.07% | -2.15% | -2.28% | -2.30% | -2.42% |
| KiteFlite | -0.45% | 0.07% | 0.25% | 0.00% | -0.05% | -0.13% |
| Harbor | 0.00% | 1.46% | 1.83% | -0.03% | -0.06% | -0.04% |
| Trolley | -0.82% | 0.18% | -0.30% | -0.16% | -0.31% | -0.38% |
| GasLamp | -0.51% | 0.31% | 0.42% | -0.03% | -0.06% | -0.05% |
| Balboa | -1.87% | -2.27% | -2.07% | -1.74% | -2.31% | -2.53% |
| Broadway | -1.24% | -1.64% | -1.46% | -1.17% | -1.75% | -2.01% |
| Landing2 | -1.65% | -0.79% | -0.92% | -1.16% | -1.81% | -1.76% |
| BranCastle2 | -2.37% | -2.69% | -2.59% | -1.57% | -2.34% | -2.55% |
| Class S1 | -1.09% | -0.30% | -0.28% | -0.82% | -0.92% | -0.98% |
| Class S2 | -1.78% | -1.85% | -1.76% | -1.41% | -2.05% | -2.21% |
| Overall | -1.37% | -0.92% | -0.88% | -1.05% | -1.37% | -1.47% |

# Conclusion

In this contribution, the performance of the proposed face row based geometry padding of reference pictures is investigated. The experimental data reports that proposed methods achieve noticeable BD-rate saving and remarkably reduce the visible seam artifacts compared to HEC.

# Introduction

- Philippe Hanhart, Jian-Liang Lin, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K1033, July 2018, Ljubljana, SI.

- J.-L. Lin, Y.-H. Lee, C.-H. Shih, S.-Y. Lin, H.-C. Lin, S.-K. Chang, P. Wang, L. Liu, and C.-C. Ju, “Description of 360° video coding technology proposal by MediaTek,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-J0019, Apr. 2018, San Diego, USA

- Cheng-Hsuan Shih, Jian-Liang Lin, Hung-Chih Lin, Shen-Kai Chang, Chi-Cheng Ju, “AHG8: 360°-based inter/intra prediction for cubemap projection”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K0141, July 2018, Ljubljana, SI.

- Cheng-Hsuan Shih, Hung-Chih Lin, Jian-Liang Lin, Shen-Kai Chang, “AHG8: Face-based padding scheme for cube projection”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-E0057, Jan. 2017, Geneva, CH.

- Y.-H. Lee, J.-L. Lin, S.-K. Chang and C.-C. Ju,”CE13: Modified Cubemap Projection in JVET-J0019 (Test 5),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0131, July 2018, Ljubljana, SI.

- Y. Ye, J. Boyce, "Algorithm descriptions of projection format conversion and video quality metrics in 360Lib Version 7", Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K1004, July 2018, Ljubljana, SI.

- Lanczos resampling, https://en.wikipedia.org/wiki/Lanczos_resampling

- J. Sauer and M.Wien, “Geometry correction for motion compensation of planar-projected 360VR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-D0067, Oct. 2016, Chengdu, China

- Y. He, Y. Ye, P. Hanhart, and X. Xiu , “AHG8: Geometry padding for 360 video coding,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-D0075, Oct. 2016, Chengdu, China

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- VVC software Benchmark Set, https://vcgit.hhi.fraunhofer.de/JVET-K-CE13/VVCSoftware_BMS/

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-K1012, July 2018, Ljubljana, SI.

- Versatile Video Coding Test Model, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).