| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0349-v1 |
| --- | --- |

| Title: | CE13: In-loop filters using spherical neighbors (Test 4.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JEVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sheng-Yen Lin, Lin Liu, Jian-Liang Lin, Hung-Chih Lin, Shen-Kai Chang, Yung-Chang Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {Sheng-Yen.Lin, Lin.Liu, jl.lin, hungchih.lin, shenkai.chang, yc.zhang, cc.ju}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of Test 4.2.a, 4.2.b, 4.2.c and 4.2.d in CE13: HEC with in-loop filters using spherical neighbors. In terms of end-to-end WS-PSNR-Y/U/V, the experimental results reportedly demonstrate that Test 4.2.a, 4.2.b, 4.2.c and 4.2.d achieve an average -0.31%/-0.76%/-0.80%, -0.31%/-0.78%/-0.76%, -0.33%/-1.34%/-1.07% and -0.32%/-1.35%/-1.09% BD-rate saving compared to HEC, respectively, and achieve an average -0.62%/-0.31%/-0.20%, -0.63%/-0.33%/-0.16%, -0.64%/-0.90%/-0.47% and -0.64%/-0.90%/-0.49% BD-rate saving compared to PHEC, respectively. In addition, the subjective quality of the tests are noticeably enhanced by alleviating the seam artifacts compared to HEC.

# Introduction

In the last meeting, Core Experiment 13 (CE13) [1] was established to study removing subjective artifacts at face boundaries for the 360° video category. This contribution reports the results for the in-loop filters, including the de-blocking filter, sample adaptive offset (SAO) filter and the adaptive loop filter (ALF), using spherical neighbors proposed in JEVT-J0019 [2] and JVET-K0142 [3]. In CE13.4.2, there are four tests as described in Table I.

The in-loop filters in a conventional video coding system are used to remove undesired artifacts and enhance encoding quality. For a 360° video, the layout for a specific projection format usually has one or multiple discontinuous edges. Applying the in-loop filters on these discontinuous edges directly may result in poor visual quality and coding efficiency because the accessed pixels (to be referenced and/or filtered) cross discontinuous edges are jointly processed (filtered). To address this problem, a 360°-based in-loop filtering method is proposed such that the discontinuous edge can be properly processed.

Table I. The description of the tests in Test 4.2

| Test # | Description |
| --- | --- |
| 4.2 | In-loop filters using spherical neighbors - De-blocking filter - De-blocking filter + SAO - De-blocking filter + ALF - All filters |

# Proposed Method

# In this contribution, the in-loop filters, including the de-blocking filter, sample adaptive offset (SAO) filter and adaptive loop filter (ALF), are performed necessarily changes to avoid inappropriately filtering at the layout boundary and/or the discontinuous edge. As proposed in JEVT-J0019 [2] and JVET-K0142 [3], based on the spherical continuity in 360° video, the in-loop filtering process makes use of the pixel data located at the spherical positions for filtering so that the discontinuous edge, as well as the frame boundary (layout boundary), can be appropriately processed.

## De-blocking filter

Based on the concept of spherical continuity, the de-blocking filter fetches the spherical pixel positions located at another face for filtering when the filtered pixels cross the frame boundary (layout boundary) or the discontinuous edge as illustrated in Fig. 1. The pixel data in the spherical positions are derived directly from the corresponding face and the boundary strength of the corresponding block edge is set to 2 for de-blocking filtering process.

Fig. 1. Spherical pixel positions from another face for 360°-based de-blocking filter based on spherical continuity

## Sample Adaptive Offset filter

When the referenced neighboring pixels of the current pixel in SAO filtering process cross the discontinuous edge, the proposed implementation uses the spherical pixel positions located at another face for SAO filtering as illustrated in Fig. 2. Moreover, the pixel data in the spherical positions are copied directly from the corresponding face without any interpolation.

Fig. 2. Spherical pixel positions from another face for 360°-based SAO filter based on spherical continuity

## Adaptive Loop Filter

In ALF, pixels are multiplied with the filter coefficients which are determined by the pixel classification methods and the Wiener filter approach. In the conventional ALF, the replication padding is used when the accessed pixels for filtering are located outside the frame boundary (layout boundary). However, in 360°-based ALF, the accessed pixels outside the frame boundary (layout boundary) can be obtained from the pixels data located at the spherical positions. During the determination of the filter coefficients or the filtering process, the accessed pixels crossing the frame boundary (layout boundary) or the discontinuous edge are changed to use the pixels data located at the spherical positions as illustrated in Fig. 3.

Fig. 3. Spherical pixel positions from another face for 360°-based ALF filter based on spherical continuity

# Experimental Results

The tests of the proposed in-loop filters using spherical neighbors are conducted on the hybrid equi-angular cubemap (HEC) [4] layout. The software is implemented on the 360Lib-7.0 [5] and the BMS-2.0.1 (with fix BMS ticket #78 and #79) [6] followed by the JVET common test conditions and evaluation procedures for 360° video [7] under the VTM-2.0 [8] encoder configurations. The resolution of each face is set to 1280x1280, which yields the frame resolution is 3840x2560. As compared to HEC and PHEC, the BD-rate performance, in terms of end-to-end WS-PSNR provided by Test 4.2.a to Test 4.2.d are listed in Table II to Table V, respectively.

Table II. BD-rate performance for Test 4.2.a compared to HEC and PHEC under VTM-2.0

|  | Anchor : HEC |  |  | Anchor : PHEC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -0.29% | -0.84% | -0.98% | -0.25% | 0.16% | 0.16% |
| ChairLift | -0.41% | -0.45% | -0.45% | -0.52% | -0.21% | -0.17% |
| KiteFlite | -0.19% | -0.25% | -0.39% | -0.64% | -0.13% | -0.02% |
| Harbor | -0.65% | -1.22% | -1.60% | -0.62% | 0.28% | 0.24% |
| Trolley | -0.17% | -0.68% | -0.43% | -0.83% | -0.20% | -0.34% |
| GasLamp | -0.21% | -0.85% | -0.62% | -0.69% | -0.48% | -0.15% |
| Balboa | -0.38% | -0.41% | -0.67% | -0.50% | -0.37% | -0.21% |
| Broadway | -0.40% | -0.30% | -0.61% | -0.47% | -0.20% | -0.05% |
| Landing2 | -0.29% | -1.99% | -1.48% | -0.79% | -0.98% | -0.65% |
| BranCastle2 | -0.13% | -0.58% | -0.74% | -0.93% | -0.94% | -0.77% |
| Class S1 | -0.32% | -0.72% | -0.74% | -0.59% | -0.10% | -0.05% |
| Class S2 | -0.30% | -0.82% | -0.88% | -0.67% | -0.62% | -0.42% |
| Overall | -0.31% | -0.76% | -0.80% | -0.62% | -0.31% | -0.20% |

Table III. BD-rate performance for Test 4.2.b compared to HEC and PHEC under VTM-2.0

|  | Anchor : HEC |  |  | Anchor : PHEC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -0.33% | -0.98% | -1.18% | -0.29% | 0.02% | -0.04% |
| ChairLift | -0.43% | -0.37% | -0.58% | -0.54% | -0.14% | -0.30% |
| KiteFlite | -0.18% | -0.32% | -0.42% | -0.63% | -0.20% | -0.05% |
| Harbor | -0.65% | -1.19% | -1.53% | -0.62% | 0.31% | 0.31% |
| Trolley | -0.16% | -0.70% | -0.40% | -0.82% | -0.22% | -0.32% |
| GasLamp | -0.21% | -0.84% | -0.61% | -0.68% | -0.47% | -0.14% |
| Balboa | -0.37% | -0.40% | -0.41% | -0.50% | -0.37% | 0.06% |
| Broadway | -0.37% | -0.45% | -0.49% | -0.44% | -0.35% | 0.07% |
| Landing2 | -0.32% | -2.12% | -1.40% | -0.82% | -1.11% | -0.56% |
| BranCastle2 | -0.12% | -0.43% | -0.61% | -0.93% | -0.79% | -0.65% |
| Class S1 | -0.33% | -0.73% | -0.79% | -0.60% | -0.12% | -0.09% |
| Class S2 | -0.30% | -0.85% | -0.73% | -0.67% | -0.65% | -0.27% |
| Overall | -0.31% | -0.78% | -0.76% | -0.63% | -0.33% | -0.16% |

Table IV. BD-rate performance for Test 4.2.c compared to HEC and PHEC under VTM-2.0

|  | Anchor : HEC |  |  | Anchor : PHEC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -0.25% | -1.66% | -1.00% | -0.22% | -0.67% | 0.14% |
| ChairLift | -0.46% | -0.96% | -0.71% | -0.58% | -0.72% | -0.43% |
| KiteFlite | -0.21% | -0.70% | -0.66% | -0.66% | -0.58% | -0.29% |
| Harbor | -0.66% | -1.77% | -1.94% | -0.64% | -0.29% | -0.10% |
| Trolley | -0.18% | -1.13% | -0.73% | -0.83% | -0.65% | -0.65% |
| GasLamp | -0.26% | -1.14% | -0.85% | -0.73% | -0.77% | -0.39% |
| Balboa | -0.40% | -0.99% | -0.70% | -0.53% | -0.96% | -0.23% |
| Broadway | -0.39% | -0.92% | -0.69% | -0.46% | -0.82% | -0.13% |
| Landing2 | -0.33% | -3.57% | -2.62% | -0.83% | -2.57% | -1.80% |
| BranCastle2 | -0.13% | -0.60% | -0.79% | -0.94% | -0.96% | -0.82% |
| Class S1 | -0.34% | -1.23% | -0.98% | -0.61% | -0.61% | -0.29% |
| Class S2 | -0.31% | -1.52% | -1.20% | -0.69% | -1.33% | -0.75% |
| Overall | -0.33% | -1.34% | -1.07% | -0.64% | -0.90% | -0.47% |

Table V. BD-rate performance for Test 4.2.d compared to HEC and PHEC under VTM-2.0

|  | Anchor : HEC |  |  | Anchor : PHEC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | -0.27% | -1.51% | -0.85% | -0.24% | -0.52% | 0.29% |
| ChairLift | -0.47% | -0.93% | -0.79% | -0.58% | -0.69% | -0.51% |
| KiteFlite | -0.21% | -0.71% | -0.64% | -0.66% | -0.58% | -0.27% |
| Harbor | -0.66% | -1.83% | -2.15% | -0.64% | -0.34% | -0.32% |
| Trolley | -0.18% | -1.16% | -0.80% | -0.84% | -0.68% | -0.72% |
| GasLamp | -0.25% | -1.16% | -0.86% | -0.73% | -0.80% | -0.40% |
| Balboa | -0.36% | -0.92% | -0.73% | -0.49% | -0.89% | -0.26% |
| Broadway | -0.37% | -1.09% | -0.62% | -0.44% | -0.99% | -0.07% |
| Landing2 | -0.31% | -3.51% | -2.62% | -0.81% | -2.52% | -1.80% |
| BranCastle2 | -0.14% | -0.64% | -0.85% | -0.95% | -1.00% | -0.88% |
| Class S1 | -0.34% | -1.22% | -1.02% | -0.61% | -0.60% | -0.32% |
| Class S2 | -0.29% | -1.54% | -1.21% | -0.67% | -1.35% | -0.75% |
| Overall | -0.32% | -1.35% | -1.09% | -0.64% | -0.90% | -0.49% |

# Subjective Quality Testing

In Fig. 4 to Fig. 13, the cropped viewport images of HEC and the four tests in Test 4.2 are demonstrated (under VTM-2.0 with QP = 37) in order to evaluate the seam artifacts appeared at the discontinuous edge.

Fig. 4. The subjective quality of BranCastle2 frame 200 (HEC)

Fig. 5. The subjective quality of BranCastle2 frame 200 (Test 4.2.a)

Fig. 6. The subjective quality of BranCastle2 frame 200 (Test 4.2.b)

Fig. 7. The subjective quality of BranCastle2 frame 200 (Test 4.2.c)

Fig. 8. The subjective quality of BranCastle2 frame 200 (Test 4.2.d)

Fig. 9. The subjective quality of SkateboardInLot frame 76 (HEC)

Fig. 10. The subjective quality of SkateboardInLot frame 76 (Test 4.2.a)

Fig. 11. The subjective quality of SkateboardInLot frame 76 (Test 4.2.b)

Fig. 12. The subjective quality of SkateboardInLot frame 76 (Test 4.2.c)

Fig. 13. The subjective quality of SkateboardInLot frame 76 (Test 4.2.d)

# Conclusion

In this contribution, the performance of Test 4.2.a, 4.2.b, 4.2.c and 4.2.d are investigated. The conventional in-loop filters are modified to avoid improper filtering at the discontinuous edge. As compared to HEC, the proposed method can remarkably reduce the visible seam artifacts with slightly improved compression efficiency.

# References

- P. Hanhart, J.-L. Lin, and C. Pujara, “Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1033, July. 2018, Ljubljana, SI.

- J.-L. Lin, Y.-H. Lee, C.-H. Shih, S.-Y. Lin, H.-C. Lin, S.-K. Chang, P. Wang, L. Liu, and C.-C. Ju, “Description of 360° video coding technology proposal by MediaTek,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J0019, Apr. 2018, San Diego, USA.

- S.-Y. Lin, L. Liu, C.-H. Shih, J.-L. Lin, H.-C. Lin, S.-K. Chang and C.-C. Ju, “AHG8: 360°-based in-loop filters for cubemap projection,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0142, July 2018, Ljubljana, SI.

- Y.-H. Lee, J.-L. Lin, S.-K. Chang and C.-C. Ju,”CE13: Modified Cubemap Projection in JVET-J0019 (Test 5),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0131, July 2018, Ljubljana, SI.

- 360Lib, https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

- VTM Benchmark Set, https://vcgit.hhi.fraunhofer.de/JVET-K-CE13/VVCSoftware_BMS

- P. Hanhart, J. Boyce, and K. Choi, “JVET common test conditions and evaluation procedures for 360° video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1012, July. 2018, Ljubljana, SI.

- Versatile Video Coding Test Model, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).