| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0358-v2 |
| --- | --- |

| Title: | CE6: Sub-block transform for inter blocks (CE6.1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang, Jianle Chen | Email: | yin.zhao@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the coding performance of a sub-block transform for inter blocks, known as Spatially Varying Transform (SVT). Simulation results reportedly show that by allowing all inter blocks using the sub-block transform -0.61% and -1.02% PSNR-Y BD-rates are achieved for RA and LB configurations, with EncT 114% and 122%, respectively.

(In the second version, the draft text is add in Section 6.)

Proposed Method

For an inter-predicted CU with root_cbf equal to 1, a sub-block transform, known as spatially varying transform (SVT) in JVET-K0139, may be used (signaled by a flag) to transform and code only a sub-part of the residual block. A CU is tiled into two TUs, one TU has residual (named residual TU) and the other is inferred to be of zero residual.

When the sub-block transform is used for a CU, the TU tiling method and the residual TU position are further decoded from the bitstream. There are two TU tiling types (denoted as SVT-V and SVT-H), each with two residual TU positions, as indicated in Figure 1. For SVT-V (or SVT-H), the residual TU width (or height) equals to half of the CU width (or height) or 1/4 of the CU width (or height), signaled by another flag. If one side of CU is 8, TU tiling with 1/4 of the side is not allowed. Hence, there are at most 8 sub-block transfom modes for a CU. Both TU and CU boundaries are processed in de-blocking filtering.

The sub-block transform is allowed when CU width and height are not larger than maxSize. The maxSize is signaled in slice header. For HD and 4K sequences, maxSize is set as 64 for all slices; for other smaller resolution sequences, maxSize is set as 32 for all slices. When one side of the TU is greater than 32, the corresponding transform is set as DCT-2, and thus the maximum non-DCT-2 transform size is kept as 32.

Position-dependent core transform is applied to the residual TU. More specifically, the horizontal and vertical transforms for each residual TU position is specified in Figure 1. For example, the horizontal and vertical transforms for SVT-V position 0 is DCT-8 and DST-7, respectively. When a CU uses the sub-block transform, the inter MTS is not allowed for the CU, and thus cu_mts_flag is inferred to be 0.

A fast algorithm is designed for the sub-block transform. For each mode, a RD cost is estimated based on the SSD of the residual-skipped part. A sub-block transform mode is skipped in RDO if the estimated RD cost of the mode is larger than the actual RD cost of the best mode. In addition, only the best 4 modes in terms of the estimated RD cost are tried in RDO. Some sub-block transform modes may be skipped based on historical best sub-block transform mode of the residual.

Figure 1 Illustration of sub-block transform modes

Therefore, the sub-block transform jointly specifies the TU tiling, cbf and core transforms of a residual block, and is optimized to the case that the major residual of a block concentrate at one side of the block. In this case, the sub-block transform is more efficient compared with splitting the CU into two or three sub-CUs.

It is found that enabling the sub-block transform for some inter-prediction modes (i.e., merge idx > 1 and imv > 0) brings small gain. Therefore, to achieve a better trade-off between coding efficiency as well as increasing diversity between different prediction modes, the sub-block transform can be disabled for those modes (e.g., by a sps flag), which is CE6.1.2-a. In another test CE6.1.2-b, the sub-block transform are allowed to all inter-prediction blocks.

The implementation of the sub-block transform simultaneously fixes a non-normative bug in BMS 2.0.1 (i.e., the cu_cbf is always decided by the first TU of a CU in encoder RDO process), which was reported by one proponent in BMS ticket #78. With the bugfix, the anchor BD-rate Y performance is improved by -0.03% and -0.03% in VTM RA and LB configurations, respectively. More detailed results can be found in the JVET-L0358_anchor_bugfix.xlsm.

Simulation results

The proposed methods were integrated into the BMS 2.0.1 reference software, and are tested in three configurations: VTM, VTM plus inter MTS, and BMS configurations. Table 1 summarizes the simulation results of the tests. Detailed results can be found in the attached excel sheets.

Table 1 Simulation results of the two tests

|  |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| Over VTM 2.0.1 |  |  |  |  |  |  |
| CE6.1.2-a | RA | -0.48% | 0.21% | 0.03% | 109% | 102% |
|  | LB | -0.94% | -0.06% | -0.10% | 114% | 104% |
| CE6.1.2-b | RA | -0.61% | 0.20% | 0.00% | 114% | 102% |
|  | LB | -1.02% | -0.04% | 0.03% | 122% | 104% |
| Over VTM 2.0.1 + Inter MTS |  |  |  |  |  |  |
| CE6.1.2-a | RA | -0.37% | 0.03% | -0.04% | 107% | 101% |
|  | LB | -0.60% | -0.30% | -0.12% | 110% | 100% |
| CE6.1.2-b | RA | -0.45% | 0.07% | -0.03% | 110% | 100% |
|  | LB | -0.65% | -0.32% | -0.03% | 114% | 100% |
| Over BMS 2.0.1 |  |  |  |  |  |  |
| CE6.1.2-a | RA | -0.35% | 0.08% | -0.02% | 104% | 100% |
|  | LB | -0.60% | -0.35% | -0.09% | 108% | 102% |
| CE6.1.2-b | RA | -0.44% | 0.07% | -0.01% | 106% | 99% |
|  | LB | -0.69% | -0.29% | -0.08% | 112% | 101% |

It is noted that proposed sub-block transform achieves higher coding gain with low encoding complexity than Inter MTS (-0.36% and -0.59% in RA and LB, with EncT 126% and 131%). The lower complexity version (i.e., CE6.1.2-a) drops around 0.1% BD-rate while reduces the additional EncT by 1/3.

On top of Inter MTS, the gain from the proposed method reduces by 0.16% and 0.37% in RA and LB, respectively. In another point of view, on top of the proposed method, inter MTS achieves around 0.2% coding gain with over 20% EncT increase (c.f. JVET-L0358_CE6.1.2-b_vtmCfg_emt3_svt_vs_svt.xlsm).

The decoder complexity is not increased, since the sub-block transform just couples existing TU tiling and core transforms in VTM. The DecT increase in VTM configuration is due to the using of non-DCT-2 transform for inter blocks (VTM anchor does not use non-DCT-2 transform for inter blocks since inter MTS is off).

Conclusions

This contribution reports the performance of a sub-block transform method. -0.61% and -1.02% BD-rates are achieved from the proposed method in RA and LB configuration.

It is recommended to adopt the sub-block transform method into VTM.

Acknowledgement

We would like to thank Qualcomm and Sony for cross-checking this proposal.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Draft text

[Note: the text is not in trace mode for clarity]

- 7.3.2.1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| … |  |
| sps_mts_intra_enabled_flag | u(1) |
| sps_mts_inter_enabled_flag | u(1) |
| sps_sbt_enable_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

- 7.3.3.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if ( slice_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_sbtmvp_enabled_flag ) { |  |
| sbtmvp_size_override_flag | u(1) |
| if( sbtmvp_size_override_flag ) |  |
| log2_sbtmvp_active_size_minus2 | u(3) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_sbt_enable_flag ) |  |
| slice_max_sbt_size_64_flag | u(1) |
| } |  |
| if ( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) |  |
| alf_data( ) |  |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| byte_alignment( ) |  |
| } |  |

- 7.3.4.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && sps_sbt_enable_flag ) { |  |
| if( cbWidth <= maxSbtSize && cbHeight <= maxSbtSize ) { |  |
| allowSbtVerHalf = cbWidth >= 8 |  |
| allowSbtVerQuad = cbWidth >= 16 |  |
| allowSbtHorHalf = cbHeight >= 8 |  |
| allowSbtHorQuad = cbHeight >= 16 |  |
| if( allowSbtVerHalf ¦ ¦ allowSbtHorHalf ¦ ¦ allowSbtVerQuad ¦ ¦ allowSbtHorQuad ) |  |
| cu_sbt_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( cu_sbt_flag[ x0 ][ y0 ] ) { |  |
| if( ( allowSbtVerHalf ¦ ¦ allowSbtHorHalf ) && ( allowSbtVerQuad ¦ ¦ allowSbtHorQuad ) ) |  |
| cu_sbt_quad_flag[ x0 ][ y0 ] | ae(v) |
| if( ( cu_sbt_quad_flag[ x0 ][ y0 ] && allowSbtVerQuad && allowSbtHorQuad ) ¦ ¦ ( !cu_sbt_quad_flag[ x0 ][ y0 ] && allowSbtVerHalf && allowSbtHorHalf ) ) |  |
| cu_sbt_horizontal_flag[ x0 ][ y0 ] | ae(v) |
| cu_sbt_pos_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |
| } |  |

- 7.3.4.7 Transform tree syntax

| transform_tree( x0, y0, tbWidth, tbHeight , treeType) { | Descriptor |
| --- | --- |
| if( tbWidth > MaxTbSizeY ¦ ¦ tbHeight > MaxTbSizeY ) { |  |
| trafoWidth = ( tbWidth > MaxTbSizeY ) ? (tbWidth / 2) : tbWidth |  |
| trafoHeight = ( tbHeight > MaxTbSizeY ) ? (tbHeight / 2) : tbHeight |  |
| transform_tree( x0, y0, trafoWidth, trafoHeight ) |  |
| if( tbWidth > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0, trafoWidth, trafoHeight, treeType ) |  |
| if( tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0, y0 + trafoHeight, trafoWidth, trafoHeight, treeType ) |  |
| if( tbWidth > MaxTbSizeY && tbHeight > MaxTbSizeY ) |  |
| transform_tree( x0 + trafoWidth, y0 + trafoHeight, trafoWidth, trafoHeight, treeType ) |  |
| } else if( cu_sbt_flag[ x0 ][ y0 ] ) |  |
| factorTb0 = cu_sbt_quad_flag[ x0 ][ y0 ] ? 1 : 2 |  |
| factorTb0 = cu_sbt_pos_flag[ x0 ][ y0 ] ? ( 4 - factorTb0 ) : factorTb0 |  |
| noResiTb0 = cu_sbt_pos_flag[ x0 ][ y0 ] ? 1 : 0 |  |
| if( !cu_sbt_horizontal_flag[ x0 ][ y0 ] ) { |  |
| trafoWidth = tbWidth * factorTb0 / 4 |  |
| transform_tree( x0, y0, trafoWidth, tbHeight, treeType , noResiTb0 ) |  |
| transform_tree( x0 + trafoWidth, y0, tbWidth − trafoWidth, tbHeight, treeType , !noResiTb0 ) |  |
| } |  |
| else { |  |
| trafoHeight = tbHeight * factorTb0 / 4 |  |
| transform_tree( x0, y0, tbWidth, trafoHeight, treeType , noResiTb0 ) |  |
| transform_tree( x0, y0 + trafoHeight, tbWidth, tbHeight − trafoHeight, treeType , !noResiTb0 ) |  |
| } |  |
| } else { |  |
| transform_unit( x0, y0, tbWidth, tbHeight, treeType , 0 ) |  |
| } |  |
| } |  |

- 7.3.4.8 Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType , noResi ) { | Descriptor |
| --- | --- |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) && !noResi ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && !noResi ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( ( ( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) && sps_mts_intra_enabled_flag ) ¦ ¦ ( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ) && sps_mts_inter_enabled_flag ) ) && tu_cbf_luma[ x0 ][ y0 ] && treeType ! = DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && !cu_sbt_flag[ x0 ][ y0 ] ) |  |
| cu_mts_flag[ x0 ][ y0 ] | ae(v) |
| if( tu_cbf_luma[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, log2( tbWidth ), log2( tbHeight ), 0 ) |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, log2( tbWidth / 2 ), log2( tbHeight / 2 ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, log2( tbWidth / 2 ), log2( tbHeight / 2 ), 2 ) |  |
| } |  |

- 7.4.3.1 Sequence parameter set RBSP semantics

sps_sbt_enabled_flag equal to 0 specifies that sub-block transform for inter-predicted CU is disabled. sps_sbt_enabled_flag equal to 1 specifies that sub-block transform for inter-predicted CU is enabled.

- 7.4.4.1 General slice header semantics

slice_sbt_max_size_64_flag equal to 0 specifies that the maximum CU width and height for allowing sub-block transform is 32. slice_sbt_max_size_64_flag equal to 1 specifies that the maximum CU width and height for allowing sub-block transform is 64.

maxSbtSize = slice_sbt_max_size_64_flag? 64 : 32

- 7.4.5.5 Coding unit semantics

cu_sbt_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, sub-block transform is used. cu_sbt_flag[ x0 ][ y0 ] equal to 0 specifies that for the current coding unit, the sub-block transform is not used.

When cu_sbt_flag[ x0 ][ y0 ] is not present, its value is inferred to be equal to 0.

NOTE – : When sub-block transform is used, a coding unit is tiled into two transform units, one transform unit has residual, the other does not have residual.

cu_sbt_quad_flag[ x0 ][ y0 ] equal to 1 specifies that for the current coding unit, the sub-block transform include a transform unit of 1/4 size of the current coding unit. cu_sbt_quad_flag[ x0 ][ y0 ] equal to 0 specifies that for the current coding unit the sub-block transform include a transform unit of 1/2 size of the current coding unit.

When cu_sbt_quad_flag[ x0 ][ y0 ] is not present, its value is inferred to be equal to 0.

cu_sbt_horizontal_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is tiled into 2 transform units by a horizontal split. cu_sbt_horizontal_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is tiled into 2 transform units by a vertical split.

When cu_sbt_horizontal_flag[ x0 ][ y0 ] is not present, its value is derived as follows:

- If cu_sbt_quad_flag[ x0 ][ y0 ] is equal to 1, cu_sbt_horizontal_flag[ x0 ][ y0 ] is set to be equal to allowSbtHoriQuad.

- Otherwise (cu_sbt_quad_flag[ x0 ][ y0 ] is equal to 0), cu_sbt_horizontal_flag[ x0 ][ y0 ] is set to be equal to allowSbtHoriHalf.

cu_sbt_pos_flag[ x0 ][ y0 ] equal to 1 specifies that the tu_cbf_luma, tu_cbf_cb and tu_cbf_cr of the first transform unit in the current coding unit are not present in the bitstream. cu_sbt_pos_flag[ x0 ][ y0 ] equal to 0 specifies that the tu_cbf_luma, tu_cbf_cb and tu_cbf_cr of the second transform unit in the current coding unit are not present in the bitstream.

    - Transformation process for scaled transform coefficients

- 8.4.4.1 General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

If cu_sbt_flag[ xTbY ][ yTbY ] is equal to 1, the variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived in Table 8-X depending on cu_sbt_horizontal_flag[ xTbY ][ yTbY ] and cu_sbt_pos_flag[ xTbY ][ yTbY ].

Otherwise (cu_sbt_flag[ xTbY ][ yTbY ] is equal to 0), the variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived in Table 8-9 depending on mts_idx[ xTbY ][ yTbY ] and CuPredMode[ xTbY ][ yTbY ].

The (nTbW)x(nTbH) array r of residual samples is derived as follows:

- Each (vertical) column of scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 is transformed to e[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.4.4.2 for each column x = 0..nTbW − 1 with the height of the transform block nTbH, the list d[ x ][ y ] with y = 0..nTbH − 1 and the transform type variable trType set equal to trTypeVer as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbH − 1.

- The intermediate sample values g[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

g[ x ][ y ] = Clip3( CoeffMin, CoeffMax, ( e[ x ][ y ] + 256 )  >>  9 )	(8-393)

- Each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 is transformed to r[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1 by invoking the one-dimensional transformation process as specified in clause 8.4.4.2 for each row y = 0..nTbH − 1 with the width of the transform block nTbW, the list g[ x ][ y ] with x = 0..nTbW − 1 and the transform type variable trType set equal to trTypeHor as inputs, and the output is the list r[ x ][ y ] with x = 0..nTbW − 1.

Table 8-X – Specification of trTypeHor and trTypeVer depending on mts_idx[ x ][ y ] and CuPredMode[ x ][ y ]

|  | cu_sbt_horizontal_flag [ xTbY ][ yTbY ] = = 0 |  | cu_sbt_horizontal_flag [ xTbY ][ yTbY ] = = 1 |  |
| --- | --- | --- | --- | --- |
|  | cu_sbt_pos_flag [ xTbY ][ yTbY ] = = 0 | cu_sbt_pos_flag [ xTbY ][ yTbY ] = = 1 | cu_sbt_pos_flag [ xTbY ][ yTbY ] = = 0 | cu_sbt_pos_flag [ xTbY ][ yTbY ] = = 1 |
| trTypeHor | 2 | 1 | nTbW > 32 ? 0 : 1 | nTbW > 32 ? 0 : 1 |
| trTypeVer | nTbH > 32 ? 0 : 1 | nTbH > 32 ? 0 : 1 | 2 | 1 |

Table 8-9 – Specification of trTypeHor and trTypeVer depending on mts_idx[ x ][ y ] and CuPredMode[ x ][ y ]

| mts_idx[ xTbY ][ yTbY ] | CuPredMode[ xTbY ][ yTbY ] = = MODE_INTRA |  | CuPredMode[ xTbY ][ xTbY ] = = MODE_INTER |  |
| --- | --- | --- | --- | --- |
|  | trTypeHor | trTypeVer | trTypeHor | trTypeVer |
| −1 (inferred) | 0 | 0 | 0 | 0 |
| 0 (00) | 1 | 1 | 2 | 2 |
| 1 (01) | 2 | 1 | 1 | 2 |
| 2 (10) | 1 | 2 | 2 | 1 |
| 3 (11) | 2 | 2 | 1 | 1 |