| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0359 |
| --- | --- |

| Title: | AHG12: Flexible Tile splitting |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| jAuthor(s) or Contact(s): | Yukinobu Yasugi Tomohiro Ikai | Tel: Email: | +81-43-299-8530 yasugi.yukinobu@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution is relevant to the tile functionality which proposed in JVET-K0155. The proposed tile allows to split pictures into flexible partitioning tile, where the width or height of the unit of tile can be multiplies of 4 (the minimum CU size), 8, 16, 32 and 64, i.e. smaller than CTU. By using partial CTUs on the right and bottom edges of each tile, a picture can be divided into tiles of more uniform size and number of CTUs in uniform spacing mode than HEVC tile, and this splitting helps better load balancing. Furthermore, it is also asserted that this feature is useful for 360 video sequences and frame packing sequences since the corresponding tile can fit the arbitrary rectangular face size. The authors implemented the proposed tile on VTM-2.0.1 and evaluated the performance.

The experimental results with this feature (unit=32) reportedly show that luma BD-rate coding losses on average are x.xx % for All Intra and x.xx % for Random Access under the common test condition (CTC) for SDR sequences.

The experimental results without this feature (i.e. HEVC like tile) reportedly show that luma BD-rate coding losses on average are x.xx % for All Intra and x.xx % for Random Access under the common test condition (CTC) for SDR sequences.

Introduction

In the previous document (JVET-K0155[1]), we introduced a tile splitting method with fine granularity unit, i.e. the smaller unit than CTU. The conventional tile segmentation by CTU sometimes has difficulty to fit frame packing boundaries or to do better load balancing because the unit of tile splitting is coarse. In this contribution, we ported and evaluated the performance of the proposed method on VTM-2.0.1.

Technical description

Figure 1 shows an example of conventional 2x2 tile provided that the syntax element uniform_spacing_flag is equal to 1. In this case, the picture is split into four tiles along the boundaries of the maximum CTB size, and the shape and number of CTUs for the resulting tiles are different each other. The odd CTUs appear only in the right and bottom edge of the picture. Figure 2 shows an example of the proposed tile segmentation. The picture is split into the same size tiles using boundaries of smaller blocks than CTU. There are CTUs of odd size in the right and bottom edge of each tile. The odd CTUs are encoded and decoded as same as in picture boundary. The conversion between the location of the top-left luma sample of each CTU and the raster scan address of CTU CtuRsAddr is done by using pre-calculated look-up tables.

Figure 2: Example of 2x2 segmentation by using boundaries of smaller units
 (uniform_spacing_flag = 1)

Figure 1: Example of segmentation by conventional 2x2 tiles
 (uniform_spacing_flag=1)

Experimental results

The authors implemented the proposed method on VTM-2.0.1 with the following macro definitions.

HEVC_TOOLS = 1, HEVC_TILES_WPP = 1

Other definitions for HEVC tools were set to zero.

The performance of the tile system for five cases with 2x2 splitting and the uniform spacng mode enabled  (--NumTileColumnsMinus1=1 --NumTileRowsMinus1=1 --UniformSpacingFlag=1) are tested. The other conditions are in accordance with the JVET CTC for SDR.

The tested five settings are as follows:

Test0: 128x128 block based tile (conventional)

Test1: 8x8 block based tile

Test2: 16x16 block based tile

Test3: 32x32 block based tile

The resulting size of 2x2 tiles for classA-C with each setting are described in Table 1.

Table 1: Resulting tile sizes in each test

|  | classA (3840x2160) | classB (1920x1080) | classC (832x480) |
| --- | --- | --- | --- |
| Test0 | (1920+1920) x (1024+1136) | (896+1024) x (512+568) | (384+448) x (256+224) |
| Test1 | (1920+1920) x (1080+1080) | (960+960) x (536+544) | (416+416) x (240+240) |
| Test2 | (1920+1920) x (1072+1088) | (960+960) x (544+536) | (416+416) x (240+240) |
| Test3 | (1920+1920) x (1088+1072) | (960+960) x (544+536) | (416+416) x (224+256) |

TBD.

References

[1] Y. Yasugi, T. Ikai, “AHG12: Flexible Tile Partitioning” JVET-K0155, Jul. 2018, Ljubljana, SI.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).