| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0364-v2 |
| --- | --- |

| Title: | CE4: Simplification of affine AMVP list construction (Test 4.1.3 and 4.1.12) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of simplification of affine AMVP list construction. In this contribution, affine AMVP list construction is simplified as follow:

- The pruning process for inherited affine candidates is simplified by comparing the coding units covering the neighboring positions, instead of comparing the derived affine candidates in VTM-2.0.1. The pruning process for constructed affine candidates is totally removed.

- For constructed candidates, the conversion from motion vectors of top-left and bottom-left to top-right MV is removed; the conversion from motion vectors of top-left and top-right to bottom-right motion vector for 6-parameter is removed.

The proposed changes reduce the complexity of affine AMVP list construction without comprising coding efficiency. Compared to VTM-2.0.1 anchor, the average BD-rate differences are 0.01% in RA and 0.01% in LDB, respectively. When combined with line buffer reduction method in JVET-L0045, the average BD-rate differences are 0.11% in RA and 0.04% in LB.

Introduction

In VTM-2.0.1, affine AMVP list is constructed [1] as follows:

- Up to two different affine candidates are derived from affine motion of the neighboring coding units. Neighboring positions A0, A1, B0, B1, and B2 as shown in Figure 1 are checked. If the coding unit covering the neighboring position is coded using affine motion model and its reference frame is same as the reference frame of the current coding unit, MVs at two (for 4-parameter affine model) or three (for 6-parameter affine model) control points of the current coding unit are derived from the affine model of this neighbor.

- Constructed affine candidates. Figure 1 shows the neighboring blocks used to generate the constructed affine candidate set. The neighboring MVs are divided into three groups: S0=mvB2,mvB3,mvA2 , S1=mvB1,mvB0 and S2=mvA1,mvA0. mv0  is the first MV in S0 that refers to the same reference picture as the current block; mv1 is the first MV in S1 that refers to the same reference picture of the current block; and mv2 is the first in S2 that refers to the same reference picture of the current block.

- If only mv0 and mv1 can be found and the affine motion model for current block is 6-parameter model, mv2 is derived as:

mv2x=mv0x-hmv1y-mv0yw, mv2y=mv0y+hmv1x-mv0xw

where the current block size is W×H.

- If only mv0 and mv2 can be found, mv1 is derived as:

mv1x=mv0x+hmv2y-mv0yw, mv1y=mv0y-hmv2x-mv0xw

Figure 1: Spatial candidates for affine

During the construction of affine AMVP list in VTM-2.0.1, the pruning operation is applied to every possible candidate.

In this contribution, a CU based pruning approach for inherited affine candidates is proposed. The comparison is conducted by comparing the coding units covering the neighboring positions, instead of comparing the derived affine candidates. Furthermore, the pruning process for constructed affine candidate is totally removed.

For constructed affine candidate, the conversion from mv0 and mv2 to mv1 is removed and the conversion from mv0 and mv1 to mv2 for 6-parameter is removed.

Test specification

The tests for this contribution are descripted in Table 1.

Table 1 test specification for this contribution

| Test # | Test description |
| --- | --- |
| 4.1.3.a | Simplify the pruning process of affine AMVP list construction |
| 4.1.3.b | Simplify the construction of constructed affine candidates |
| 4.1.3.c | a) + b) |
| 4.1.12 | c) + line buffer reduction in JVET-L0045 |

Simulation results

The contribution has been implemented on top of VTM-2.0.1. The test condition descripted in JVET-K1024 [1] is used for verifying the contribution. The results are summarized in Table 2. More detailed results are provided from Table 3 to Table 6.

The proposed changes reduce the complexity of affine AMVP list construction without comprising coding efficiency. Compared to VTM-2.0.1 anchor, the average BD-rate differences are 0.01% in RA and 0.01% in LDB, respectively. When combined with line buffer reduction method in JVET-L0045, the average BD-rate differences are 0.11% in RA and 0.04% in LB.

Table 2 Summary of overall performance in RA/LB configuration (over VTM-2.0.1)

|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.1.3.a | 0.02% | 0.06% | -0.03% | 101% | 100% | 0.02% | -0.06% | -0.16% | 101% | 100% |
| 4.1.3.b | 0.01% | 0.03% | -0.02% | 101% | 100% | 0.01% | -0.09% | -0.16% | 101% | 100% |
| 4.1.3.c | 0.01% | 0.05% | -0.03% | 101% | 100% | 0.01% | -0.14% | -0.29% | 101% | 100% |
| 4.1.12 | 0.11% | 0.14% | 0.07% | 101% | 101% | 0.04% | 0.03% | -0.02% | 101% | 100% |

Test 4.1.3.a

Table 3 Coding performance of test 4.1.3.a over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.17% | -0.04% | 101% | 100% |
| Class A2 | 0.04% | -0.03% | -0.05% | 101% | 100% |
| Class B | 0.03% | 0.01% | -0.04% | 101% | 100% |
| Class C | 0.02% | 0.12% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.06% | -0.03% | 101% | 100% |
| Class D | -0.01% | 0.03% | 0.09% | 100% | 100% |
| Class F (optional) | -0.02% | -0.06% | -0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.03% | -0.07% | 101% | 100% |
| Class C | 0.03% | -0.14% | -0.13% | 101% | 99% |
| Class E | -0.01% | -0.02% | -0.35% | 101% | 99% |
| Overall | 0.02% | -0.06% | -0.16% | 101% | 100% |
| Class D | 0.05% | 0.41% | -0.30% | 101% | 101% |
| Class F (optional) | -0.04% | 0.25% | 0.72% | 101% | 100% |

Test 4.1.3.b

Table 4 Coding performance of test 4.1.3.b over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.09% | 0.01% | 101% | 100% |
| Class A2 | 0.01% | -0.05% | -0.08% | 101% | 100% |
| Class B | 0.02% | -0.01% | -0.06% | 101% | 100% |
| Class C | 0.00% | 0.08% | 0.07% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | -0.02% | 101% | 100% |
| Class D | 0.03% | 0.02% | 0.00% | 101% | 101% |
| Class F (optional) | -0.02% | 0.01% | 0.03% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.08% | -0.12% | 102% | 100% |
| Class C | 0.05% | -0.07% | -0.06% | 101% | 101% |
| Class E | -0.09% | -0.11% | -0.37% | 102% | 100% |
| Overall | 0.01% | -0.09% | -0.16% | 101% | 100% |
| Class D | 0.06% | 0.33% | -0.24% | 101% | 102% |
| Class F (optional) | -0.10% | 0.56% | 0.13% | 101% | 100% |

Test 4.1.3.c

Table 5 Coding performance of test 4.1.3.c over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.03% | 101% | 101% |
| Class A2 | 0.02% | 0.01% | -0.02% | 101% | 100% |
| Class B | 0.03% | 0.10% | -0.08% | 101% | 100% |
| Class C | 0.00% | 0.06% | -0.01% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.05% | -0.03% | 101% | 100% |
| Class D | 0.04% | 0.13% | 0.02% | 100% | 101% |
| Class F (optional) | -0.01% | 0.00% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.21% | -0.15% | 101% | 100% |
| Class C | 0.05% | -0.12% | -0.21% | 101% | 100% |
| Class E | -0.09% | -0.06% | -0.62% | 101% | 99% |
| Overall | 0.01% | -0.14% | -0.29% | 101% | 100% |
| Class D | -0.02% | 0.64% | -0.36% | 100% | 101% |
| Class F (optional) | -0.10% | 0.26% | 0.77% | 101% | 100% |

Test 4.1.12

Table 6 Coding performance of test 4.1.12 over VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.16% | 0.11% | 101% | 101% |
| Class A2 | 0.26% | 0.16% | 0.20% | 102% | 100% |
| Class B | 0.11% | 0.08% | -0.01% | 101% | 101% |
| Class C | 0.04% | 0.16% | 0.04% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.11% | 0.14% | 0.07% | 101% | 101% |
| Class D | 0.04% | -0.03% | 0.02% | 101% | 100% |
| Class F (optional) | 0.03% | 0.04% | 0.04% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.04% | -0.06% | 101% | 100% |
| Class C | 0.06% | 0.01% | -0.04% | 101% | 101% |
| Class E | -0.06% | 0.17% | 0.07% | 101% | 99% |
| Overall | 0.04% | 0.03% | -0.02% | 101% | 100% |
| Class D | 0.07% | 0.54% | -0.25% | 101% | 101% |
| Class F (optional) | 0.01% | 0.54% | 0.97% | 101% | 99% |

Conclusions

This contribution reports the results of simplification of affine AMVP list construction by reducing the pruning process and conversion of constructed candidates.

The proposed changes reduce the complexity of affine AMVP list construction without comprising coding efficiency. Compared to VTM-2.0.1 anchor, the average BD-rate differences are 0.01% in RA and 0.01% in LDB, respectively. When combined with line buffer reduction method in JVET-L0045, the average BD-rate differences are 0.11% in RA and 0.04% in LB.

References

- Y. Han, H. Huang, Y. Zhang, C. Hung, C. Chen, W. Chien and M. Karczewicz, “CE4.1.3: Affine motion compensation prediction”, JVET-K0337,  the 11th JVET meeting, July 2018, Ljubljana, SI.

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-K1024, the 11th JVET meeting, July 2018, Ljubljana, SI.

- M. Zhou, “CE4: Test results of CE4.1.11 on line buffer reduction for affine mode”, JVET-L0045, the 12th JVET meeting, Oct. 2018, Macao, CN.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).