| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0372-v1 |
| --- | --- |

| Title: | CE1-related: Constrained chroma block partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang, Jianle Chen | Email: | yin.zhao@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution constrains the split of chroma blocks in I slice to avoid 2xN or Nx2 chroma blocks. For dual-tree setting, the minimum QT/BT/TT size of chroma tree is configured as 8. For single tree setting, when a split mode generates 2xN or Nx2 chroma blocks in child nodes, the luma block of the node splits using this split mode while the chroma blocks do not further split. The proposed method reportedly leads to 0.04%/1.51%/1.43% and 1.54%/-0.88%/-0.22% Y/U/V BD-rates loss for separate trees and single tree in AI settings (one RAS), respectively.

Introduction

In HEVC, the minimum chroma block size is 4x4. In VVC, chroma block size may be 2xN or Nx2. This may reduce the throughput of intra coded CUs. In this proposal, chroma block splitting restriction is used to avoid chroma blocks of size 2xN or Nx2, and thus aligning the worst case to HEVC. Two constraints are proposed for separate luma & chroma trees setting and single tree setting, respectively.

Proposal

For separate trees setting, the minimum QT/BT/TT sizes for chroma tree are set as 4 (measured in luma samples) in VTM. To avoid chroma block 2xN and Nx2, it is proposed to limit the minimum QT/BT/TT sizes for chroma tree as 8.

For single tree setting, when a split mode generates 2xN or Nx2 chroma blocks in child nodes, the luma block of the node splits using this split mode while the chroma blocks do not further split. In other words, the node turns from luma-chroma joint tree into separate trees. The luma tree is inferred to be split using the split mode (and the child luma tree nodes can further split), and the chroma tree is inferred to be non-split.

Accordingly, the chroma prediction mode and residual are signaled for the chroma CU, and the bitstream of chroma CU appears right after the bitstream of luma CUs.

Due to limited time, the encoder RDO process is not studied thoroughly. The encoder fast algorithm of cu coding data save and load is turned off when a CU is in separate tree status.

Simulation results

The proposed method was integrated into the VTM 2.0.1 reference software. In this version, all the coding experiments were conducted using AI configuration with the first RAS.

Test 1: limiting min QT/BT/TT size for chroma tree.

Anchor: VTM2.0.1 with dual tree off (--DualITree=0)

Test: anchor + min QT/BT/TT size modification

Test 2: limiting the split of chroma blocks into 2xN or Nx2 child nodes

Anchor: VTM2.0.1 with dual tree on (--DualITree=1, i.e., default configuration)

Test: anchor + luma block further splits and chorma blocks do not split

Table 1 summarizes the simulation results for the two tests under the VTM AI configurations with the first RAS of each sequence.

Table 1 Simulation results of Test 1 and Test 2 (first RAS, AI)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM2.0.1 DualTree (Test 1) |  |  |  |  | Over VTM2.0.1 SingleTree (Test 2) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.74% | 0.07% | 99% | 100% | 0.83% | 0.43% | 0.14% | 91% | 102% |
| Class A2 | 0.20% | 0.99% | 0.90% | 96% | 100% | 1.33% | -1.86% | -0.66% | 88% | 102% |
| Class B | 0.10% | 1.21% | 1.63% | 100% | 103% | 1.48% | -0.27% | -0.47% | 92% | 103% |
| Class C | 0.28% | 2.26% | 3.29% | 98% | 103% | 1.93% | 0.54% | 1.58% | 89% | 102% |
| Class E | -0.67% | 2.28% | 0.52% | 77% | 104% | 2.03% | -4.09% | -2.13% | 120% | 103% |
| Overall | 0.04% | 1.51% | 1.43% | 94% | 102% | 1.54% | -0.88% | -0.22% | 95% | 102% |
| Class D | 0.26% | 3.87% | 4.93% | 95% | 103% | 2.45% | 2.18% | 3.74% | 88% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

In general, smaller resolution sequences has larger BD-rate loss for limiting the split of chroma blocks not to reach 2xN and Nx2. The loss of Test 2 is higher compared with Test 1, and it need further investigation on the harmonization with encoder RDO process and CU splitting related fast algorithms.

Conclusions

This contribution introduces two schemes for removing 2xN and Nx2 chroma blocks, and provides preliminary coding performance results.

It is recommended to further study the proposed method in CE.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).