| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0372-v2 |
| --- | --- |

| Title: | CE1-related: Constrained chroma block partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang, Jianle Chen | Email: | yin.zhao@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution constrains the split of chroma blocks in I slice to avoid 2xN or Nx2 chroma blocks. For dual-tree setting, the minimum QT/BT/TT size of chroma tree is configured as 8 or limiting the chroma tree splits that result in 2xN or Nx2 chroma blocks. For single tree setting, when a split mode generates 2xN or Nx2 chroma blocks in child nodes, the luma block of the node splits using this split mode while the chroma blocks do not further split. In one test, N is from 2 to CTUSize/2, i.e., removing all chroma blocks with one side equal to 2, and the proposed method reportedly leads to 0.15%/1.21%/1.45% and 0.22%/-0.21%/-0.11% Y/U/V BD-rates loss for separate trees and single tree in AI settings, respectively. In another test, N is from 2 to 4, i.e., remove chroma blocks of 2x2, 2x4 and 4x2 which contain fewer than 16 chroma samples, the proposed method leads to 0.03%/0.30%/0.41% and 0.08%/-0.35%/-0.18% Y/U/V BD-rates loss for separate trees and single tree in AI settings, respectively.

(In the second version, an encoder bug is fixed which corrects the RD performance of the proposed method in single tree setting, and the VTM 2.0.1 CTC AI results are provided.)

Introduction

In HEVC, the minimum chroma block size is 4x4. In VVC, chroma block size may be 2xN or Nx2. This may reduce the throughput of intra coded CUs. In this proposal, chroma block splitting restriction is used to avoid chroma blocks of size 2xN or Nx2, and thus aligning the worst case to HEVC. Two constraints are proposed for separate luma & chroma trees setting and single tree setting, respectively.

In this contribution, two tests are conducted to investigate the RD performance of removing 2xN and Nx2 chroma blocks in I slice. The first test removes all 2xN or Nx2 chroma blocks. The second test only removes 2x2, 2x4 and 4x2 chroma blocks, i.e., N is limited to be no greater than 4, by which chroma blocks containing fewer than 16 pixels are disabled.

Proposal

2.1 Test 1: Remove all chroma blocks of size 2xN and Nx2 in I slice (no constraint on N)

For separate trees setting, the minimum QT/BT/TT sizes for chroma tree are set as 4 (measured in luma samples) in VTM. To avoid all chroma blocks of 2xN and Nx2, it is proposed to disable the chroma tree split modes that result in chroma blocks of 2xN and Nx2 by simply limiting the minimum QT/BT/TT sizes for chroma tree as 8. This is named Test 1-dualTree.

For single tree setting, when a split mode generates 2xN or Nx2 chroma blocks in child nodes, the luma block of the node splits using this split mode while the chroma blocks of the node do not further split. In other words, the node turns from luma-chroma joint tree into separate trees. The luma tree is inferred to be split using the split mode (and the child luma tree nodes can further split), and the chroma tree is inferred to be non-split, becoming a chroma CU. This is named Test 1-singleTree.

Accordingly, the chroma prediction mode and residual are signaled for the chroma CU, and the bitstream of the chroma CU appears right after the bitstream of the luma CUs.

Due to limited time, the encoder RDO process is not studied thoroughly. The encoder fast algorithm of cu coding data save and load is turned off when a CU is in separate tree status.

2.2 Test 2: Remove chroma blocks of size 2xN and Nx2 in I slice (N≤4)

For separate trees setting, it is proposed to disable the chroma tree split modes that result in chroma blocks of 2xN and Nx2 (N≤4), i.e., when the current chroma-tree node containing fewer than 64 chroma pixels, the TT and QT splits are disabled, and when the current chroma-tree node containing fewer than 32 chroma pixels, the BT splits are disabled. This is named Test 2-dualTree.

For single tree setting, when a split mode generates 2xN or Nx2 (N≤4) chroma blocks in child nodes, the luma block of the node splits using this split mode while the chroma blocks do not further split. This is identical to Test 1 except for limiting N≤4. This is named Test 2-singleTree.

Simulation results

The proposed method was integrated into the VTM 2.0.1 reference software. In this version, all the coding experiments were conducted using VTM2.0.1 CTC AI configuration. For the two single tree setting tests, the anchor is VTM2.0.1 with parameter –DualITree=0; for the two dual tree setting tests, the anchor is VTM2.0.1. Table 1 summarizes the simulation results for the two tests under the VTM AI configurations.

Table 1 Simulation results of Test 1 and Test 2

In general, smaller resolution sequences has larger BD-rate loss for limiting the split of chroma blocks not to reach 2xN and Nx2.

In single tree and separate trees settings, the average BD across three color components (= Y BD-rate + (U BD-rate + V BD-rate) /16) from constraining chroma blocks 2xN and Nx2 are similar, i.e., around 0.31% for separate trees and 0.20% for single tree in AI configuration. When only disabling the chroma blocks containing fewer than 16 pixels, the average BD-rates are 0.07% for separate trees and 0.05% for single tree in AI configuration.

Conclusions

This contribution introduces two schemes for removing 2xN and Nx2 chroma blocks, one for N being any value, and the other for N being no greater than 4, and provides AI coding performance results.

It is recommended to further study the proposed method in CE.

Acknowledgement

We would like to thank HHI for cross-checking this proposal.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).