| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0377 |
| --- | --- |

| Title: | Adaptive Motion Vector Resolution Rounding Align |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Zhang, Chun-Chi Chen, Han Huang, Yu Han, Wei-Jung Chien, and Marta Karczewicz | Email: | yzh@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution proposes to align the motion vector (MV) rounding used in AMVR [1] to be the same with MV rounding in other inter modes as described in the spec draft, e.g. affine mode, where the MV components with negative values are rounded toward 0. Also, the current VTM’s two-round separate roundings used in constructing the AMVP candidate list is also modified to be jointly performed to align with the spec draft among different categories of AMVP candidates, e.g. spatial AMVP candidates, temporal AMVP candidates.

With the above mentioned changes are on top of VTM-2.0.1, the reported average Y BD-rate numbers are 0.02% in RA and -0.01% in LB.

# Introduction

Motion vector differences (MVDs) (between the motion vector and predicted motion vector of a PU) can be signalled in different precision. In the current working draft of VVC [2], a locally adaptive motion vector resolution (LAMVR) is introduced and MVD can be coded in units of quarter-pel luma samples, integer-pel luma samples or four-pel luma samples. Correspondingly, when adding MVD to the selected motion vector predictor (MVP) for the reconstructed motion vector (MV), MVP needs to be rounded to the same precision as the MVD. Due to the introduction of affine mode, the motion vector storage is also increased to be 1/16-pel luma sample precision. For this reason, when an AMVP candidate is in 1/16-pel luma sample precision, two rounding processes need to be applied in current VTM [3]: one for AMVR rounding and the other for setting the representation precision of the MV to ¼-pel luma sample.

# Technical Description

There are two major issues with the current implementation of the AMVR rounding when combined with 1/16-pel luma sample precion motion vector.

The first issue is that the negative MV component rounding in AMVR is different from the MV rounding used in all other modes as defined in the spec draft. Taking the example of affine mode, the difference can be easily seen from the formulae below.

For AMVR MV rounding:

rMV.setHor( ( ( rMV.getHor() + offset ) >> imvShift ) << imvShift );		(1)

For Affine MV rounding:

mvx = mvx >= 0 ? (mvx + nOffset) >> nShift : -((-mvx + nOffset) >> nShift);	(2)

To address this issue, it is proposed to align the MV rounding for AMVR to be the same as for affine mode as following:

rMV.setHor(rMV.getHor() >= 0 ? ((rMV.getHor() + offset) >> imvShift)<< imvShift:

-(((- rMV.getHor() + offset) >> imvShift)) << imvShift);				(3)

The second issue is with respect to the construction of the AMVP candidate list. In the current VTM implementation, when constructing the list with AMVR mode on, a two-round rounding is performed if the AMVP candidate is stored in 1/16-pel luma sample precision: AMVR rounding (roundMv) and setting the MV representation to ¼-pel luma sample precision (setLowPrec). However, in the spec draft, these two rounding behaviors are defined to be jointly performed. It is proposed to modify VTM to be aligned with working draft.

# Test results

The Random Access (RA) and Low Delay B (LB) conditions as pecified in the common test conditions [4] are used in this contribution. The simulation results are reported in Table 1.

Table 1. Simulation result for proposed AMVR rounding align

# Conclusion

In this contribution, it is proposed to align the motion vector rounding used in AMVR to be the same as defined in the working draft such as Affine prediction, so that MV components with negative values are rounded towards 0. It is also proposed to merge the two-round rounding procedure when constructing the AMVP candidate list into one combined rounding when AMVR is turned on for different categories of AMVP candidates to align with the spec draft . The redulting BD-rate numbers are 0.02% and -0.01% on average for RA and LD, respectively. We would like to recommend the committees to consider adopting the proposed techniques in the corresponding VTM reference software.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- Jianle Chen, Yan Ye, Seung Hwan Kim, “Algorithm description for Versatile Video Coding and Test Model 2 (VTM 2)”, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- B. Bross, “Versatile Video Coding (Draft 1),” document JVET-K1001, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- Frank Bossen, Xiang Li and Karsten Suehring, “Guidelines for VVC reference software development “, document JVET-J1003, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- Frank Bossen, Jill Boyce, Karsten Suehring, Xiang Li, Vadim Seregin, “JVET common test conditions and software reference configurations for SDR video”, document JVET-J1010, 11th Meeting: Ljubljana, SI, 10–18 July 2018.