| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0381-v4 |
| --- | --- |

| Title: | CE3-related: 4-tap interpolation filter selection with quantization parameter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kei Kawamura Kyohei Unno Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes 4-tap cubic and Gaussian interpolation filter selection with quantization parameter (QP) which is related to Core Experiment 3 (CE3). These filters are applied to JEM-7.0 and are studied in CE3.3 and which filter should be used is selected with intra block sizes and intra directional modes. The thresholds of block size to separate the usage of both filters is kept constant at 8 width or height even though QP values change. In this contribution, the thresholds are proposed to be optimized by versatile QP ranges and the simulation results for this technique reportedly show -0.40%/-0.14%/-0.15% BD-rate for luma gains in AI/RA/LDB respectively at CTC QP range compared to VTM2.0.1 where 2-tap interpolation is applied. Moreover, from the simulation results by versatile QP range, it is confirmed that at high QP range BD-rate for luma in AI increase -0.17% compared to the results with conventional block-sized-based selected filters.

Introduction and Problem Statement

In CE3 [1], different interpolation filter to the reference samples for directional intra prediction are studied to improve the coding efficiency. The 4-tap cubic and Gaussian filter are firstly applied to JEM-7.0 [2] and are currently proposed in JVET-J0017 [3] in CE3. In the applicable conditions of the interpolation filters, intra block size and intra modes define whether the two type filters are used: (i) 4-tap Cubic filter for blocks with the size smaller than or equal to 8 samples (or with the square smaller than or equal to 64 samples), and (ii) Gaussian interpolation filters for blocks with the size larger than 8 samples (or with the square larger than 64 samples). The above threshold of intra block size is the static value even though QP values of coding block change.

Proposed method

The proposed method makes decision on what interpolation filter uses should be selected by quantization parameter in addition to intra block sizes and intra modes. Table 1 show the proposed decision table. In contrast to the conventional decision table, the thresholds of block size to separate usage of 4-tap cubic and gaussian filters change by QP values.

Table 1 The proposed decision table which defines the usage of 4-tap cubic or Gaussian filter. W|H, 4-C and 4-G indicate block width or height, 4-tap cubic filter and 4-tap Gaussian filter respectively. The redline indicates the threshold of block size to separate the usage of both filters.

|  | W¦H = 4 | W¦H = 8 | W¦H = 16 | W¦H = 32 | W¦H => 64 |
| --- | --- | --- | --- | --- | --- |
| QP < 17 | 4-C | 4-G | 4-G | 4-G | 4-G |
| QP = 17 | 4-C | 4-G | 4-G | 4-G | 4-G |
| QP = 18 | 4-C | 4-C | 4-G | 4-G | 4-G |
| QP = … | 4-C | 4-C | 4-G | 4-G | 4-G |
| QP = 24 | 4-C | 4-C | 4-G | 4-G | 4-G |
| QP = 25 | 4-C | 4-C | 4-C | 4-G | 4-G |
| QP = … | 4-C | 4-C | 4-C | 4-G | 4-G |
| QP = 36 | 4-C | 4-C | 4-C | 4-G | 4-G |
| QP = 37 | 4-C | 4-C | 4-C | 4-C | 4-G |
| QP > 37 | 4-C | 4-C | 4-C | 4-C | 4-G |

Experimental Results

Coding conditions

The proposed interpolation filters were implemented on top of the BMS-2.0.1 software and we have conducted simulations in accordance with CTC in JVET-K1010 [4]. To verify the impact of this proposal for versatile QPs range we obtained simulation data at QP =12, 17, 22, 27, 32, 37, 42, 47, 52 and 57 range.

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Objective results

As a matter of convenience, the simulation data of versatile QP range show separately three QP range as follows:

- Low QP range: QP = 12, 17, 22, 27

- CTC QP range: QP = 22, 27, 32, 37

- High QP range: QP = 32 37, 42, 47

It is noted that the anchor of the all simulation results below is VTM-2.0.1 which means we obtained these results with BMS.exe and VTM.cfg and VTM-2.0.1 is applied to not 4-tap cubic and Gaussian filter but 2-tap bilinear filter by default. The simulation results of our proposal compared to conventional (block-sized-based selected) 4-tap cubic and Gaussian filter are described as Appendix of this contribution.

Low QP range (QP = 12, 17, 22, 27)

From Table 2 to Table 4 show the low QP range results and overall BD-rates for luma in AI/RA/LDB are -0.30%/-0.11%-0.06% without increasing running time.

Table 2 Experimental results for AI condition compared to VTM-2.0.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.13% | -0.09% | 104% | 103% |
| Class A2 | -0.14% | -0.09% | -0.13% | 103% | 102% |
| Class B | -0.33% | -0.27% | -0.28% | 103% | 101% |
| Class C | -0.42% | -0.44% | -0.48% | 100% | 100% |
| Class E | -0.41% | -0.43% | -0.45% | 102% | 101% |
| Overall | -0.30% | -0.28% | -0.30% | 102% | 101% |
| Class D | -0.32% | -0.30% | -0.34% | 100% | 97% |

Table 3 Experimental results for RA condition compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.22% | -0.14% | 102% | 100% |
| Class A2 | -0.06% | -0.26% | -0.27% | 101% | 99% |
| Class B | -0.09% | -0.26% | -0.37% | 101% | 100% |
| Class C | -0.17% | -0.17% | -0.23% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.23% | -0.26% | 101% | 100% |
| Class D | -0.13% | -0.21% | -0.21% | 101% | 100% |

Table 4 Experimental results for LDB condition compared to VTM-2.0.1

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.10% | -0.10% | 101% | 100% |
| Class C | -0.06% | -0.15% | -0.05% | 101% | 100% |
| Class E | -0.04% | 0.15% | -0.27% | 101% | 100% |
| Overall | -0.06% | -0.05% | -0.12% | 101% | 100% |
| Class D | -0.07% | -0.11% | -0.05% | 101% | 100% |

CTC QP range (QP = 22, 27, 32, 37)

From Table 5 to Table 7 show the CTC QP range results and overall BD-rates for luma in AI/RA/LDB are -0.40%/-0.14%-0.08% without increasing running time.

Table 5 Experimental results for AI condition compared to VTM-2.0.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.13% | -0.14% | 104% | 100% |
| Class A2 | -0.16% | -0.25% | -0.34% | 103% | 101% |
| Class B | -0.39% | -0.58% | -0.55% | 103% | 101% |
| Class C | -0.68% | -1.03% | -0.96% | 101% | 101% |
| Class E | -0.63% | -0.62% | -0.87% | 102% | 101% |
| Overall | -0.40% | -0.56% | -0.59% | 103% | 101% |
| Class D | -0.50% | -0.56% | -0.90% | 101% | 100% |

Table 6 Experimental results for RA condition compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.22% | -0.11% | 101% | 101% |
| Class A2 | -0.06% | -0.30% | -0.33% | 101% | 100% |
| Class B | -0.14% | -0.45% | -0.37% | 101% | 101% |
| Class C | -0.24% | -0.30% | -0.35% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.33% | -0.30% | 101% | 100% |
| Class D | -0.23% | -0.69% | -0.22% | 101% | 100% |

Table 7 Experimental results for LDB condition compared to VTM-2.0.1

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.24% | -0.19% | 101% | 100% |
| Class C | -0.09% | -0.25% | -0.31% | 101% | 100% |
| Class E | -0.15% | -0.18% | 0.57% | 101% | 99% |
| Overall | -0.08% | -0.23% | -0.04% | 101% | 100% |
| Class D | -0.06% | 0.28% | -0.19% | 101% | 100% |

High QP range (QP = 32, 37, 42, 47)

From Table 8 to Table 10 show the high QP range results and overall BD-rates for luma in AI/RA/LDB -0.37%/-0.15%/0.01% without increasing running time.

Table 8 Experimental results for AI condition compared to VTM-2.0.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.09% | -0.15% | 106% | 99% |
| Class A2 | -0.04% | -0.12% | -0.11% | 104% | 99% |
| Class B | -0.46% | -0.66% | -0.48% | 104% | 100% |
| Class C | -0.63% | -0.56% | -1.01% | 103% | 103% |
| Class E | -0.52% | -0.62% | -0.67% | 103% | 102% |
| Overall | -0.37% | -0.45% | -0.51% | 104% | 101% |
| Class D | -0.43% | -0.70% | -1.09% | 103% | 103% |

Table 9 Experimental results for RA condition compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.05% | 0.02% | 101% | 101% |
| Class A2 | -0.05% | -0.15% | -0.11% | 101% | 100% |
| Class B | -0.26% | -0.52% | -0.40% | 102% | 103% |
| Class C | -0.26% | -0.84% | -0.13% | 101% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.15% | -0.44% | -0.19% | 101% | 103% |
| Class D | -0.31% | -0.79% | -0.37% | 101% | 112% |

Table 10 Experimental results for LDB condition compared to VTM-2.0.1

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.10% | -0.39% | 101% | 100% |
| Class C | 0.09% | -0.11% | -0.29% | 101% | 100% |
| Class E | 0.00% | 0.15% | 0.01% | 101% | 100% |
| Overall | 0.01% | -0.04% | -0.26% | 101% | 100% |
| Class D | -0.04% | 0.78% | -0.77% | 101% | 100% |

Conclusion

This contribution proposes 4-tap cubic and Gaussian interpolation filter selection with quantization parameter (QP). The thresholds of block size were proposed to be optimized by versatile QP ranges and the simulation results for this technique reportedly showed -0.40%/-0.14%/-0.15% BD-rate for luma gains in AI/RA/LDB respectively at CTC QP range compared to VTM2.0.1 where 2-tap interpolation is applied. From the simulation results by versatile QP range, it was confirmed that at high QP range BD-rate for luma in AI increase -0.17% compared to the results with conventional block-sized-based selected filters so further study in CE3 was recommended.

- Acknowledgment

We would like to thank LGE and Huawei for doing crosschecking of our proposal.

References

- G. Auwera, J. Heo and A. Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding”, JVET-K1023, Ljubljana, SI, July 2018.

- J. Chen, E. Alshina, G. J. Sullivan, J. R. Ohm, J. Boyce, “Algorithm description of Joint Exploration Test Model 7 (JEM7)”, JVET-G1001, Torino, IT, July 2018.

- M. Koo, J. Heo, J. Nam, N. Park, J. Lee, J. Choi, S. Yoo H. Jang, L. Ki, J. Lim, S. Paluri, M. Salehifar and S. Kim, “Description of SDR video coding technology proposal by LG Electronics”, JVET-J0017, San-Diego, US, April 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Suehring, “JVET common test conditions and software reference configurations”, JVET-K1010, Ljubljana, SI, July 2018.

- S. Yoo, J. Heo, J. Choi, L. Li, J. Lim, “CE3-3.1.1 : Interpolation filter selection regarding intra mode and block size”, JVET-L0130, Macau, CN, October 2018.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix

The simulation results of our proposal compared to conventional (block-sized-based selected) 4-tap cubic and Gaussian filter are described as follows.

Low QP range (QP = 12, 17, 22, 27)

Table 11 Experimental results for AI condition compared with VTM-2.0.1 with the block-size-based selected 4-tap cubic/Gaussian filter

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.04% | -0.02% | 100% | 103% |
| Class A2 | -0.01% | 0.05% | 0.03% | 100% | 102% |
| Class B | -0.12% | -0.06% | -0.03% | 100% | 101% |
| Class C | 0.08% | 0.07% | 0.07% | 98% | 99% |
| Class E | 0.10% | 0.08% | 0.06% | 100% | 101% |
| Overall | -0.01% | 0.01% | 0.02% | 100% | 101% |
| Class D | 0.09% | 0.07% | 0.08% | 99% | 98% |

Table 12 Experimental results for RA condition compared with VTM-2.0.1 with the block-size-based selected 4-tap cubic/Gaussian filter

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.06% | 0.04% | 100% | 100% |
| Class A2 | 0.00% | 0.02% | 0.03% | 100% | 100% |
| Class B | -0.04% | -0.01% | -0.04% | 100% | 99% |
| Class C | 0.00% | 0.05% | 0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.03% | 0.02% | 100% | 100% |
| Class D | 0.01% | 0.02% | 0.04% | 100% | 100% |

Table 13 Table 14 Experimental results for LDB condition compared with VTM-2.0.1 with the block-size-based selected 4-tap cubic/Gaussian filter

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.01% | 0.00% | 100% | 100% |
| Class C | 0.01% | 0.01% | 0.02% | 100% | 100% |
| Class E | 0.03% | -0.03% | -0.25% | 100% | 100% |
| Overall | 0.00% | 0.00% | -0.06% | 100% | 100% |
| Class D | 0.00% | -0.03% | -0.04% | 100% | 100% |

CTC QP range (QP = 22, 27, 32, 37)

Table 15 Experimental results for AI condition compared with VTM-2.0.1 with the block-size-based selected 4-tap cubic/Gaussian filter

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | -0.06% | -0.09% | 100% | 100% |
| Class A2 | -0.03% | -0.10% | -0.17% | 99% | 100% |
| Class B | -0.03% | -0.20% | -0.19% | 99% | 100% |
| Class C | -0.11% | -0.18% | -0.15% | 99% | 100% |
| Class E | -0.14% | -0.31% | -0.33% | 99% | 100% |
| Overall | -0.04% | -0.17% | -0.18% | 99% | 100% |
| Class D | 0.01% | -0.12% | -0.28% | 100% | 99% |

Table 16 Experimental results for RA condition compared with VTM-2.0.1 with the block-size-based selected 4-tap cubic/Gaussian filter

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.02% | 0.03% | 100% | 101% |
| Class A2 | 0.01% | 0.00% | -0.06% | 100% | 100% |
| Class B | 0.02% | -0.11% | -0.12% | 100% | 101% |
| Class C | 0.01% | 0.04% | 0.18% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.02% | 0.00% | 100% | 100% |
| Class D | 0.01% | -0.03% | -0.01% | 100% | 100% |

Table 17 Table 18 Experimental results for LDB condition compared with VTM-2.0.1 with the block-size-based selected 4-tap cubic/Gaussian filter

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.06% | 0.22% | 100% | 100% |
| Class C | -0.04% | 0.07% | -0.11% | 100% | 100% |
| Class E | 0.11% | -0.40% | 0.95% | 100% | 100% |
| Overall | 0.03% | -0.10% | 0.29% | 100% | 100% |
| Class D | -0.01% | 0.46% | -0.16% | 100% | 100% |

High QP range (QP = 32, 37, 42, 47)

Table 19 Experimental results for AI condition compared with VTM-2.0.1 with the block-size-based selected 4-tap cubic/Gaussian filter

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.06% | -0.11% | 101% | 98% |
| Class A2 | 0.03% | -0.01% | -0.06% | 100% | 98% |
| Class B | -0.25% | -0.59% | -0.55% | 100% | 99% |
| Class C | -0.31% | -0.52% | -0.56% | 100% | 101% |
| Class E | -0.32% | -0.91% | -0.65% | 99% | 100% |
| Overall | -0.17% | -0.44% | -0.41% | 100% | 100% |
| Class D | -0.10% | -0.53% | -0.53% | 100% | 104% |

Table 20 Experimental results for RA condition compared with VTM-2.0.1 with the block-size-based selected 4-tap cubic/Gaussian filter

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.02% | 0.04% | 100% | 100% |
| Class A2 | -0.04% | -0.08% | 0.09% | 100% | 100% |
| Class B | -0.07% | -0.35% | -0.32% | 100% | 101% |
| Class C | -0.01% | -0.25% | 0.17% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.20% | -0.03% | 100% | 100% |
| Class D | -0.12% | -0.22% | 0.17% | 100% | 100% |

Table 21 Experimental results for LDB condition compared with VTM-2.0.1 with the block-size-based selected 4-tap cubic/Gaussian filter

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.42% | -0.42% | 100% | 100% |
| Class C | 0.00% | 0.23% | -0.10% | 100% | 100% |
| Class E | 0.04% | -0.06% | 0.23% | 100% | 100% |
| Overall | 0.00% | -0.11% | -0.15% | 100% | 100% |
| Class D | -0.10% | -0.05% | -0.09% | 100% | 101% |

We have conducted the simulation where we introduced our proposal into CE3.1.1[5] to evaluate interaction between our proposal and CE3.1.1. The simulation has been conducted only AI at CTC-QP and High-QP range.

CTC QP range (QP = 22, 27, 32, 37)

At CTC-QP range, simulation results show that the coding efficiency of CE3.1.1 with our proposal improves -0.04%/-0.06%/-0.10% comparing that of CE3.1.1. Besides, encoder and decoder runtime of CE3.1.1 with our proposal decrease 1%/1% comparing that of CE3.1.1.

Table 22 Result of CE3.1.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.16% | -0.15% | 103% | 100% |
| Class A2 | -0.15% | -0.07% | -0.14% | 103% | 101% |
| Class B | -0.41% | -0.41% | -0.40% | 103% | 101% |
| Class C | -0.65% | -0.78% | -0.69% | 101% | 102% |
| Class E | -0.59% | -0.71% | -0.64% | 102% | 101% |
| Overall | -0.41% | -0.44% | -0.42% | 102% | 101% |
| Class D | -0.53% | -0.45% | -0.69% | 101% | 101% |

Table 23 Result of CE3.1.1 with our proposal

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.07% | -0.12% | 102% | 100% |
| Class A2 | -0.17% | -0.15% | -0.18% | 102% | 100% |
| Class B | -0.43% | -0.43% | -0.48% | 101% | 101% |
| Class C | -0.76% | -0.93% | -0.93% | 100% | 101% |
| Class E | -0.73% | -0.80% | -0.79% | 100% | 100% |
| Overall | -0.45% | -0.50% | -0.52% | 101% | 100% |
| Class D | -0.55% | -0.32% | -0.61% | 100% | 100% |

High QP range (QP = 32, 37, 42, 47)

At high-QP range, simulation results show that the coding efficiency of CE3.1.1 with our proposal improves -0.23%/-0.19%/-0.21% comparing that of CE3.1.1. Besides, encoder and decoder runtime of CE3.1.1 with our proposal decrease 2%/1% comparing that of CE3.1.1.

Table 24 Result of CE3.1.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.16% | -0.16% | 103% | 100% |
| Class A2 | -0.03% | -0.20% | -0.04% | 103% | 100% |
| Class B | -0.20% | -0.07% | 0.02% | 104% | 101% |
| Class C | -0.37% | 0.01% | -0.16% | 103% | 102% |
| Class E | -0.20% | 0.02% | -0.18% | 103% | 101% |
| Overall | -0.20% | -0.08% | -0.09% | 103% | 101% |
| Class D | -0.40% | -0.46% | -0.62% | 102% | 101% |

Table 25 Result of CE3.1.1 with our proposal

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.08% | -0.01% | 102% | 100% |
| Class A2 | -0.10% | -0.07% | -0.02% | 102% | 100% |
| Class B | -0.52% | -0.44% | -0.40% | 101% | 100% |
| Class C | -0.72% | -0.30% | -0.71% | 101% | 101% |
| Class E | -0.57% | -0.35% | -0.61% | 101% | 101% |
| Overall | -0.43% | -0.27% | -0.38% | 101% | 100% |
| Class D | -0.55% | -0.32% | -0.61% | 100% | 100% |