| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0384-v1 |
| --- | --- |

| Title: | CE7: Entropy Coding for Dependent Quantization (test 7.1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong Muhammed Coban Marta Karczewicz | Email: | jiedong@qti.qualcomm.com mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this document a dependent quantization entropy coding scheme that avoids interleaving of syntax elements, reduces maximum number of regular coded bins relative to VTM-2 and groups bypass coded bins. The state machine driving the quantizer selection and coefficient coding algorithm is driven by the significance of a coefficient rather than the coefficient level parity allowing non-interleaved syntax element coding. The simulations results show 0.20% AI, 0.19% RA, -0.04% LB BD-Rate versus the VTM-2.0 anchor for common test conditions. For low-complexity encoding configuration simulation results show, 0.02% AI, 0.04% RA, -0.01% LB BD-Rate versus the VTM-2.0 low-complexity encoding anchor.

# Introduction

In VTM-2 dependent quantization and coefficient coding algorithm the dependent quantization state machine states depend on the parity of the coefficient levels. The significance flag coding context as well as the quantizers used for a coefficient depend on the state of the previously coded coefficient. In order to group by-pass coded bins in a coefficient group, the significance (sig) and the parity (par) flags have to be interleaved, hence in the first coding pass sig, par, and gt1_flags are interleaved.

In this proposal an alternate dependent quantization entropy coding scheme that allows grouping of bypass coded bins at coefficient group level while avoiding interleaving of syntax elements, with reduced maximum regular coded bins is presented [1].

# Proposed algorithm

In this algorithm a 4-state, two scalar quantizer Trellis Coded Quantization described in JVET-K0070 is used [3].  In the proposed scheme, in a coefficient group, significance of information of all coefficients are coded in the first pass using the same neighboring coefficient info template for context derivation as in JVET-K0070 using regular coded bins (maximum 16). In the second pass, up to 10 gt1 flags of coefficients are coded using regular coded bins.  In the third pass, up to 2 gt2_flags are coded as regular coded bins. After the third pass, all remaining absolute levels for coefficients are coded as bypass coded bins using the described in JVET-K0070. Total of maximum 28 bins per coefficient group are coded as regular coded bins.

The sig flag context derivation is modified to support non-interleaved coding of coefficient syntax elements. Instead of using coefficient level information from coding up to gt4 flag in JVET-K0070, only sum of significance of neighboring template coefficients are used for sumAbs1 derivation. The context index is derived as follows.

- For luma component,

ctxIdSig = 18 * max(0, state−1) + min( sumAbs1, 5 ) + ( d < 2 ? 12 : ( d < 5 ? 6 : 0 ) ),

- For chroma component,

ctxIdSig = 12 * max(0, state−1) + min(sumAbs1, 5 ) + ( d < 2 ? 6 : 0 ) )

for diagonal position d, d = x + y.

Similarly, for context index derivation of gt1 and gt2 flags, partial sum of neighboring template coefficients (sumAbsX, X:{2,3}) are used.

- For luma component, the context index becomes

ctxIdGtX = 1 + min(sumAbsX– numSig, 4 ) + ( d == 0 ? 15 : ( d < 3 ? 10 : ( d < 10 ? 5 : 0 ) ) )

- For chroma component, the context index becomes

ctxIdGtX = 1 + min(sumAbsX– numSig, 4 ) + ( d == 0 ? 5 : 0 ).

The rest of coefficient coding is left unchanged.

# Results

The proposed scheme is implemented on top of VTM-2.0.1 software [2].  The simulations were carried out under the JVET common test conditions and CE7 low complexity encoding conditions [1][4].

The test results for high complexity case is tabulated in Table 1.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.27% | 0.44% | 0.54% | 112% | 103% |
| Class A2 | -0.52% | -0.13% | -0.18% | 114% | 106% |
| Class B | 0.36% | 0.53% | 0.56% | 114% | 103% |
| Class C | 0.38% | 0.61% | 0.74% | 116% | 105% |
| Class E | 0.32% | 0.41% | 0.34% | 114% | 102% |
| Overall | 0.20% | 0.40% | 0.44% | 114% | 104% |
| Class D | 0.35% | 0.77% | 0.42% | 117% | 109% |
| Class F (optional) | 0.55% | 0.93% | 0.86% | 118% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.24% | 0.35% | 0.47% | 105% | 102% |
| Class A2 | 0.07% | 0.14% | 0.01% | 107% | 104% |
| Class B | 0.16% | 0.18% | 0.30% | 105% | 100% |
| Class C | 0.27% | 0.56% | 0.49% | 107% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.19% | 0.31% | 0.33% | 106% | 101% |
| Class D | 0.32% | 0.23% | 0.48% | 108% | 100% |
| Class F (optional) | 0.57% | 0.88% | 0.83% | 109% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.12% | 0.50% | 105% | 100% |
| Class C | 0.12% | 0.37% | 0.06% | 106% | 100% |
| Class E | -0.32% | 0.96% | -0.50% | 103% | 101% |
| Overall | -0.04% | 0.41% | 0.10% | 105% | 100% |
| Class D | 0.10% | 0.65% | -0.17% | 106% | 102% |
| Class F (optional) | 0.27% | 0.51% | 0.73% | 106% | 103% |

Table 1: Average Bjøntegaard delta rate savings of the proposed coefficient coding (CE7.1.2) relative to VTM-2.0.1 in high-complexity encoding mode (CTC, dependent quantization enabled).

The test results for lowd complexity case is tabulated in Table 2.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.06% | 0.07% | 105% | 104% |
| Class A2 | 0.03% | 0.09% | 0.03% | 106% | 104% |
| Class B | 0.01% | 0.09% | 0.07% | 105% | 101% |
| Class C | 0.00% | 0.08% | 0.06% | 107% | 100% |
| Class E | 0.06% | 0.06% | 0.13% | 105% | 100% |
| Overall | 0.02% | 0.08% | 0.07% | 106% | 101% |
| Class D | 0.01% | 0.10% | -0.04% | 108% | 100% |
| Class F (optional) | 0.16% | 0.21% | 0.45% | 109% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.14% | 0.07% | 103% | 102% |
| Class A2 | 0.03% | -0.03% | -0.17% | 104% | 102% |
| Class B | 0.04% | 0.10% | -0.16% | 99% | 98% |
| Class C | 0.03% | -0.03% | -0.11% | 100% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.05% | -0.10% | 101% | 99% |
| Class D | 0.05% | 0.05% | 0.05% | 102% | 97% |
| Class F (optional) | 0.30% | 0.37% | 0.64% | 101% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.04% | 0.33% | 101% | 102% |
| Class C | -0.03% | -0.03% | 0.29% | 101% | 100% |
| Class E | 0.02% | -0.40% | -0.64% | 100% | 98% |
| Overall | -0.01% | -0.13% | 0.08% | 101% | 100% |
| Class D | 0.03% | -0.27% | 0.27% | 101% | 99% |
| Class F (optional) | 0.07% | 0.24% | 0.42% | 101% | 97% |

Table 2: Average Bjøntegaard delta rate savings of the proposed coefficient coding (CE7.1.2) relative to VTM-2.0.1 in low-complexity encoding mode (CTC, RDOQ disabled).

# Conclusion

In this document an alternate dependent quantization entropy coding scheme that allows grouping of bypass coded bins at coefficient group level was described. The state machine driving the quantizater selection and coefficient coding algorithm is driven by the significance of a coefficient rather than the coefficient level parity allowing non-interleaved syntax element coding as well as grouping of remaining level bypass coded bins for efficient parsing.

# References

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” JVET-K1027, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

- https://vcgit.hhi.fraunhofer.de/JVET-K-CE7/VVCSoftware_VTM/tree/CE7.1.2

- M. Coban, J. Dong, M. Karczewicz, “Non-CE7: Entropy Coding for Dependent Quantization”, JVET-K0070, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

- “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).