| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0389-v1 |
| --- | --- |

| Title: | CE4-related: Control point MV offset for Affine merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chiao Yang Yao-Jen Chang | Email: | johnny.yc.yang@fii-foxconn.com morriss.yj.chang@foxconn.com |
| Source: | Foxconn |  |  |

_____________________________

- Abstract

This contribution proposes to extend more Affine merge candidates based on the control points motion vectors (CPMVs) offsets of the first Affine merge candidate in the list. Extended Affine merge candidates are created with different offset directions and different offset magnitudes to the first Affine merge candidate. The proposed CPMV offset is implemented on top of Affine common code base (CE4.2.1). Compared to VTM2.0.1, -x.x%/- x.x%/- x.x% YUV-BDrate with 10x/10x% coding time change are achieved in RA condition, and -x.x%/- x.x%/- x.x% YUV-BDrate with 10x/10x% coding time change are achieved in LB condition.

Introduction

Affine merge mode uses the MV information of the neighboring blocks to derive the Affine control points motion vectors (CPMVs) of the current CU. CE4.2.1 is a common codebase [1] for Affine merge mode. This contribution proposes an improved Affine merge mode on top of CE4.2.1 by extending more Affine merge candidates from the first Affine merge candidate in the list.

The proposed Affine merge mode

New Affine merge candidates are extended from the first Affine merge candidate. The CPMVs of the new Affine merge candidates are the CPMVs offset of the first Affine merge candidate. If the first Affine merge candidate uses 4-parameter Affine model, then 2 CPMVs are generated; Otherwise (6-parameter Affine model), then 3 CPMVs are generated. In this contribution, various offset directions with various offset magnitudes are used to generate new Affine merge candidates.

Experiment result

The proposed CPMV offset is applied to Affine merge common codebase (CE4.2.1), and was tested to compare with the VTM2.0.1 under common test conditions [2]. Table 1 shows the experimental results.

Table 1. Summary of experimental results (CPMV v.s. VTM anchor)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.08% | -0.86% | -0.94% | 128% | 103% |
| Class C | -0.65% | -0.49% | -0.67% | 126% | 105% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.81% | -0.63% | -0.88% | 129% | 108% |
| Class F (optional) | -0.22% | -0.19% | -0.20% | 118% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.50% | -0.35% | -0.53% | 125% | 106% |
| Class E | -0.77% | -0.79% | -0.81% | 132% | 107% |
| Overall |  |  |  |  |  |
| Class D | -0.68% | 0.25% | -0.75% | 123% | 108% |
| Class F (optional) |  |  |  |  |  |

Conclusion

This contribution proposes CPMV offset to add more Affine merge candidates based on the first Affine candidate in the candidate list. It is observed that the proposed CPMV offset merge candidate provides -x.x%/- x.x%/- x.x% YUV-BDrate with 10x/10x% coding time.

Reference

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-K1024, Ljubljana, Slovenia, Jul. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Ljubljana, Slovenia, Jul. 2018.

Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).