| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0389r2 |
| --- | --- |

| Title: | CE4-related: Control point MV offset for Affine merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chiao Yang Yao-Jen Chang | Email: | johnny.yc.yang@mail.foxconn.com morriss.yj.chang@foxconn.com |
| Source: | Foxconn |  |  |

_____________________________

- Abstract

This contribution proposes to generate new Affine merge candidates based on the control points motion vectors (CPMVs) offsets of the first candidate in the Affine merge list. The new Affine merge candidates are created with different offset directions and different offset magnitudes to the first Affine merge candidate. Two experiments were implemented on top of Affine common code base (CE4.2.1). The first experiment selects at most 20 candidates among 15 original Affine merge candidates and 16 new CPMV offset candidates into the Affine merge list. Compared to VTM2.0.1, -1.16%/-0.91%/-1.08% YUV-BDrate with 127/104% coding time change are observed in RA condition, and -0.59%/-0.55%/-0.68% YUV-BDrate with 128/105% coding time change are observed in LB condition. Another experiment selects at most 5 candidates among 11 original Affine merge candidates (4 original candidates are removed from the design) and 4 new CPMV offset candidates into the Affine merge list. Compared to VTM2.0.1, -0.85%/-0.72%/-0.90% YUV-BDrate with 108/106% coding time change are observed in RA condition, and -0.52%/-0.47%/-0.57% YUV-BDrate with 109/109% coding time change are observed in LB condition.

Introduction

Affine merge mode uses the MV information of the neighboring and temporal blocks to derive the Affine control points motion vectors (CPMVs) of the current CU. CE4.2.1 is a common codebase [1] for Affine merge mode. This contribution proposes an improved Affine merge mode on top of CE4.2.1 by adding new Affine merge candidates based on the CPMV offsets of the first Affine merge candidate.

The proposed Affine merge mode

New Affine merge candidates are generated based on the CPMVs offsets of the first Affine merge candidate. If the first Affine merge candidate enables 4-parameter Affine model, then 2 CPMVs for each new Affine merge candidate are derived by offsetting 2 CPMVs of the first Affine merge candidate; Otherwise (6-parameter Affine model enabled), then 3 CPMVs for each new Affine merge candidate are derived by offsetting 3 CPMVs of the first Affine merge candidate. In Uni-prediction, the CPMV offsets are applied to the CPMVs of the first candidate. In Bi-prediction with List 0 and List 1 on the same direction, the CPMV offsets are applied to the first candidate as follows:

MVnew(L0), i = MVold(L0) + MVoffset(i)

MVnew(L1), i = MVold(L1) + MVoffset(i)

In Bi-prediction with List 0 and List 1 on the opposite direction, the CPMV offsets are applied to the first candidate as follows:

MVnew(L0), i = MVold(L0) + MVoffset(i)

MVnew(L1), i = MVold(L1) - MVoffset(i)

In this contribution, various offset directions with various offset magnitudes are used to generate new Affine merge candidates. Two implementations were tested:

- 16 new Affine merge candidates with 8 different offset directions with 2 different offset magnitudes are generated as shown in the following offsets set:

Offset set = { (4, 0), (0, 4), (-4, 0), (0, -4), (-4, -4), (4, -4), (4, 4), (-4, 4), (8, 0), (0, 8), (-8, 0), (0, -8), (-8, -8), (8, -8), (8, 8), (-8, 8) }.

The Affine merge list is increased to 20 for this design. The number of potential Affine merge candidates is 31 in total.

- 4 new Affine merge candidates with 4 different offset directions with 1 offset magnitude are generated as shown in the following offsets set:

Offset set = { (4, 0), (0, 4), (-4, 0), (0, -4) }.

The Affine merge list is kept to 5 as VTM2.0.1 does. Four temporal constructed Affine merge candidates are removed to keep the number of potential Affine merge candidates unchanged, i.e., 15 in total. Suppose the coordinates of CPMV1, CPMV2, CPMV3 and CPMV4 are (0, 0), (W, 0), (H, 0) and (W, H). Note that CPMV4 is derived from the temporal MV as shown in Fig. 1. The removed candidates are the following four temporal-related constructed Affine merge candidates: {CP2, CP3, CP4}, {CP1, CP4}, {CP2, CP4}, {CP3, CP4}.

Fig. 1. Candidates position for affine merge mode

Experiment result

The proposed CPMV offset candidates are applied to Affine merge common codebase (CE4.2.1), and was tested to compare with the VTM2.0.1 under common test conditions [2]. Table 1 shows the experimental results for 16 new Affine merge candidates with Affine merge list size = 20. Table 2 shows the experimental results for 4 new Affine merge candidates with Affine merge list size = 5 (the same as Affine common codebase).

Table 1. Summary of experimental results (The proposed 16 new CPMV offset candidates with Affine merge list size = 20 v.s. VTM anchor)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.47% | -0.94% | -1.19% | 123% | 102% |
| Class A2 | -1.66% | -1.54% | -1.74% | 130% | 109% |
| Class B | -1.08% | -0.86% | -0.94% | 128% | 103% |
| Class C | -0.65% | -0.49% | -0.67% | 126% | 105% |
| Class E |  |  |  |  |  |
| Overall | -1.16% | -0.91% | -1.08% | 127% | 104% |
| Class D | -0.81% | -0.63% | -0.88% | 129% | 108% |
| Class F (optional) | -0.22% | -0.19% | -0.20% | 118% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.55% | -0.58% | -0.72% | 129% | 105% |
| Class C | -0.50% | -0.35% | -0.53% | 125% | 106% |
| Class E | -0.77% | -0.79% | -0.81% | 132% | 107% |
| Overall | -0.59% | -0.55% | -0.68% | 128% | 105% |
| Class D | -0.68% | 0.25% | -0.75% | 123% | 108% |
| Class F (optional) |  |  |  |  |  |

Table 2. Summary of experimental results (The proposed 4 new CPMV offset candidates with Affine merge list size = 5 v.s. VTM anchor)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.10% | -0.68% | -1.02% | 107% | 110% |
| Class A2 | -1.16% | -1.17% | -1.29% | 109% | 109% |
| Class B | -0.78% | -0.67% | -0.84% | 108% | 102% |
| Class C | -0.53% | -0.48% | -0.57% | 108% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.85% | -0.72% | -0.90% | 108% | 106% |
| Class D | -0.71% | -0.61% | -0.77% | 109% | 107% |
| Class F (optional) | -0.17% | -0.19% | -0.21% | 106% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.43% | -0.37% | -0.60% | 109% | 114% |
| Class C | -0.42% | -0.34% | -0.37% | 108% | 105% |
| Class E | -0.82% | -0.82% | -0.77% | 109% | 107% |
| Overall | -0.52% | -0.47% | -0.57% | 109% | 109% |
| Class D | -0.59% | -0.30% | -0.68% | 107% | 106% |
| Class F (optional) | -0.05% | 0.19% | 0.36% | 107% | 106% |

Conclusion

This contribution proposes to generate new Affine merge candidates by adding CPMV offsets to the first candidate in the Affine candidate list. It is observed that the proposed CPMV offset with 16 new candidates under Affine merge list size =20 provides -1.16%/-0.91%/-1.08% YUV-BDrate with 127/104% coding time. It is further observed that the proposed CPMV offset with 4 new candidates under Affine merge list size = 5 provides -0.85%/-0.72%/-0.90% YUV-BDrate with 108/106% coding time. Since the better performance in terms of BD-rate and the complexity compared to the Affine common codebase (CE4.2.1), it is recommended to adopt the second method (without merge list size change) into the next version of VTM.

Reference

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-K1024, Ljubljana, Slovenia, Jul. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Ljubljana, Slovenia, Jul. 2018.

Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).