| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0392-v1 |
| --- | --- |

| Title: | CE2-related: Test results for corrected initial context states for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz | Tel: Email: | nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In VTM-2, for each coding tree block, a flag is signalled with context to indicate whether ALF is applied. This contribution presents the results of fix of the initial context states for CTB flags.

# Introduction

In VTM-2.0.1, the initial context state for CTB’s ALF enable flags is defined as

const CtxSet ContextSetCfg::ctbAlfFlag =

{

ContextSetCfg::addCtxSet

( {

{ 100, 100, 100, 100, 100, 100, 100, 100, 100 },

{ 153, 153, 153, 153, 153, 153, 153, 153, 153 },

{ 200, 200, 200, 200, 200, 200, 200, 200, 200 },

} )

};

Each line defines the initial context states for each slice type. In this contribution, corrected initial states are tested such that, for the same state index, all types of slices have consistent initial context state. These initial states were correctly mentioned in JVET-K0564.

const CtxSet ContextSetCfg::ctbAlfFlag =

{

ContextSetCfg::addCtxSet

({

{ 100, 153, 200, 100, 153, 200, 100, 153, 200 },

{ 100, 153, 200, 100, 153, 200, 100, 153, 200 },

{ 100, 153, 200, 100, 153, 200, 100, 153, 200 },

})

};

# Experimental Results

The tests were applied on VTM-2.0.1 with common test configuration.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
| AI | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.02% | 0.03% | 100% | 101% |
| Class B | 0.00% | 0.04% | 0.04% | 101% | 103% |
| Class C | 0.01% | 0.04% | 0.04% | 100% | 102% |
| Class E | 0.01% | 0.13% | 0.06% | 100% | 98% |
| Overall | 0.01% | 0.05% | 0.04% | 100% | 101% |
| Class D | 0.01% | 0.02% | 0.02% | 100% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
| RA | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.00% | -0.06% | 100% | 99% |
| Class A2 | 0.02% | -0.01% | -0.18% | 99% | 99% |
| Class B | -0.01% | -0.04% | -0.15% | 100% | 100% |
| Class C | -0.04% | 0.03% | -0.01% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.01% | -0.10% | 99% | 99% |
| Class D | -0.09% | -0.12% | -0.13% | 99% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
| LDB | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.22% | -0.19% | 101% | 103% |
| Class C | -0.03% | -0.11% | 0.02% | 100% | 100% |
| Class E | -0.03% | -0.45% | -0.55% | 99% | 99% |
| Overall | 0.00% | -0.24% | -0.21% | 100% | 101% |
| Class D | -0.02% | 0.33% | -0.20% | 100% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Patent rights declaration(s)

Qualcomm Incorporated does not have any current or pending patent rights relating to the technology described in this contribution.