| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0395-v2 |
| --- | --- |

| Title: | CE6-related: MTS with 4-point DST/DCT-4 and large block support |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Akshay Gadde, Vadim Seregin, Marta Karczewicz, Amir Said 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents test results for enabling MTS for inter CUs with the flowing modifications: for CU’s side length of 64 DCT-2 transform is used without signaling, MTS is not applied to 4x4 inter CUs, intra coefficient threshold based signaling is applied for inter MTS indices, encoder fast methods are applied. The experimental results show that the proposed changes provide 0.16%, 0.44% and 0.66% average luma BD-rate gains over VTM 2.0.1 under AI, RA and LDB configurations.

# Introduction

In VTM 2.0.1, pairs of DCT-2, DST-7 or DCT-8 are used for separable transformation of residual block, where DCT-2 is used up to size 64x64 and DST-7 and DCT-8 are allowed up to size 32x32. When MTS (AMT) flag is 0, DCT-2 is used for both horizontal and vertical transformation. If the MTS flag is 1, one of the four separable transform options derived from DST-7 and DCT-8 is signalled and used to transform a block.

The current VTM 2.0.1 has the following problems/restrictions:

- DST-7 and DCT-8 may not be good candidates for certain block sizes as the average block statistics vary depending on the block size. In other words, one type of DCT/DST may not provide a good energy compaction across different block sizes.

- For 64x64 only DCT-2 is allowed. However, blocks with size Nx64 or 64xN can still use different types of N-point MTS transforms for N < 64, which are available in VTM-2.0.1.

- Encoder can be further optimized by eliminating redundant searches.

This contribution addresses these problems and restrictions.

# Proposed methods

## Replacement of 4-point DST-7 and DCT-8 with DST-4 and DCT-4

In order to better capture the average residual statistics for small blocks, 4-point DST-7 and DCT-8 are replaced with DST-4 and DCT-4 respectively. Based on the experimental results, on average, DST-4 and DCT-4 provide better coding gains as compared to DST-7 and DCT-8 for small blocks.

The integer 4-point DST-4 used to replace 4-point DST-7 in our experiments is as follows:

{     71,    201,    301,    355 },

{    201,    355,     71,   -301 },

{    301,     71,   -355,    201 },

{    355,   -301,    201,    -71 },

The integer 4-point DCT-4 used to replace 4-point DCT-8 in our experiments is as follows:

{    355,    301,    201,     71 },

{    301,    -71,   -355,   -201 },

{    201,   -355,     71,    301 },

{     71,   -201,    301,   -355 },

## Enabling MTS for large blocks and signaling modification

To incorporate DST-7 or DCT-8 for large rectangular blocks with sizes Nx64 and 64xN for N<64, the 64-point DCT-2 is used to transform the longer side of the block, and N-point DCT-8 or N-point DST-7 is used for the shorter side. One bit is signalled to determine whether DST-7 or DCT-8 is used for the shorter side of the block, and no signaling is required for the side using DCT-2. Moreover, since current VTM has both DST-7 and DCT-8 available for N<64, no additional memory or transform kernels are required to implement this method.

In the test, inter MTS is not applied for 4x4 TUs, and MTS overhead is not signaled for those blocks.

The intra MTS coefficient threshold scheme is applied to inter MTS, i.e. if number of non-zero coefficients is less than 2, MTS index is not signaled, and transform index is inferred equal to 0.

## Encoder Optimizations

To lower encoder complexity the following encoder only methods are applied:

- MTS search is applied only for the best merge candidate, while DCT-2 is used to find the best merge candidate. In the current VTM, MTS is searched for all merge candidates.

- Zero out of 16x16 is applied for non DCT-2 transforms.

# Experimental Results

The proposed method is implemented based on BMS-2.0.1 software and tested under CTC in VTM configuration with enabling inter MTS (Inter AMT) via EMT=3 and EMTFast =3.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.01% | 101% | 101% |
| Class A2 | -0.08% | -0.06% | -0.08% | 101% | 103% |
| Class B | -0.12% | -0.12% | -0.03% | 102% | 99% |
| Class C | -0.26% | -0.23% | -0.05% | 100% | 101% |
| Class E | -0.33% | -0.25% | -0.32% | 100% | 102% |
| Overall | -0.16% | -0.13% | -0.09% | 101% | 101% |
| Class D | -0.32% | -0.04% | -0.47% | 100% | 98% |
| Class F (optional) | -0.17% | -0.17% | -0.25% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | 0.15% | 0.08% | 113% | 100% |
| Class A2 | -0.66% | 0.45% | 0.40% | 117% | 102% |
| Class B | -0.45% | 0.28% | 0.36% | 119% | 102% |
| Class C | -0.35% | 0.65% | 0.65% | 122% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.44% | 0.39% | 0.39% | 118% | 102% |
| Class D | -0.47% | 0.09% | 0.64% | 123% | 106% |
| Class F (optional) | -0.24% | 0.21% | 0.16% | 115% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.77% | 0.81% | 0.67% | 123% | 103% |
| Class C | -0.57% | 0.94% | 0.89% | 125% | 101% |
| Class E | -0.60% | 0.88% | 0.00% | 117% | 99% |
| Overall | -0.66% | 0.87% | 0.58% | 122% | 101% |
| Class D | -0.72% | 0.75% | 0.82% | 129% | 105% |
| Class F (optional) | -0.65% | 1.04% | 0.15% | 121% | 104% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).