| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0396 – v4 |
| --- | --- |

| Title: | CE4-related: Affine restrictions for the worst-case bandwidth reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van, Wei-Jung Chien, Han Huang, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Tel: Email: | +1-858-658-5664 lphamvan@qti.qualcomm.com |
| Source: | Qualcomm Technologies Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the restrictions to the affine mode for the worst-case bandwidth reduction. To ensure the worst-case bandwidth of the affine block is not worse than an INTER_4x8 or INTER_8x4 block or even INTER9x9 block, the motion vector differences between affine control points are used to decide whether the subblock size of the affine block is 4x4 or 8x8. The simulation results show average Luma BD-rate increase of 0.02% for RA and 0.01% for LDB conditions, respectively.

# Introduction

In real decoder implementation, it is desired to reduce bandwidth requirement, while keeping a good trade-off between bandwidth reduction and coding performance.

In VTM2.0, affine motion compensation is performed at 4x4 sub-block level. The motion vectors of these sub-blocks are derived using the motion vectors of the control points. Decoder needs to fetch to several sub-blocks in the memory buffer and the worst-case scenario is that the motion vectors of the sub-blocks are very different from each other and result in non-overlapped 4x4 blocks. This case leads to very high memory bandwidth access.

# Proposed Method

  - General affine restriction for worst-case bandwidth reduction

The memory bandwidth reduction for the affine mode is controlled by restricting the motion vector difference between the affine control points (also named as the control points difference). In general, if the control points differences satisfy the restriction below, the affine motion is using 4x4 subblocks (namely 4x4 affine mode). Otherwise, it is using 8x8 subblocks (8x8 affine mode). The restrictions for the 6-parameters and 4-parameters model are given as follows.

To derive the constraints for different block sizes (wxh), the motion vector differences of the control points are normalized as:

Normv1x-v0x=v1x-v0x*128/w

Normv1y-v0y=v1y-v0y*128/w

Normv2x-v0x=v2x-v0x*128/h

Normv2x-v0x=v2x-v0x*128/h			(1)

In the 4-parameters affine model, v2x-v0x and v2y-v0y  are set as the follows:

v2x-v0x=-v1y-v0y

(v2y-v0y)=-v1x-v0x							(2)

Hence, the Norms of v2x-v0x and v2y-v0y are given as:

Normv2x-v0x=-Normv1y-v0y

Norm(v2y-v0y)=Normv1x-v0x	     		        (3)

- The restriction to ensure the worst-case bandwidth is achieve an INTER_4x8 or INTER_8x4

Normv1x-v0x+Normv2x-v0x +128+

Normv1y-v0y+Normv2y-v0y +128+

Normv1x-v0x-Normv2x-v0x+

Normv1y-v0y-Normv2y-v0y

<  128*3.25			               (4)

where the left-hand side of (4) represent the shrink of span level of sub affine blocks and (3.25) indicates 3.25 pixels shift.

- The restriction to ensure the worst-case bandwidth is achieve an INTER_9x9

(4*Normv1x-v0x> -4*pel &&+4*Normv1x-v0x < pel) &&

(4*Normv1y-v0y > -pel && 4*Normv1y-v0y < pel) &&

4*Normv2x-v0x> -pel && 4*Normv2x-v0x < pel &&

4*Normv2y-v0y> -4*pel && 4*Normv2y-v0y < pel &&

((4*Normv1x-v0x + 4*Normv2x-v0x> -4*pel) &&

(4*Normv1x-v0x + 4*Normv2x-v0x< pel)) &&

((4*Normv1y-v0y + 4*Normv2y-v0y> -4*pel) &&

(4*Normv1y-v0y + 4*Normv2y-v0y< pel))		(5)

where pel=128*16 (128 and 16 indicate the normalization factor and motion vector precision, respectively).

# Experimental Results

The following tables indicate the performance of the tests under the VTM RA and VTM LB settings over VTM-2.0.1. In this test, the proposed restriction is applied to affine mode. The performances of the restriction approach to achieve an INTER8x4/INTER4x8 and an INTER_9x9 are summarised in Table 1 and Table 2, respectively.

It is reported in Table 1 that to achieve an INTER8x4 or INTER4x8, the restriction leads to a Luma loss of 0.01% and 0.00% for Class B and Class C, respectively under VTM RA. In the VTM LB condition, the proposed approach leads to a loss of 0.02% for class C and a gain of 0.03% for class E.

Table 1. Performance of the proposed affine restriction to achieve an INTER8x4 or INTER4x8

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.01% | 0.04% | -0.07% | 82% | 93% |
| Class C | 0.00% | 0.06% | 0.04% | 96% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.04% | 0.03% | 96% | 97% |
| Class F (optional) | 0.02% | 0.00% | 0.02% | 83% | 94% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.02% | 0.06% | -0.16% | 113% | 103% |
| Class E | -0.03% | -0.43% | -0.14% | 94% | 96% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.05% | 0.24% | 0.13% | 97% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2. Performance of the proposed affine restriction to achieve an INTER9x9

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.02% | 0.14% | -0.01% | 83% | 96% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.03% | 0.06% | 0.03% | 84% | 95% |
| Class F (optional) | 0.12% | 0.15% | 0.17% | 83% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.15% | -0.03% | 84% | 91% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusions

This contribution proposes an affine restriction to achieve the worst-case bandwidth as the same as INTER4x8 or INTER8x4, or INTER9x9 block. The restriction is applied to the motion vector difference of the control points. The proposed approach an average Luma BD-rate increase of 0.02% for RA and 0.01% for LDB conditions, respectively.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).