| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0400 |
| --- | --- |

| Title: | CE4-related: Simplification on Non-Adjacent Merge/Skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu Han, Wei-Jung Chien, Han Huang, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1-858-845-1795 yuhan@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution describes several modifications on Merge/Skip that were firstly presented in the contribution [1], several improvement and line buffer restriction algorithms were presented in proposals [2] and [3]. This contribution relating to CE4 test 4.4.6 [4] includes further optimization and simplification. For the “VTM2.0.1 as reference” tests in RA case with the size of merge list equaling to 10, the sub test a) “CE4.4.6.a + maximum number of pruning equals to 20” achieved objective gain of 0.73% in terms of average luma BD-rate improvement; the sub test b) “CE4.4.6.b + maximum number of pruning equals to 20” achieved -0.62% coding gain; c) “CE4.4.6.c + maximum number of pruning equals to 20” achieved -1.05% coding gain; d) “CE4.4.6.d + maximum number of pruning equals to 20” achieved -1.01% coding gain.

# Introduction

For the skip mode and the merge mode, a motion vector competition scheme is introduced to select a motion candidate from a given candidate set that includes spatial and temporal motion candidates. In the case of a skip mode, the residual signal is omitted. Only merge indices are transmitted for the block coded in merge or skip mode, and the current block inherits the inter prediction indicator, reference indices, and motion vectors from a neighboring block referred by the coded merge index.

In this contribution, non-adjacent spatial candidates and non-sub-PU based STMVP are included in the merge candidate list to improve coding efficiency.

# Technical description

The pattern of spatial merge candidates is shown in Figure 1. For the description of non-adjacent merge mode and non-sub-PU STMVP mode please refer to the CE4.4.6 JVET-L0399.

Figure 1: Spatial neighboring blocks used to derive the spatial merge candidates.

## Simplified pruning method

Pre-define the maximum number of pruning operations in both encoder side and decoder side without signaling. When the number of pruning has reached the maximum number, don’t perform pruning anymore. The rest of non-adjacent potential candidates will not be added in the list. The default combined candidates will be added. For example, in the simulations of this contribution, the pre-defined maximum number of pruning is 20.

# Simulation results

As descripted in CE4 [5], the test specification is as below, and the VTM2.0.1 with default encoder configuration are used as anchors.

## Test a: CE4.4.6.a + maximum number of pruning equals to 20

Simulation results as below:

Merge list equal to 10

Merge list equal to 8

Merge list equal to 6

## Test4.4.6.b + maximum number of pruning equals to 20

Simulation results as below:

Merge list equal to 10

Merge list equal to 8

Merge list equal to 6

## Test4.4.6.c + maximum number of pruning equals to 20

Simulation results as below:

Merge list equal to 10

Merge list equal to 8

Merge list equal to 6

## Test4.4.6.d + maximum number of pruning equals to 20

Simulation results as below:

Merge list equal to 10

Merge list equal to 8

Merge list equal to 6

# Conclusion

This contribution presents improvement on Merge/Skip prediction technologies which are studied in CE4. For the “VTM2.0.1 as reference” tests in RA case with the size of merge list equaling to 10, the sub test a) “CE4.4.6.a + maximum number of pruning equals to 20” achieved objective gain of 0.73% in terms of average luma BD-rate improvement; the sub test b) “CE4.4.6.b + maximum number of pruning equals to 20” achieved -0.62% coding gain; c) “CE4.4.6.c + maximum number of pruning equals to 20” achieved -1.05% coding gain; d) “CE4.4.6.d + maximum number of pruning equals to 20” achieved -1.01% coding gain.

# References

- JVET-J0021, “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions”, Macau, Apr. 2018.

- Y. Han, H. Huang, Y. Zhang, C.-H. Hung, C.-C. Chen W.-J. Chien, M. Karczewicz, JVET-J0339, “CE4.2.3: Improvement on Merge/Skip mode”, Ljubljana, July 2018.

- Y. Han, Y.-H. Chao, W.-J. Chien, M. Karczewicz, JVET-J0350, “CE4-related: Improvement on Merge/Skip mode with line buffer restriction”, Ljubljana, July 2018.

- Y. Han, W.-J. Chien, H. Huang, M. Karczewicz, JVET-L0399, “CE4.4.6: Improvement on Merge/Skip mode”, Macao, Oct. 2018.

- H. Yang, S. Liu, K. Zhang, JVET-K1024, “Description of CE4: Inter prediction and motion vector coding”, Ljubljana, July 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).