| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0407-v2 |
| --- | --- |

| Title: | CE6-related: Transform skip for 2x2 chroma blocks and disable 2x2 chroma blocks in intra slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van, Wei-Jung Chien, Vadim Seregin, Ted Hsieh, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Tel: Email: | +1-858-658-5664 lphamvan@qti.qualcomm.com |
| Source: | Qualcomm Technologies Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two modifications for 2x2 chroma blocks. The first modification is to disable 2x2 chroma blocks in intra slice. The second modification is to force 2x2 chroma blocks to be coded in transform skip mode. The first method reportedly leads to an average [Y, U, V] BD-rate change of (0.00%, 0.05%, 0.09%) for VTM AI configuration. It is also reported an average [Y, U, V] BD-rate change of (0.01%, 0.13%, 0.19%) and (-0.03%, -0.01%, -0.13%) for RA and LB configurations, respectively for the combination of the two proposed techniques.

# Introduction

The tree structure used in VVC is a generalization of the QT-BTT (Quad-tree plus Binary and Triple tree). The fundamental structure of this structure constitutes of two types of tree nodes: Region Tree (RT) and Prediction Tree (PT), supporting five types of partitions, as shown in Figure 1. A region tree can recursively split a CTU into square blocks down to a 4x4 size region tree leaf node. At each node in a region tree, a prediction tree can be formed from one of three tree types to form coding units (CUs): Binary Tree, Triple Tree. In a PT split, it is prohibited to have a quadtree partition in branches of the prediction tree.

A CTU consists of a luma coding tree block (CTB) and two chroma coding tree blocks. At the CU level, a CU is associated with a luma coding block (CB) and two chroma coding blocks. As in JEM (the reference software of the VVC), the luma tree and the chroma tree are separated in intra slices while they are shared in inter slices.  The size of a CTU can be 128x128 (luma component) while the size of a coding unit can be ranges from 4x4 to the size of CTU. In this scenario, the size of a chroma block can be as small as a 2x2 block.

The split of the chroma components into blocks of size 2x2 may result in a high number of small blocks. This increases the number of transform stages that require multiplication and addition.

Figure 1: (a) quad-tree partitioning (b) vertical binary-tree partitioning (c) horizontal binary-tree partitioning (d) vertical triple-tree partitioning (e) horizontal triple-tree partitioning.

# Proposed Method

  - Disable 2x2 chroma blocks for the slices with dual coding tree

If separate tree coding is applied, meaning that luma and chroma components may have different partition structure, the partition splitting to 2x2 chroma blocks is prohibited, in such case there will be no 2x2 chroma blocks. In addition, the split flag leading to 2x2 chroma blocks is no longer signalled in the bitstream.

  - Transform skip mode for 2x2 chroma blocks

2x2 chroma blocks are always to be coded in transform skip mode. It means that the residual of the 2x2 chroma blocks is be quantized directly in the pixel domain. Therefore, the transform skip flag is no longer signaled in the bit stream for those blocks. With the use of the proposed technique, the transform stage is removed from the pipeline and the transform skip is simply carried out by a bit shift operation.

# Experimental Results

The proposed techniques are implemented on the top of the reference software VTM-2.0.1 and are tested under the AI, RA and LB settings.

The performance of disabling 2x2 chroma blocks in intra slice is summarized in Table 1. It is reported an average [Y, U, V] BD-rate change of (0.00%, 0.05%, 0.09%) when 2x2 chroma blocks are disabled in intra slices.

The performance of the combined approach (including disabling 2x2 chroma blocks in intra slice and forcing 2x2 blocks in inter slices to be transform skip) is presented in Table 2 for VTM RA and LB conditions. As can be seen from the Table 2, the combination of two proposed approach leads to an average [Y, U, V] BD-rate change of (0.01%, 0.13%, 0.19%) and (-0.02%, 0.01%, -0.13%) compared with VTM-2.0.1 under RA and LB configurations, respectively.

Table 1. Disable 2x2 chroma in intra slice

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.03% | 99% | 100% |
| Class A2 | -0.01% | 0.02% | 0.01% | 99% | 100% |
| Class B | 0.01% | 0.01% | 0.06% | 100% | 100% |
| Class C | -0.02% | 0.13% | 0.36% | 99% | 102% |
| Class E | 0.01% | 0.05% | -0.06% | 100% | 102% |
| Overall | 0.00% | 0.05% | 0.09% | 100% | 101% |
| Class D | 0.00% | 0.21% | 0.23% | 99% | 99% |
| Class F (optional) | 0.01% | 0.26% | 0.45% | 99% | 102% |

Table 2. 2x2 chroma blocks are disabled in intra slices and 2x2 chroma blocks are forced to be transform skip in inter slices

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | 0.04% | 99% | 99% |
| Class A2 | -0.02% | 0.02% | -0.01% | 96% | 99% |
| Class B | 0.01% | 0.09% | 0.17% | 99% | 99% |
| Class C | 0.03% | 0.32% | 0.50% | 98% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.13% | 0.19% | 98% | 99% |
| Class D | 0.02% | 0.40% | 0.43% | 98% | 97% |
| Class F (optional) | 0.02% | 0.33% | 0.44% | 98% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.14% | 0.02% | 102% | 119% |
| Class C | 0.05% | 0.28% | 0.19% | 97% | 96% |
| Class E | -0.19% | -0.63% | -0.81% | 99% | 102% |
| Overall | -0.03% | -0.01% | -0.13% | 100% | 107% |
| Class D | 0.01% | 0.75% | 0.30% | 99% | 96% |
| Class F (optional) | -0.09% | 0.36% | 0.60% | 99% | 99% |

# Conclusions

This contribution proposes an approach to simplify 2x2 chroma blocks in video coding. The first technique is to disable 2x2 chroma block in intra slices when dual coding tree is enabled. The second method is to force 2x2 blocks to be transform skip encoded.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).