| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0414 |
| --- | --- |

| Title: | CE11: Luma-adaptive deblocking filter (CE11.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Atsuro Ichigaya Shunsuke Iwamura Shimpei Nemoto | Tel: Email: | +81-3-5494-3318 ichigaya.a-go@nhk.or.jp iwamura.s-gc@nhk.or.jp nemoto.s-fy@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution reports test results of CE11.2.2 on luma-adaptive deblocking filter (LADF). The filter strength of the proposed deblocking filter is coSntrolled based on the averaged luma level of the reconstructed samples. This contribution investigates the performances of LADF with two settings. The first setting is the same as original proposed setting which is applied modified filtering to brighter areas only. The second setting is the alternative setting which is applied to brighter areas and darker areas both.

The additional complexity of this method is slightly small. The results of the first setting without ALF shows it does not achieves objective gain of luma for AI and RA. In the case of LD-B and LD-P, it achieves 0.03% loss for LD-B and 0.02% gain of luma for LD-P. The results of the second setting without ALF shows 0.06%, 0.02%, and 0.08% BD-Rate loss for AI, RA, and LDB and it shows 0.04% gain for LD-P.

Proposed method

The proposed deblocking filter applies variable filter strength depending on the level of reconstructed samples before in-loop filters. The distortion of decoded pictures is caused by a quantization process. The distortion is displayed through nonlinear process called Electro-Optical Transfer Function (EOTF) in the linear light domain. Additionally, the scene referred video system such as conventional SDR format and one of the HDR formats, Hybrid Log-Gamma (HLG), requires an additional nonlinear process called Opto-Optical Transfer Function (OOTF) also known as a system gamma. The displayed subjective distortion is enhanced by the nonlinear transfer function depending on the video signal level. Such mechanism makes block distortion visible at brighter areas, even if the quantization noise is uniformly distributed at all video level. Especially, for HDR formats, this enhanced distortion becomes salient due to its strong nonlinearity.

In HEVC, the filter strength of the deblocking filter is controlled by the variables β and tC which are derived from the averaged quantization parameters qPL. The proposed deblocking filter controls the strength of the deblocking filter by adding offset to qPL according to the luma level of the reconstructed samples.

The reconstructed luma level LL is derived as follow:

LL= ( ( p0,0 + p0,3 + q0,0 + q0,3 ) >> 2 ) / ( 1 << iBitDepth ),

where, the sample values pi,k and qi,k with i = 0..3 and k = 0 and 3 are derived as shown in Fig.1.

Fig.1 Sample position of pi,k and qi,k

The variable qPL is derived as follows:

qPL = ( ( QpQ + QpP +1 ) >> 1 ) + qpOffset,

where QpQ and QpP denote the quantization parameters of the coding units containing the sample q0,0 and p0,0, respectively. The offset qpOffset should be derived according to the transfer characteristics of the contents since the EOTF and OOTF vary among video formats. VTM and BMS software currently are not enabled the parameter of transfer_characteristics indicating the transfer characteristics of the contents, an additional flag “transfer_characteristics” is signaled in the SPSNext class.

Test 1

The condition of qpOffset is defined for each transfer characteristics as shown in Table 1. This test is a condition to investigate the performance of the method applied to brighter areas only.

Table 1 Specification of qpOffset as a function of LL

| transfer_characteristics == 1 (conventional SDR format) |  | transfer_characteristics == 16 (PQ format) |  | transfer_characteristics == 18 (HLG format) |  |
| --- | --- | --- | --- | --- | --- |
| Conditions | qpOffset | Conditions | qpOffset | Conditions | qpOffset |
| LL <= 0.875 | 0 | LL <= 0.84 | 0 | LL <= 0.8 | 0 |
| 0.875 < LL | 1 | 0.84 < LL <= 0.91 | 1 | 0.8 < LL <= 0.9 | 1 |
|  |  | 0.91 < LL <= 0.99 | 2 | 0.9 < LL | 2 |
|  |  | 0.99 < LL | 3 |  |  |

Test 2

The condition of qpOffset is defined for each transfer characteristics as shown in Table 2. This test is a condition to investigate the performance of the method applied to brighter areas and darker areas.

Table 2 Specification of qpOffset as a function of LL

| transfer_characteristics == 1 (conventional SDR format) |  | transfer_characteristics == 16 (PQ format) |  | transfer_characteristics == 18 (HLG format) |  |
| --- | --- | --- | --- | --- | --- |
| Conditions | qpOffset | Conditions | qpOffset | Conditions | qpOffset |
| LL≤0.326 | 1 | LL≤0.608 | 2 | LL≤0.504 | 2 |
| 0.326<LL≤0.878 | 0 | 0.608<LL≤0.685 | 1 | 0.504<LL≤0.612 | 1 |
| 0.878 < LL | 1 | 0.685<LL≤0.837 | 0 | 0.612<LL≤0.822 | 0 |
|  |  | 0.837<LL≤0.910 | 1 | 0.822<LL≤0.926 | 1 |
|  |  | 0.910< LL | 2 | 0.926 < LL | 2 |

Experimental results

The proposed method is implemented on VTM-2.0.1. The test is under CTC and the results are summarized in Table 3-7. More detailed results can be found in the accompanied Excel tables.

Table 3 Comparison of Test 1 with VTM-2.0.1 (w/o ALF)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 103% |

Table 4 Comparison of Test 1 with VTM-2.0.1 (w/o ALF)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.25% | 0.01% | 99% | 98% |
| Class A2 | -0.02% | -0.02% | -0.03% | 99% | 99% |
| Class B | 0.00% | 0.03% | -0.03% | 100% | 100% |
| Class C | -0.01% | -0.03% | -0.02% | 100% | 100% |
| Overall | 0.00% | -0.05% | -0.02% | 100% | 99% |
| Class D | 0.00% | -0.03% | 0.00% | 100% | 102% |

Table 5 Comparison of Test1 with VTM-2.0.1 (w/o ALF)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class B | 0.01% | 0.09% | -0.21% | 98% | 101% |
| Class C | 0.01% | -0.19% | -0.06% | 100% | 102% |
| Class E | 0.10% | -0.22% | -0.08% | 100% | 102% |
| Overall | 0.03% | -0.08% | -0.13% | 99% | 101% |
| Class D | 0.03% | -0.33% | -0.05% | 100% | 97% |

Table 6 Comparison of Test 2 with VTM-2.0.1 (w/o ALF)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.05% | 0.00% | 0.00% | 100% | 98% |
| Class B | 0.08% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.04% | 0.00% | 0.00% | 100% | 102% |
| Class E | 0.07% | -0.01% | 0.00% | 100% | 98% |
| Overall | 0.06% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.04% | 0.00% | 0.00% | 99% | 102% |

Table 7 Comparison of Test 2 with VTM-2.0.1 (w/o ALF)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.03% | -0.01% | 100% | 100% |
| Class A2 | 0.03% | -0.03% | -0.04% | 100% | 100% |
| Class B | 0.04% | -0.03% | 0.03% | 100% | 100% |
| Class C | 0.00% | -0.01% | -0.07% | 100% | 99% |
| Overall | 0.02% | -0.03% | -0.02% | 100% | 100% |
| Class D | 0.01% | -0.17% | -0.07% | 99% | 100% |

Table 8 Comparison of Test 2 with VTM-2.0.1 (w/o ALF)

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class B | 0.07% | 0.11% | -0.22% | 100% | 102% |
| Class C | 0.02% | -0.09% | -0.19% | 100% | 102% |
| Class E | 0.18% | -0.06% | 0.70% | 99% | 102% |
| Overall | 0.08% | 0.00% | 0.02% | 100% | 102% |
| Class D | 0.00% | -0.16% | -0.05% | 100% | 103% |

Conclusions

This contribution proposes luma-adaptive deblocking filter. The filter strength of the proposed deblocking filter is controlled based on the averaged luma level of the reconstructed samples. The proposed method is applied to the brighter area (Test 1) and the brighter and darker area. The result of test1, the modification has small impact on the results because such brighter area is very small. We proposed this mechanism as unified deblocking filter strategy of new video standard.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).