| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0414 |
| --- | --- |

| Title: | Syntax and Semantics changes of Luma adaptive deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Atsuro Ichigaya Shunsuke Iwamura Shimpei Nemoto | Tel: Email: | +81-3-5494-3318 ichigaya.a-go@nhk.or.jp iwamura.s-gc@nhk.or.jp nemoto.s-fy@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Syntax and semantics changes regarding JVET-L0414

The syntax and semantics changes added regarding luma adaptive deblocking filter are marked in red.

…

    - Raw byte sequence payloads, trailing bits and byte alignment syntax

      - Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| qtbtt_dual_tree_intra_flag | ue(v) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_qt_size_intra_slices_minus2 | ue(v) |
| log2_min_qt_size_inter_slices_minus2 | ue(v) |
| max_mtt_hierarchy_depth_inter_slices | ue(v) |
| max_mtt_hierarchy_depth_intra_slices | ue(v) |
| sps_cclm_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| if( sps_sbtmvp_enabled_flag ) |  |
| log2_sbtmvp_default_size_minus2 | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) |  |
| sps_affine_type_flag | u(1) |
| sps_mts_intra_enabled_flag | u(1) |
| sps_mts_inter_enabled_flag | u(1) |
| sps_ladf_enabled_flag | u(1) |
| if( sps_ladf_enabled_flag ) { |  |
| sps_num_ladf_intervals_minus2 | u(2) |
| sps_ladf_qp_offset[ 0 ] |  |
| for( i=0; i < sps_num_ladf_intervals_minus2 + 1; i++ ) |  |
| sps_ladf_delta_threshold_minus1[ i ] | ue(v) |
| sps_ladf_qp_offset[ i + 1 ] | ue(v) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

…

    - Raw byte sequence payloads, trailing bits and byte alignment semantics

      - Sequence parameter set RBSP semantics

…

sps_ladf_enalbed_flag equal to 1, specifies that sps_num_ladf_intervals_minus2 and sps_ladf_qp_offset[ 0 ] are present in the SPS.

sps_num_ladf_intervals_minus2 plus 1 specifies the number of sps_ladf_delta_threshold_minus1[ i ] and the number of sps_ladf_qp_offset[ i ] minus 1 syntax elements that are present in the SPS. The value of sps_num_ladf_intervals_minus2 shall be in the range of 0 to 3, inclusive.

sps_ladf_delta_threshold_minus1[ i ] is used to compute the values of SpsLadfIntervalLowerBound[ i ], which specifies the lower bound of the interval i of luma intensity level. The value of sps_ladf_delta_threshold_minus1[ i ] shall be in the range of 0 to 2BitDepthY − 3, inclusive.

The value of SpsLadfIntervalLowerBound[ 0 ] is set equal to 0.

For each value of i in the range of 1 to sps_num_ladf_intervals_minus2 plus 1, inclusive, the variable SpsLadfIntervalLowerBound[ i ] is derived as follows:

SpsLadfIntervalLowerBound[ i ] = SpsLadfIntervalLowerBound[ i - 1 ] 
						+ sps_ladf_delta_threshold_minus1[ i - 1 ] + 1		(8-TBD)

sps_ladf_qp_offset[ i ] specifies the offset used to derive the variable qPL as specified in clause TBD.The value of sps_ladf_qp_offset[ i ] shall be in the range of 0 to [TBD: max QP value minus 1].

…

In-loop Filter Process

    - General

[Ed. (SL): Place holder, assuming we will have more than one in-loop filter and one of them is deblocking filter.]

The two in-loop filters, namely deblocking filter and adaptive loop filter, are applied as specified by the following ordered steps:

- For the deblocking filter, the following applies:

–	The deblocking filter process as specified in clause 8.5.2.1 is invoked with the reconstructed picture sample arrays SL, SCb and SCr as inputs, and the modified reconstructed picture sample arrays S′L, S′Cb and S′Cr after deblocking as outputs.

–	The array S′L, S′Cb and S′Cr are assigned to the arrays SL, SCb and SCr (which represent the decoded picture), respectively.

- When sps_alf_enabled_flag is equal to 1, the following applies:

–	The adaptive loop filter process as specified in clause 8.5.3.1 is invoked with the reconstructed picture sample arrays SL, SCb and SCr as inputs, and the modified reconstructed picture sample arrays S′L, S′Cb and S′Cr after sample adaptive offset as outputs.

–	The arrays S′L, S′Cb and S′Cr are assigned to the arrays SL, SCb and SCr (which represent the decoded picture), respectively.

…

Deblocking filter process

General

[Ed. (SL): Place holder, assuming we will have a deblocking filter.]

Decision process for luma block edges (grey text copied from HEVC spec text)

Inputs to this process are:

–	a luma picture sample array recPictureL,

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a luma location ( xBl, yBl ) specifying the top-left sample of the current luma block relative to the top-left sample of the current luma coding block,

–	a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

–	a variable bS specifying the boundary filtering strength.

Outputs of this process are:

–	the variables dE, dEp and dEq containing decisions,

–	the variables β and tC.

If edgeType is equal to EDGE_VER, the sample values pi,k and qi,k with i = 0..3 and k = 0 and 3 are derived as follows:

qi,k = recPictureL[ xCb + xBl + i ][ yCb + yBl + k ]	(8-345)

pi,k = recPictureL[ xCb + xBl − i − 1 ][ yCb + yBl + k ]	(8-346)

Otherwise (edgeType is equal to EDGE_HOR), the sample values pi,k and qi,k with i = 0..3 and k = 0 and 3 are derived as follows:

qi,k = recPictureL[ xCb + xBl + k ][ yCb + yBl + i ]	(8-347)

pi,k = recPictureL[ xCb + xBl + k ][ yCb + yBl − i − 1 ]	(8-348)

The variable LL of the reconstructed luma level is derived as follow:

LL = ( ( p0,0 + p0,3 + q0,0 + q0,3 ) >> 2 ) / ( 1 << BitDepthY ),				(8-TBD)

The variable qpOffset is derived as follows:

qpOffset = sps_ladf_qp_offset[ 0 ]
for( i = 1; i < sps_num_ladf_intervals_minus2 + 2; i++ ){
	if( LL > 	SpsLadfIntervalLowerBound[ i ] )	
		qpOffset = sps_ladf_qp_offset[ i ]
	else
		break					(8-TBD)

The variables QpQ and QpP are set equal to the QpY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

The variable qPL is derived as follows:

qPL = ( ( QpQ + QpP + 1 )  >>  1 ) + qpOffset	(8-349)

The value of the variable β′ is determined as specified in Table 8-12 based on the luma quantization parameter Q derived as follows:

Q = Clip3( 0, 51, qPL + ( slice_beta_offset_div2  <<  1 ) )	(8-350)

where slice_beta_offset_div2 is the value of the syntax element slice_beta_offset_div2 for the slice that contains sample q0,0.

The variable β is derived as follows:

β = β′ * ( 1  <<  ( BitDepthY − 8 ) )	(8-351)

The value of the variable tC′ is determined as specified in Table 8-12 based on the luma quantization parameter Q derived as follows:

Q = Clip3( 0, 53, qPL + 2 * ( bS − 1 ) + ( slice_tc_offset_div2  <<  1 ) )	(8-352)

where slice_tc_offset_div2 is the value of the syntax element slice_tc_offset_div2 for the slice that contains sample q0,0.

The variable tC is derived as follows:

tC = tC′ * ( 1  <<  ( BitDepthY − 8 ) )	(8-353)

Depending on the value of edgeType, the following applies:

–	If edgeType is equal to EDGE_VER, the following ordered steps apply:

- The variables dpq0, dpq3, dp, dq and d are derived as follows:

dp0 = Abs( p2,0 − 2 * p1,0 + p0,0 )		(8-354)

dp3 = Abs( p2,3 − 2 * p1,3 + p0,3 )		(8-355)

dq0 = Abs( q2,0 − 2 * q1,0 + q0,0 )		(8-356)

dq3 = Abs( q2,3 − 2 * q1,3 + q0,3 )		(8-357)

dpq0 = dp0 + dq0		(8-358)

dpq3 = dp3 + dq3		(8-359)

dp = dp0 + dp3		(8-360)

dq = dq0 + dq3		(8-361)

d = dpq0 + dpq3		(8-362)

- The variables dE, dEp and dEq are set equal to 0.

- When d is less than β, the following ordered steps apply:

- The variable dpq is set equal to 2 * dpq0.

- For the sample location ( xCb + xBl, yCb + yBl ), the decision process for a luma sample as specified in clause 8.7.2.5.6 is invoked with sample values p0,0, p3,0, q0,0, and q3,0, the variables dpq, β and tC as inputs, and the output is assigned to the decision dSam0.

- The variable dpq is set equal to 2 * dpq3.

- For the sample location ( xCb + xBl, yCb + yBl + 3 ), the decision process for a luma sample as specified in clause 8.7.2.5.6 is invoked with sample values p0,3, p3,3, q0,3, and q3,3, the variables dpq, β and tC as inputs, and the output is assigned to the decision dSam3.

- The variable dE is set equal to 1.

- When dSam0 is equal to 1 and dSam3 is equal to 1, the variable dE is set equal to 2.

- When dp is less than ( β + ( β  >>  1 ) )  >>  3, the variable dEp is set equal to 1.

- When dq is less than ( β + ( β  >>  1 ) )  >>  3, the variable dEq is set equal to 1.

–	Otherwise (edgeType is equal to EDGE_HOR), the following ordered steps apply:

- The variables dpq0, dpq3, dp, dq and d are derived as follows:

dp0 = Abs( p2,0 − 2 * p1,0 + p0,0 )		(8-363)

dp3 = Abs( p2,3 − 2 * p1,3 + p0,3 )		(8-364)

dq0 = Abs( q2,0 − 2 * q1,0 + q0,0 )		(8-365)

dq3 = Abs( q2,3 − 2 * q1,3 + q0,3 )		(8-366)

dpq0 = dp0 + dq0		(8-367)

dpq3 = dp3 + dq3		(8-368)

dp = dp0 + dp3		(8-369)

dq = dq0 + dq3		(8-370)

d = dpq0 + dpq3		(8-371)

- The variables dE, dEp and dEq are set equal to 0.

- When d is less than β, the following ordered steps apply:

- The variable dpq is set equal to 2 * dpq0.

- For the sample location ( xCb + xBl, yCb + yBl ), the decision process for a luma sample as specified in clause 8.7.2.5.6 is invoked with sample values p0,0, p3,0, q0,0 and q3,0, the variables dpq, β and tC as inputs, and the output is assigned to the decision dSam0.

- The variable dpq is set equal to 2 * dpq3.

- For the sample location ( xCb + xBl + 3, yCb + yBl ), the decision process for a luma sample as specified in clause 8.7.2.5.6 is invoked with sample values p0,3, p3,3, q0,3 and q3,3, the variables dpq, β and tC as inputs, and the output is assigned to the decision dSam3.

- The variable dE is set equal to 1.

- When dSam0 is equal to 1 and dSam3 is equal to 1, the variable dE is set equal to 2.

- When dp is less than ( β + ( β  >>  1 ) )  >>  3, the variable dEp is set equal to 1.

- When dq is less than ( β + ( β  >>  1 ) )  >>  3, the variable dEq is set equal to 1.

Table 8-12 – Derivation of threshold variables β′ and tC′ from input Q

| Q | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| β′ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 6 | 7 | 8 |
| tC′ | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| Q | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |
| β′ | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 | 36 |
| tC′ | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| Q | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |  |  |  |
| β′ | 38 | 40 | 42 | 44 | 46 | 48 | 50 | 52 | 54 | 56 | 58 | 60 | 62 | 64 | - | - |  |  |  |
| tC′ | 5 | 5 | 6 | 6 | 7 | 8 | 9 | 10 | 11 | 13 | 14 | 16 | 18 | 20 | 22 | 24 |  |  |  |

- Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).