| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0431 |
| --- | --- |

| Title: | CE3: Multiple Reference Line Intra Prediction (Tests 1.2.1 and 1.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Core Experiment |  |  |
| Author(s) or Contact(s): | Steve Keating | Email: | steve.keating@sony.com |
| Source: | Sony Europe |  |  |

_____________________________

# Abstract

Results are reported for JVET-K1023 Core Experiments CE3-1.2.1 and 1.2.2 regarding the use of multiple (in this case, 2) lines of reference samples for intra prediction.  Simulation results show an average luma BD-rate variation of -0.29% and -0.25% respectively in All Intra (AI) and -0.16% and 
-0.14% respectively in Random Access configuration, compared with VTM.

# Introduction

The use of two or more rows (or columns) of samples for angular intra prediction can provide a better prediction in most cases, one downside of this is the requirement for an additional line store for the extra row of samples.  These two Core Experiments test the effect of disabling the use of the second row of samples at the top of the CTU to avoid the need for this additional line store.  Within the CTU the extra row of samples is used, with the restriction that the block being predicted must be 8x8 or greater in size, and a 50/50 weighting of the two prediction values is used.  Test 1.2.1 removes the restriction at the top of the CTU whereas test 1.2.2 includes this restriction.

# Results

The tests were implemented in the BMS 2.0.1 software run using VTM configuration files, as described in the Common Test Conditions.

## CE3-1.2.1

This experiment was designed to test the effect of disabling the use of multiple reference lines for the first line in a CTU in order to remove the requirement to add extra line store(s). The results with enabling for the first line are:

### VTM

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (50:50 except small blocks) including 1st line of CTU (CE3-1.2.1) Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.16% | -0.18% | 109% | 103% |
| Class A2 | -0.27% | -0.13% | -0.19% | 110% | 105% |
| Class B | -0.32% | -0.20% | -0.13% | 106% | 103% |
| Class C | -0.32% | -0.35% | -0.16% | 107% | 105% |
| Class E | -0.26% | -0.36% | -0.33% | 107% | 104% |
| Overall | -0.29% | -0.24% | -0.19% | 107% | 104% |
| Class D | -0.15% | 0.12% | -0.13% | 104% | 102% |
| Class F | -0.18% | -0.22% | -0.10% | 106% | 104% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (50:50 except small blocks) including 1st line of CTU (CE3-1.2.1) Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.21% | -0.22% | 102% | 101% |
| Class A2 | -0.16% | -0.22% | -0.23% | 102% | 101% |
| Class B | -0.16% | -0.18% | 0.07% | 102% | 101% |
| Class C | -0.11% | -0.01% | -0.03% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.15% | -0.07% | 102% | 101% |
| Class D | -0.10% | -0.32% | -0.06% | 102% | 100% |
| Class F | -0.07% | -0.02% | -0.09% | 103% | 102% |

## CE3-1.2.2

This experiment was designed to test the effect of disabling the use of multiple reference lines for the first line in a CTU in order to remove the requirement to add extra line store(s). The results with disabling for the first line are:

### VTM

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (50:50 except small blocks) disabling 1st line of CTU (CE3-1.2.2) Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.04% | -0.12% | 108% | 102% |
| Class A2 | -0.24% | -0.16% | -0.16% | 112% | 107% |
| Class B | -0.29% | -0.18% | -0.12% | 105% | 102% |
| Class C | -0.28% | -0.29% | -0.23% | 102% | 101% |
| Class E | -0.20% | -0.38% | -0.26% | 104% | 101% |
| Overall | -0.25% | -0.21% | -0.18% | 106% | 103% |
| Class D | -0.15% | 0.10% | -0.20% | 103% | 101% |
| Class F | -0.16% | -0.20% | -0.06% | 105% | 103% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (50:50 except small blocks) disabling 1st line of CTU (CE3-1.2.2) Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.08% | -0.09% | 103% | 101% |
| Class A2 | -0.14% | -0.13% | -0.20% | 101% | 100% |
| Class B | -0.13% | -0.14% | 0.12% | 103% | 101% |
| Class C | -0.12% | -0.01% | 0.02% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.09% | -0.01% | 103% | 101% |
| Class D | -0.05% | -0.12% | -0.11% | 101% | 99% |
| Class F | -0.08% | -0.07% | -0.17% | 103% | 101% |

# Comment on results

It can be seen that disabling the use of the second row of reference samples for the first line in a CTU does reduce the BD-rate improvement very slightly (-0.25% vs. -0.29% for Intra VTM).

# Additional Results

## Enabling MRL for pure H and V

Tests 1.2.1 and 1.2.2 were both defined as using multiple (2) reference lines for all angular modes except pure horizontal or vertical.  In order to see what effect including pure H and V directions in MRL prediction gives an additional test was run in All Intra mode, the results are shown below:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 2-line intra pred. (50:50 except small blocks) Including pure H and V disabling 1st line of CTU (CE3-1.2.2 + HV) Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.18% | -0.12% | 106% | 100% |
| Class A2 | -0.23% | -0.16% | -0.19% | 104% | 100% |
| Class B | -0.29% | -0.17% | -0.15% | 102% | 99% |
| Class C | -0.28% | -0.30% | -0.14% | 103% | 101% |
| Class E | -0.21% | -0.24% | -0.14% | 104% | 101% |
| Overall | -0.25% | -0.21% | -0.15% | 104% | 100% |
| Class D | -0.15% | 0.15% | -0.15% | 102% | 100% |
| Class F | -0.16% | -0.19% | -0.15% | 102% | 101% |

As can be seen, allowing MRL prediction for pure horizontal and vertical directions gives no overall benefit.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).