| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0448-v2 |
| --- | --- |

| Title: | CE4-related: Constraint of Pruning in History-based Motion Vector Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weiwei Xu Haitao Yang Yin Zhao Jianle Chen | Tel: Email: | +8613758193753 xuweiwei3@huawei.com haitao.yang@huawei.com yin.zhao@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of constraint of pruning in history-based motion vector prediction. In this test, the inter prediction mode of history-based motion vector candidates and merge candidates are taken into consideration in the pruning between history-based motion vector candidates and merge candidates, when adding the history-based motion vector candidates into merge candidate list. Compared to CE4.4.7 anchor of history-based method, the average BD-rate is reportedly 0.03%~0.07% in different configurations, respectively.

Introduction

In history-based MVP (HMVP) method, a HMVP candidate is defined as the motion information of a previously coded block. A table with multiple HMVP candidates is maintained during the encoding/decoding process. HMVP candidates could be used in the merge candidate list construction process.

Figure 1. Adding HMVP into Merge candidate list

When HMVP candidates are added into merge candidate list, pruning must be done for each candidate with each merge candidate that are already in the merge candidate list as shown in figure1. For HPMVm, it should be compared to merge cand0, merge cand1, and merge candn. If HPMV candidate is equal to any merge candidates that already in the merge candidate list, it will not be added into merge candidate list like HPMVl-2 in figure 2.

For a merge candidate list of length n and a HMVP list of length l, the complexity is n plus l. The complexity is high.

Due to the feature of different inter prediction mode, the MV of merge mode block is equal to coded block in some neighboring position, the mv of inter mode block is different from all neighboring coded block. So when the history-based motion vector candidates or/and merge candidates are coded in inter AMVP mode in original CU, pruning can be saved for these candidates.

In this contribution, four cases are tested:

1, (test1) when a HMVP candidate is coded in inter mode, the pruning between it and all the merge candidates are jumped, and this HMVP is added into merge candidate list. Then the next HMVP candidate will be checked with merge candidates.

2, (test2) when a merge candidate is coded in inter mode, the pruning between it and every HMVP candidate are jumped.

3, (test3) when a HMVP candidate or a HMVP candidate is coded in inter mode, the pruning between it and the candidate in the other list will not be performed.

4, (test4) when a HMVP candidate is coded in inter mode, the pruning between it and all the merge candidates are jumped, and this HMVP candidate is not added into merge candidate list. Then the next HMVP candidate will be checked with merge candidates.

Simulation results

The contribution has been implemented on top of CE4.4.7. The test condition descripted in JVET-K1010 [1] is used for verifying the contribution.

Test1: The summary results when history-based candidate is inter then no pruning, are provided in Table 1.

Table 1 Coding performance over CE4.4.7

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.03% | 0.13% | 100% | 99% |
| Class A2 | 0.08% | 0.01% | 0.07% | 101% | 100% |
| Class B | 0.05% | 0.10% | 0.05% | 100% | 99% |
| Class C | 0.07% | -0.05% | 0.06% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.03% | 0.07% | 100% | 99% |
| Class D | 0.01% | -0.02% | 0.01% | 101% | 99% |
| Class F (optional) | 0.03% | 0.00% | 0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | 0.24% | -0.10% | 100% | 99% |
| Class C | 0.00% | 0.01% | -0.08% | 101% | 99% |
| Class E | 0.04% | 0.38% | 0.24% | 101% | 101% |
| Overall | 0.04% | 0.20% | -0.01% | 101% | 100% |
| Class D | 0.04% | -0.25% | -0.94% | 101% | 100% |
| Class F (optional) | 0.03% | 0.11% | 0.24% | 100% | 101% |

Test2: The summary results when merge candidate is inter then no pruning, are provided in Table 2. On average, the BD-rate differences are 0.07% in RA, respectively.

Table 2 Coding performance over CE4.4.7

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.05% | 0.06% | 100% | 99% |
| Class A2 | 0.09% | 0.12% | 0.05% | 100% | 100% |
| Class B | 0.08% | 0.13% | -0.04% | 100% | 99% |
| Class C | 0.04% | 0.01% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.06% | 0.01% | 100% | 99% |
| Class D | 0.04% | -0.04% | 0.04% | 100% | 99% |
| Class F (optional) | 0.01% | 0.04% | 0.03% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | 0.10% | -0.12% | 101% | 100% |
| Class C | 0.03% | -0.01% | -0.11% | 101% | 100% |
| Class E | 0.14% | 0.35% | 0.16% | 101% | 101% |
| Overall | 0.08% | 0.13% | -0.05% | 101% | 100% |
| Class D | 0.04% | -0.24% | -0.68% | 102% | 103% |
| Class F (optional) | 0.26% | 0.85% | -0.22% | 101% | 100% |

Test3: The summary results when merge candidate or history-based candidate is inter then no pruning, are provided in Table 3. On average, the BD-rate differences are 0.07% in RA, respectively.

Table 3 Coding performance over CE4.4.7

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.05% | 0.07% | 100% | 100% |
| Class A2 | 0.08% | 0.13% | 0.05% | 101% | 100% |
| Class B | 0.07% | 0.14% | 0.03% | 101% | 99% |
| Class C | 0.05% | 0.00% | -0.03% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.06% | 0.03% | 101% | 100% |
| Class D | 0.03% | -0.04% | 0.05% | 101% | 99% |
| Class F (optional) | 0.02% | 0.04% | 0.02% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | -0.01% | -0.06% | 101% | 101% |
| Class C | 0.04% | 0.00% | -0.11% | 102% | 100% |
| Class E | 0.15% | 0.38% | -0.10% | 101% | 101% |
| Overall | 0.08% | 0.09% | -0.09% | 101% | 100% |
| Class D | 0.04% | -0.68% | -0.31% | 102% | 104% |
| Class F (optional) | 0.21% | 0.43% | -0.16% | 101% | 101% |

Test4: The summary results when history-based candidate is inter no pruning and not added in merge candidate list, are provided in Table 4. On average, the BD-rate differences are 0.03% in RA, respectively.

Table 4 Coding performance over CE4.4.7

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.04% | 0.06% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.04% | 100% | 100% |
| Class B | 0.01% | 0.00% | 0.06% | 100% | 99% |
| Class C | 0.04% | -0.13% | 0.17% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.04% | 0.09% | 100% | 99% |
| Class D | 0.05% | -0.10% | -0.04% | 100% | 98% |
| Class F (optional) | -0.03% | -0.02% | 0.00% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.21% | 0.00% | 101% | 100% |
| Class C | 0.01% | -0.13% | -0.37% | 101% | 100% |
| Class E | 0.11% | 0.57% | 0.08% | 101% | 101% |
| Overall | 0.04% | 0.19% | -0.10% | 101% | 100% |
| Class D | 0.06% | -0.29% | -0.72% | 101% | 101% |
| Class F (optional) | 0.14% | 0.17% | -0.16% | 101% | 101% |

Conclusions

This contribution reports the results of constraint of pruning in history-based motion vector prediction. Compared to history based MVP, the average BD-rate differences are about 0.03%~0.07% in RA, respectively.

We recommend to study the proposed method in CE.

References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-K1010, July 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).