| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K1001-v7 |
| --- | --- |

| Title: | Versatile Video Coding (Draft 2) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text of video coding specification |  |  |
| Author(s) or Contact(s): | Benjamin Bross Jianle Chen Shan Liu | Email: | firstname.lastname@hhi.fraunhofer.de jianle.chen@huawei.com shanl@tencent.com |
| Source: | Editors |  |  |

_____________________________

This document provides the proposed text of VVC Working Draft. The changes are highlighted in green and strikethrough. The changes shown are for the method 3 of the proposal.

ITU-T Rec. H.VVC

Versatile video coding

Syntax and semantics

Syntax in tabular form

Raw byte sequence payloads, trailing bits and byte alignment syntax

Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| qtbtt_dual_tree_intra_flag | ue(v) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_qt_size_intra_slices_minus2 | ue(v) |
| log2_min_qt_size_inter_slices_minus2 | ue(v) |
| max_mtt_hierarchy_depth_inter_slices | ue(v) |
| max_mtt_hierarchy_depth_intra_slices | ue(v) |
| sps_cclm_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) |  |
| sps_affine_type_flag | u(1) |
| sps_mts_intra_enabled_flag | u(1) |
| sps_mts_inter_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Slice header syntax

General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if ( slice_type != I ) { |  |
| log2_diff_ctu_max_bt_size | ue(v) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| } |  |
| if ( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) |  |
| alf_data( ) |  |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| byte_alignment( ) |  |
| } |  |

Adaptive loop filter data syntax

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_chroma_idc | tu(v) |
| alf_luma_num_filters_signalled_minus1 | tb(v) |
| alf_luma_type_flag | u(1) |
| if( alf_luma_num_filters_signalled_minus1 > 0 ) { |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | tb(v) |
| } |  |
| alf_luma_coeff_delta_flag | u(1) |
| if ( !alf_luma_coeff_delta_flag && alf_luma_num_filters_signalled_minus1 > 0 ) |  |
| alf_luma_coeff_delta_prediction_flag | u(1) |
| alf_luma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < ( alf_luma_type_flag = = 1 ) ? 2 : 3; i++ ) |  |
| alf_luma_eg_order_increase_flag[ i ] | u(1) |
| if ( alf_luma_coeff_delta_flag ) { |  |
| for( sigFiltIdx = 0; sigFiltIdx <= alf_luma_num_filters_signalled_minus1; sigFiltIdx++ ) |  |
| alf_luma_coeff_flag[ sigFiltIdx ] | u(1) |
| } |  |
| for( sigFiltIdx = 0; sigFiltIdx <= alf_luma_num_filters_signalled_minus1; sigFiltIdx++ ) { |  |
| if ( alf_luma_coeff_flag[ sigFiltIdx ] ) { |  |
| for ( j = 0; j < ( alf_luma_type_flag = = 1 ) ? 6 : 12; j++ ) { |  |
| alf_luma_coeff_delta_abs[ sigFiltIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_delta_abs[ sigFiltIdx ][ j ] ) |  |
| alf_luma_coeff_delta_sign[ sigFiltIdx ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| if ( alf_chroma_idc > 0 ) { |  |
| alf_chroma_min_eg_order_minus1 | ue(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_eg_order_increase_flag[ i ] | u(1) |
| alf_chroma_ctb_present_flag | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ j ] | uek(v) |
| if( alf_chroma_coeff_abs[ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ j ] | u(1) |
| } |  |
| } |  |
| } |  |

Semantics

Raw byte sequence payloads, trailing bits and byte alignment semantics

Sequence parameter set RBSP semantics

sps_seq_parameter_set_id provides an identifier for the SPS for reference by other syntax elements. The value of sps_seq_parameter_set_id shall be in the range of 0 to 15, inclusive.

chroma_format_idc specifies the chroma sampling relative to the luma sampling as specified in clause 6.2. The value of chroma_format_idc shall be in the range of 0 to 3, inclusive.

separate_colour_plane_flag equal to 1 specifies that the three colour components of the 4:4:4 chroma format are coded separately. separate_colour_plane_flag equal to 0 specifies that the colour components are not coded separately. When separate_colour_plane_flag is not present, it is inferred to be equal to 0. When separate_colour_plane_flag is equal to 1, the coded picture consists of three separate components, each of which consists of coded samples of one colour plane (Y, Cb, or Cr) and uses the monochrome coding syntax. In this case, each colour plane is associated with a specific colour_plane_id value.

NOTE 1 – There is no dependency in decoding processes between the colour planes having different colour_plane_id values. For example, the decoding process of a monochrome picture with one value of colour_plane_id does not use any data from monochrome pictures having different values of colour_plane_id for inter prediction.

Depending on the value of separate_colour_plane_flag, the value of the variable ChromaArrayType is assigned as follows:

–	If separate_colour_plane_flag is equal to 0, ChromaArrayType is set equal to chroma_format_idc.

–	Otherwise (separate_colour_plane_flag is equal to 1), ChromaArrayType is set equal to 0.

pic_width_in_luma_samples specifies the width of each decoded picture in units of luma samples. pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

pic_height_in_luma_samples specifies the height of each decoded picture in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

bit_depth_luma_minus8 specifies the bit depth of the samples of the luma array BitDepthY and the value of the luma quantization parameter range offset QpBdOffsetY as follows:

BitDepthY      = 8 + bit_depth_luma_minus8		(7-2)

QpBdOffsetY = 6 * bit_depth_luma_minus8		(7-3)

bit_depth_luma_minus8 shall be in the range of 0 to 8, inclusive.

bit_depth_chroma_minus8 specifies the bit depth of the samples of the chroma arrays BitDepthC and the value of the chroma quantization parameter range offset QpBdOffsetC as follows:

BitDepthC      = 8 + bit_depth_chroma_minus8		(7-4)

QpBdOffsetC = 6 * bit_depth_chroma_minus8		(7-5)

bit_depth_chroma_minus8 shall be in the range of 0 to 8, inclusive.

qtbtt_dual_tree_intra_flag equal to 1 specifies that for I slices, each CTU is split into coding units with 64x64 luma samples using an implicit quadtree split and that these coding units are the root of two separate coding_quadtree syntax structure for luma and chroma.

log2_ctu_size_minus2 plus 2 specifies the luma coding tree block size of each CTU.

The variables CtbLog2SizeY, CtbSizeY, MinCbLog2SizeY, MinCbSizeY, MinTbLog2SizeY, MaxTbLog2SizeY, PicWidthInCtbsY, PicHeightInCtbsY, PicSizeInCtbsY, PicWidthInMinCbsY, PicHeightInMinCbsY, PicSizeInMinCbsY, PicSizeInSamplesY, PicWidthInSamplesC and PicHeightInSamplesC are derived as follows:

CtbLog2SizeY = log2_ctu_size_minus2 + 2		(7-6)

CtbSizeY = 1  <<  CtbLog2SizeY		(7-7)

MinCbLog2SizeY = 2		(7-8)

MinCbSizeY = 1  <<  MinCbLog2SizeY		(7-9)

MinTbSizeY = 4		(7-10)

MaxTbSizeY = 64		(7-11)

PicWidthInCtbsY = Ceil( pic_width_in_luma_samples ÷ CtbSizeY )	(7-12)

PicHeightInCtbsY = Ceil( pic_height_in_luma_samples ÷ CtbSizeY )	(7-13)

PicSizeInCtbsY = PicWidthInCtbsY * PicHeightInCtbsY	(7-14)

PicWidthInMinCbsY = pic_width_in_luma_samples / MinCbSizeY	(7-15)

PicHeightInMinCbsY = pic_height_in_luma_samples / MinCbSizeY	(7-16)

PicSizeInMinCbsY = PicWidthInMinCbsY * PicHeightInMinCbsY	(7-17)

PicSizeInSamplesY = pic_width_in_luma_samples * pic_height_in_luma_samples	(7-18)

PicWidthInSamplesC = pic_width_in_luma_samples / SubWidthC	(7-19)

PicHeightInSamplesC = pic_height_in_luma_samples / SubHeightC	(7-20)

[Ed. (BB): Currently the minimum CU size is fixed (4x4 luma samples and corresponding chroma samples) as well as the maximum transform size (64x64 luma samples and corresponding chroma sample size) and the minimum transform size (4x4 luma samples and corresponding chroma samples), pending further specification development.]

The variables CtbWidthC and CtbHeightC, which specify the width and height, respectively, of the array for each chroma CTB, are derived as follows:

–	If chroma_format_idc is equal to 0 (monochrome) or separate_colour_plane_flag is equal to 1, CtbWidthC and CtbHeightC are both equal to 0.

–	Otherwise, CtbWidthC and CtbHeightC are derived as follows:

CtbWidthC = CtbSizeY / SubWidthC		(7-21)

CtbHeightC = CtbSizeY / SubHeightC		(7-22)

For log2BlockWidth ranging from 0 to 4 and f or log2BlockHeight ranging from 0 to 4, inclusive, the up-right diagonal scan order array initialization process as specified in clause 6.5.2 is invoked with 1  <<  log2BlockWidth and 1  <<  log2BlockHeight as input, and the output is assigned to DiagScanOrder[ log2BlockWidth ][ log2BlockHeight ].

log2_min_qt_size_intra_slices_minus2 plus 2 specifies the minimum luma size of a leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 2 (I). The value of log2_min_qt_size_intra_slices_minus2 shall be in the range of 0 to CtbLog2SizeY − 2, inclusive.

MinQtLog2SizeIntraY = log2_min_qt_size_intra_slices_minus2 + 2	(7-23)

[Ed. (BB): The leaf of a quadtree can either be a coding unit or the root of a nested multi-type tree.]

log2_min_qt_size_inter_slices_minus2 plus 2 specifies the minimum luma size of a leaf block resulting from quadtree splitting of a CTU in slices with slice_type equal to 0 (B) or 1 (P). The value of log2_min_qt_size_inter_slices_minus2 shall be in the range of 0 to CtbLog2SizeY − 2, inclusive.

MinQtLog2SizeInterY = log2_min_qt_size_inter_slices_minus2 + 2	(7-24)

max_mtt_hierarchy_depth_inter_slices specifies the maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf in slices with slice_type equal to 0 (B) or 1 (P). The value of max_mtt_hierarchy_depth_inter_slices shall be in the range of 0 to CtbLog2SizeY − MinTbLog2SizeY, inclusive.

max_mtt_hierarchy_depth_intra_slices specifies the maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf in slices with slice_type equal to 2 (I). The value of max_mtt_hierarchy_depth_intra_slices shall be in the range of 0 to CtbLog2SizeY − MinTbLog2SizeY, inclusive.

sps_cclm_enabled_flag equal to 0 specifies that the cross-component linear model intra prediction from luma component to chroma component is disabled. sps_cclm_enabled_flag equal to 1 specifies that the cross-component linear model intra prediction from luma component to chroma componenent is enabled.

sps_alf_enabled_flag equal to 0 specifies that the adaptive loop filter is disabled. sps_alf_enabled_flag equal to 1 specifies that the adaptive loop filter is enabled.

sps_temporal_mvp_enabled_flag equal to 1 specifies that slice_temporal_mvp_enabled_flag is present in the slice headers of slices with slice_type not equal to I in the CVS. sps_temporal_mvp_enabled_flag equal to 0 specifies that slice_temporal_mvp_enabled_flag is not present in slice headers and that temporal motion vector predictors are not used in the CVS.

sps_sbtmvp_enabled_flag equal to 1 specifies that subblock-based temporal motion vector predictors may be used in decoding of pictures with all slices having slice_type not equal to I in the CVS. sps_sbtmvp_enabled_flag equal to 0 specifies that subblock-based temporal motion vector predictors are not used in the CVS. When sps_sbtmvp_enabled_flag is not present, it is inferred to be equal to 0

sps_amvr_enabled_flag equal to 1 specifies that adaptive motion vector difference resolution is used in motion vector coding. amvr_enabled_flag equal to 0 specifies that adaptive motion vector difference resolution is not used in motion vector coding.

sps_affine_enabled_flag specifies whether affine model based motion compensation can be used for inter prediction. If sps_affine_enabled_flag is equal to 0, the syntax shall be constrained such that no affine model based motion compensation is used in the CVS, and merge_affine_flag, inter_affine_flag and cu_affine_type_flag are not present in coding unit syntax of the CVS. Otherwise (sps_affine_enabled_flag is equal to 1), affine model based motion compensation can be used in the CVS.

sps_affine_type_flag specifies whether 6-parameter affine model based motion compensation can be used for inter prediction. If sps_affine_type_flag is equal to 0, the syntax shall be constrained such that no 6-parameter affine model based motion compensation is used in the CVS, and cu_affine_type_flag is not present in coding unit syntax in the CVS. Otherwise (sps_affine_type_flag is equal to 1), 6-parameter affine model based motion compensation can be used in the CVS. When not present, the value of sps_affine_type_flag is inferred to be equal to 0.

sps_mts_intra_enabled_flag equal to 1 specifies that cu_mts_flag may be present in the residual coding syntax for intra coding units. sps_mts_intra_enabled_flag equal to 0 specifies that cu_mts_flag is not present in the residual coding syntax for intra coding units.

sps_mts_inter_enabled_flag specifies that cu_mts_flag may be present in the residual coding syntax for inter coding units. sps_mts_inter_enabled_flag equal to 0 specifies that cu_mts_flag is not present in the residual coding syntax for inter coding units.

Slice header semantics

General slice header semantics

When present, the value of the slice header syntax element slice_pic_parameter_set_id shall be the same in all slice headers of a coded picture.

slice_pic_parameter_set_id specifies the value of pps_pic_parameter_set_id for the PPS in use. The value of slice_pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

slice_address specifies the address of the first CTB in the slice, in CTB raster scan of a picture. The length of the slice_address syntax element is Ceil( Log2( PicSizeInCtbsY ) ) bits. The value of slice_address shall be in the range of 0 to PicSizeInCtbsY − 1, inclusive, and the value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

The variable CtbAddrInRs, specifying a CTB address in CTB raster scan of a picture, is set equal to slice_address.

slice_type specifies the coding type of the slice according to Table 7-2.

Table 7-2 – Name association to slice_type

| slice_type | Name of slice_type |
| --- | --- |
| 0 | B (B slice) |
| 1 | P (P slice) |
| 2 | I (I slice) |

When nal_unit_type is equal to IRAP_NUT, i.e., the picture is an IRAP picture, slice_type shall be equal to 2.

log2_diff_ctu_max_bt_size specifies the difference between the luma CTB size and the maximum luma size (width or height) of a coding block that can be split using a binary split. The value of log2_diff_ctu_max_bt_size shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive.

When log2_diff_ctu_max_bt_size is not present, the value of log2_diff_ctu_max_bt_size is inferred to be equal to 2.

The variables MinQtLog2SizeY, MaxBtLog2SizeY, MinBtLog2SizeY, MaxTtLog2SizeY, MinTtLog2SizeY, MaxBtSizeY, MinBtSizeY, MaxTtSizeY, MinTtSizeY and MaxMttDepth are derived as follows:

MinQtLog2SizeY  =  ( slice_type  = =  I )  ?  MinQtLog2SizeIntraY  :  MinQtLog2SizeInterY	(7-26)

MaxBtLog2SizeY  =  CtbLog2SizeY − log2_diff_ctu_max_bt_size	(7-27)

MinBtLog2SizeY  =  MinCbLog2SizeY		(7-28)

MaxTtLog2SizeY  =  ( slice_type  = =  I )  ?  5  :  6	(7-29)

MinTtLog2SizeY  =  MinCbLog2SizeY		(7-30)

MinQtSizeY  =  1  <<  MinQtLog2SizeY		(7-31)

MaxBtSizeY  =  1  <<  MaxBtLog2SizeY		(7-32)

MinBtSizeY  =  1  <<  MinBtLog2SizeY		(7-33)

MaxTtSizeY  =  1  <<  MaxTtLog2SizeY		(7-34)

MinTtSizeY  =  1  <<  MinTtLog2SizeY		(7-35)

MaxMttDepth  =  ( slice_type  = =  I )  ?  max_mtt_hierarchy_depth_intra_slices  :  
				max_mtt_hierarchy_depth_inter_slices	(7-36)

[Ed. (BB): Currently the maximum TT size is fixed (32x32 luma samples and corresponding chroma samples for I-slices and 64x64 luma samples and corresponding chroma samples for P/B-slices ) as well as the maximum BT size for I-slices (CtbLog2SizeY − 2, e.g. 32x32 luma samples and corresponding chroma samples for a CTU size of 128x128 luma samples).]

(7-37)

slice_temporal_mvp_enabled_flag specifies whether temporal motion vector predictors can be used for inter prediction. If slice_temporal_mvp_enabled_flag is equal to 0, the syntax elements of the current picture shall be constrained such that no temporal motion vector predictor is used in decoding of the current picture. Otherwise (slice_temporal_mvp_enabled_flag is equal to 1), temporal motion vector predictors may be used in decoding of the current picture. When not present, the value of slice_temporal_mvp_enabled_flag is inferred to be equal to 0.

mvd_l1_zero_flag equal to 1 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is not parsed and MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to 0 for compIdx = 0..1. mvd_l1_zero_flag equal to 0 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is parsed.

collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1. When collocated_from_l0_flag is not present, it is inferred to be equal to 1.

six_minus_max_num_merge_cand specifies the maximum number of merging motion vector prediction (MVP) candidates supported in the slice subtracted from 6. The maximum number of merging MVP candidates, MaxNumMergeCand is derived as follows:

MaxNumMergeCand = 6 − six_minus_max_num_merge_cand	(7-38)

The value of MaxNumMergeCand shall be in the range of 1 to 6, inclusive.

slice_alf_enabled_flag equal to 1 specifies that adaptive loop filter is enabled and may be applied to Y, Cb, or Cr colour component in a slice. slice_alf_enabled_flag equal to 0 specifies that adaptive loop filter is disabled for all colour components in a slice.

dep_quant_enabled_flag equal to 0 specifies that dependent quantization is disabled. dep_quant_enabled_flag equal to 1 specifies that dependent quantization is enabled.

sign_data_hiding_enabled_flag equal to 0 specifies that sign bit hiding is disabled. sign_data_hiding_enabled_flag equal to 1 specifies that sign bit hiding is enabled.  When sign_data_hiding_enabled_flag is not present, it is inferred to be equal to 0.

Decoding process

Decoding process for coding units coded in inter prediction mode

General decoding process for coding units coded in inter prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Output of this process is a modified reconstructed picture before in-loop filtering.

The decoding process for coding units coded in inter prediction mode consists of the following ordered steps:

- The motion vector components and reference indices of the current coding unit are derived as follows:

- If MotionModelIdc[ xCb ][ yCb ] is equal to 0, the following applies:

- The derivation process for motion vector components and reference indices as specified in clause 8.3.2.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the luma motion vectors mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ], the chroma motion vectors mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ], the reference indices refIdxL0 and refIdxL1 and the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1 as outputs.

- Otherwise ( MotionModelIdc[ xCb ][ yCb ] is equal to 1 or 2), the following applies:

- The derivation process for affine motion vector components and reference indices as specified in clause 8.3.3.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight as inputs, and the reference indices refIdxL0 and refIdxL1, the prediction list utilization flags predFlagL0 and predFlagL1, the luma motion vector array mvLX[ xSbIdx ][ ySbIdx ], and the chroma motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..(cbWidth >> 2) − 1, and ySbIdx = 0..( cbHeight >> 2 ) − 1, with X being 0 or 1 as outputs.

- The prediction samples of the current coding unit are derived as follows:

- The variables numSbX and numSbY are modified as follows

numSbX  =  ( MotionModelIdc[ xCb ][ yCb ]  =  =  0 )  ?  numSbX  :  ( cbWidth >> 2 )	(8-99

numSbY  =  ( MotionModelIdc[ xCb ][ yCb ]  =  =  0 )  ?  numSbY  :  ( cbHeight >> 2 )	(8-100

- The decoding process for inter blocks as specified in clause 8.3.4.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight, the luma motion vectors mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ], the chroma motion vectors mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, and ySbIdx = 0..numSbY − 1, the reference indices refIdxL0 and refIdxL1, and the prediction list utilization flags predFlagL0 and predFlagL1 as inputs, and the inter prediction samples (predSamples) that are an (cbWidth)x(cbHeight) array predSamplesL of prediction luma samples and two (cbWidth / 2)x(cbHeight / 2) arrays predSamplesCr and predSamplesCr of prediction chroma samples, one for each of the chroma components Cb and Cr, as outputs.

- The residual samples of the current coding unit are derived as follows:

- The decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.3.5 is invoked with the location ( xTb0, yTb0 ) set equal to the luma location ( xCb, yCb ), the width nTbW set equal to the luma coding block width cbWidth, the height nTbH set equal to the luma coding block height cbHeight and the variable cIdx as inputs, and the array resSamplesL as output.

- The decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.3.5 is invoked with the location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / 2, yCb / 2 ), the width nTbW set equal to the chroma coding block width cbWidth / 2, the height nTbH set equal to the chroma coding block height cbHeight / 2 and the variable cIdx as inputs, and the array resSamplesCb as output.

- The decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.3.5 is invoked with the location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / 2, yCb / 2 ), the width nTbW set equal to the chroma coding block width cbWidth / 2, the height nTbH set equal to the chroma coding block height cbHeight / 2 and the variable cIdx as inputs, and the array resSamplesCr as output.

- The reconstructed samples of the current coding unit are derived as follows:

- The picture reconstruction process for a colour component as specified in clause 8.4.5 is invoked with the block location ( xB, yB ) set equal to ( xCb, yCb ), the block width bWidth set equal to cbWidth, the block height bHeight set equal to cbHeight, the variable cIdx, the (cbWidth)x(cbHeight) array predSamples set equal to predSamplesL and the (cbWidth)x(cbHeight) array resSamples set equal to resSamplesL as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- The picture reconstruction process for a colour component as specified in clause 8.4.5 is invoked with the block location ( xB, yB ) set equal to ( xCb / 2, yCb / 2 ), the block width bWidth set equal to cbWidth / 2, the block height bHeight set equal to cbHeight / 2, the variable cIdx, the (cbWidth / 2)x(cbHeight / 2) array predSamples set equal to predSamplesCb and the (cbWidth / 2)x(cbHeight / 2) array resSamples set equal to resSamplesCb as inputs, and the output is a modified reconstructed picture before in-loop filtering.

- The picture reconstruction process for a colour component as specified in clause 8.4.5 is invoked with the block location ( xB, yB ) set equal to ( xCb / 2, yCb / 2 ), the block width bWidth set equal to cbWidth / 2, the block height bHeight set equal to cbHeight / 2, the variable cIdx, the (cbWidth / 2)x(cbHeight / 2) array predSamples set equal to predSamplesCb and the (cbWidth / 2)x(cbHeight / 2) array resSamples set equal to resSamplesCb as inputs, and the output is a modified reconstructed picture before in-loop filtering.

Derivation process for motion vector components and reference indices

Derivation process for luma motion vectors for merge mode

This process is only invoked when merge_flag[ xCb ][ yPb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY,

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1,

- the reference indices refIdxL0 and refIdxL1,

- the prediction list utilization flags predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1.

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1 and the prediction utilization flags predFlagL0 and predFlagL1 are derived by the following ordered steps:

- The derivation process for merging candidates from neighbouring coding units as specified in clause 8.3.2.3 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the luma coding block width as inputs, and the output being the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1 and refIdxLXB2, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and predFlagLXB2, and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1 and mvLXB2, with X being 0 or 1.

- When cbWidth and cbHeight are greater than or equal to 8, The derivation process for subblock-based temporal merging candidates as specified in clause 8.3.2.4 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight , the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1, the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1, the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and the motion vectors mvLXA0, mvLXA1, mvLXB0, mvLXB1 as inputs and the output being the availability flag availableFlagSbCol, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the reference indices refIdxLXSbCol, the luma motion vectors mvLXSbCol[ xSbIdx ][ ySbIdx ] and the prediction list utilization flags predFlagLXSbCol[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1 and X being 0 or 1.

- The reference indices for the temporal merging candidate, refIdxLXCol, with X being 0 or 1, are set equal to 0.

- The derivation process for temporal luma motion vector prediction as specified in in clause 8.3.2.11 is invoked with the luma location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL0Col as inputs, and the output being the availability flag availableFlagL0Col and the temporal motion vector mvL0Col. The variables availableFlagCol, predFlagL0Col and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col		(8-113)

predFlagL0Col = availableFlagL0Col		(8-114)

predFlagL1Col = 0		(8-115)

- When slice_type is equal to B, the derivation process for temporal luma motion vector prediction as specified in clause 8.3.2.11 is invoked with the luma location ( xCb, yCb ), the the luma coding block width cbWidth, the luma coding block height cbHeight and the variable refIdxL1Col as inputs, and the output being the availability flag availableFlagL1Col and the temporal motion vector mvL1Col. The variables availableFlagCol and predFlagL1Col are derived as follows:

availableFlagCol = availableFlagL0Col  | |  availableFlagL1Col	(8-116)

predFlagL1Col = availableFlagL1Col		(8-117)

- The merging candidate list, mergeCandList, is constructed as follows:

i = 0
if( availableFlagA1 )
	mergeCandList[ i++ ] = A1
if( availableFlagB1 )
	mergeCandList[ i++ ] = B1
if( availableFlagB0 )
	mergeCandList[ i++ ] = B0		(8-118)
if( availableFlagA0 )
	mergeCandList[ i++ ] = A0
if( availableFlagSbCol )
	mergeCandList[ i++ ] = SbCol
if( availableFlagB2 )
	mergeCandList[ i++ ] = B2
if( availableFlagCol )
	mergeCandList[ i++ ] = Col

- The variable numCurrMergeCand and numOrigMergeCand are set equal to the number of merging candidates in the mergeCandList.

- When slice_type is equal to B, the derivation process for combined bi-predictive merging candidates specified in clause 8.3.2.6 is invoked with mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList, numCurrMergeCand and numOrigMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0combCandk and refIdxL1combCandk, the prediction list utilization flags predFlagL0combCandk and predFlagL1combCandk and the motion vectors mvL0combCandk and mvL1combCandk of every new candidate combCandk being added into mergeCandList. The number of candidates being added, numCombMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand ). When numCombMergeCand is greater than 0, k ranges from 0 to numCombMergeCand − 1, inclusive.

- The derivation process for zero motion vector merging candidates specified in clause 8.3.2.7 is invoked with the mergeCandList, the reference indices refIdxL0N and refIdxL1N, the prediction list utilization flags predFlagL0N and predFlagL1N, the motion vectors mvL0N and mvL1N of every candidate N in mergeCandList and numCurrMergeCand as inputs, and the output is assigned to mergeCandList, numCurrMergeCand, the reference indices refIdxL0zeroCandm and refIdxL1zeroCandm, the prediction list utilization flags predFlagL0zeroCandm and predFlagL1zeroCandm and the motion vectors mvL0zeroCandm and mvL1zeroCandm of every new candidate zeroCandm being added into mergeCandList. The number of candidates being added, numZeroMergeCand, is set equal to ( numCurrMergeCand − numOrigMergeCand − numCombMergeCand ). When numZeroMergeCand is greater than 0, m ranges from 0 to numZeroMergeCand − 1, inclusive.

- The following applies:

- If mergeCandList[ merge_idx[ xCb ][ yCb ] ] is equal to SbCol, the following assignments are made for xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1

refIdxLX = refIdxLXSbCol		(8-119)

predFlagLX[ xSbIdx ][ ySbIdx ] = predFlagLXSbCol[ xSbIdx ][ ySbIdx ]	(8-120)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = mvLXSbCol[ xSbIdx ][ ySbIdx ][ 0 ]	(8-121)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = mvLXSbCol[ xSbIdx ][ ySbIdx ][ 1 ]	(8-122)

- Otherwise, the following assignments are made with N being the candidate at position merge_idx[ xCb ][ yCb ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xCb ][ yCb ] ] ) and X being replaced by 0 or 1:

numSbX = 1		(8-123)

numSbY = 1		(8-124)

refIdxLX = refIdxLXN		(8-125)

predFlagLX[ 0 ][ 0 ] = predFlagLXN		(8-126)

mvLX[ 0 ][ 0 ][ 0 ] = mvLXN[ 0 ]		(8-127)

mvLX[ 0 ][ 0 ][ 1 ] = mvLXN[ 1 ]		(8-128)

Derivation process for subblock-based temporal merging candidates

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

- the availability flags availableFlagA0, availableFlagA1, availableFlagB0, and availableFlagB1 of the neighbouring coding units,

- the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, and refIdxLXB1 of the neighbouring coding units,

- the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, and predFlagLXB1 of the neighbouring coding units,

- the motion vectors in 1/16 fractional-sample accuracy mvLXA0, mvLXA1, mvLXB0, and mvLXB1 of the neighbouring coding units.

Outputs of this process are:

- the availability flag availableFlagSbCol,

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY,

- the reference indices refIdxL0SbCol and refIdxL1SbCol,

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0SbCol[ xSbIdx ][ ySbIdx ] and mvL1SbCol[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1,

- the prediction list utilization flags predFlagL0SbCol[ xSbIdx ][ ySbIdx ] and predFlagL1SbCol[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1.

The availability flag availableFlagSbCol is derived as follows.

- If either slice_temporal_mvp_enable_flag or sps_sbtmvp_flag is equal to 0, availableFlagSbCol is set equal to 0.

- Otherwise, the following ordered steps apply.

- The location ( xCtb, yCtb ) of the top-left sample of the luma coding tree block that contains the current coding block and the location ( xCtr, yCtr ) of the below-right center sample of the current luma coding block are derived as follows:

xCtb = ( xCb  >>  CtuLog2Size )  <<  CtuLog2Size	(8-144)

yCtb = ( yCb  >>  CtuLog2Size )  <<  CtuLog2Size	(8-145)

xCtr = xCb + ( cbWidth / 2 )		(8-146)

yCtr = yCb + ( cbHeight / 2 )		(8-147)

- The luma location ( xColCtrCb, yColCtrCb ) is set equal to the top-left sample of the collocated luma coding block covering the location given by ( xCtr, yCtr ) inside ColPic relative to the top-left luma sample of the collocated picture specified by ColPic.

- The derivation process for subblock-based temporal merging base motion data as specified in clause 8.3.2.5 is invoked with the location ( xCtb, yCtb ), the location ( xColCtrCb, yColCtrCb ), the availability flags availableFlagA0, availableFlagA1, availableFlagB0 and availableFlagB1, and the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0 and predFlagLXB1, and the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0 and refIdxLXB1, and the motion vectors mvLXA0, mvLXA1, mvLXB0 and mvLXB1, with X being 0 and 1 as inputs and the motion vectors ctrMvLX, the prediction list utilization flags ctrPredFlagLX and the reference indices ctrRefIdxLX of the collocated block, with X being 0 and 1, and the temporal motion vector tempMV as outputs.

- The variable availableFlagSbCol is derived as follows:

- If both ctrPredFlagL0 and ctrPredFlagL1 are equal to 0, availableFlagSbCol is set equal to 0.

- Otherwise, availableFlagSbCol is set equal to 1.

When availableFlagSbCol is equal to 1, the following applies:

- The variables numSbX, numSbY, sbWidth, sbHeight and refIdxLXSbCol are derived as follows:

numSbX  =  cbWidth >> 3	(8-148)

numSbY  =  cbHeight >> 3	(8-149)

sbWidth  =  cbWidth / numSbX		(8-150)

sbHeight  =  cbHeight / numSbY		(8-151)

refIdxLXSbCol  =  0			(8-152)

- For xSbIdx = 0..numSbX − 1 and ySbIdx = 0 .. numSbY − 1, the motion vectors mvLXSbCol[ xSbIdx ][ ySbIdx ] and prediction list utilization flags predFlagLXSbCol[ xSbIdx ][ ySbIdx ] are derived as follows:

- The luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows:

xSb  =  xCb + xSbIdx * sbWidth		(8-153)

ySb  =  yCb + ySbIdx * sbHeight		(8-154)

- The location ( xColSb, yColSb ) of the collocated subblock inside ColPic is derived as follows.

xColSb = Clip3( xCtb, 
	Min( CurPicWidthInSamplesY − 1, xCtb + ( 1  <<  CtbLog2SizeY ) +3 ),	(8-155)
 	xSb + ( tempMv[0]  >>  4 ) )

yColSb = Clip3( yCtb, 
	Min( CurPicHeightInSamplesY − 1, yCtb + ( 1  <<  CtbLog2SizeY ) +3 ),	(8-156)
 	ySb + ( tempMv[1]  >>  4 ) )

- The variable currCb specifies the luma coding block covering the sub-block inside the current picture.

- The variable colCb specifies the luma coding block covering the modified location given by ( ( xColSb >> 3 ) << 3, ( yColSb >> 3 ) << 3 ) inside the ColPic.

- The luma location ( xColCb, yColCb ) is set equal to the top-left sample of the collocated luma coding block specified by colCb relative to the top-left luma sample of the collocated picture specified by ColPic.

- The derivation process for collocated motion vectors as specified in clause 8.3.2.12 is invoked with currCb, colCb, ( xColCb, yColCb ), refIdxL0 set equal to 0 and sbFlag set equal to 1 as inputs and the output being assigned to the motion vector of the subblock mvL0SbCol[ xSbIdx ][ ySbIdx ] and availableFlagL0SbCol.

- The derivation process for collocated motion vectors as specified in clause 8.3.2.12 is invoked with currCb, colCb, ( xColCb, yColCb ), refIdxL1 set equal to 0 and sbFlag set equal to 1 as inputs and the output being assigned to the motion vector of the subblock mvL1SbCol[ xSbIdx ][ ySbIdx ] and availableFlagL1SbCol.

- When availableFlagL0SbCol and availableFlagL1SbCol are both equal to 0, the following applies:

mvLXSbCol[ xSbIdx ][ ySbIdx ] = ctrMvLX		(8-157)

predFlagLXSbCol[ xSbIdx ][ ySbIdx ] = ctrPredFlagLX	(8-158)

Derivation process for subblock-based temporal merging base motion data

Inputs to this process are:

- the location ( xCtb, yCtb ) of the top-left sample of the luma coding tree block that contains the current coding block,

- the location ( xColCtrCb, yColCtrCb ) of the top-left sample of the collocated luma coding block that covers the below-right center sample.

- the availability flags availableFlagA0, availableFlagA1, availableFlagB0, and availableFlagB1 of the neighbouring coding units,

- the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, and refIdxLXB1 of the neighbouring coding units,

- the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, and predFlagLXB1 of the neighbouring coding units,

- the motion vectors in 1/16 fractional-sample accuracy mvLXA0, mvLXA1, mvLXB0, and mvLXB1 of the neighbouring coding units.

Outputs of this process are:

- the motion vectors ctrMvL0 and ctrMvL1,

- the prediction list utilization flags ctrPredFlagL0 and ctrPredFlagL1,

- the reference indices ctrRefIdxL0 and ctrRefIdxL1,

- the temporal motion vector tempMV.

The variables tempMv, numRefLists and bTerminate are set as follows:

tempMv[ 0 ] = 0		(8-159)

tempMv[ 0 ] = 0		(8-160)

numRefLists = (slice_type = = B) ? 2: 1		(8-161)

bTerminate = FALSE		(8-162)

The variable currPic specifies the current picture.

For each reference picture list LX with X ranging from 0 to ( numRefList − 1 ), mvTemp is derived as follows:

- When all of the following conditions are equal to true, X is set equal to !X:

  - DiffPicOrderCnt(aPic, currPic) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice,

  - slice_type is equal to B,

  - collocated_from_l0_flag is equal to 0.

- When bTerminate is equal to FALSE, and availableFlagA0 and predFlagLXA0 are equal to 1 and DiffPicOrderCnt(ColPic, RefPicListX[refIdxLXA0]) is equal to 0, bTerminate is set to TRUE and mvTemporal is set equal to mvLXA0:

- When bTerminate is equal to FALSE, availableFlagLB0 and predFlagLXB0 are equal to 1, DiffPicOrderCnt(ColPic, RefPicListX[refIdxLXB0]) is equal to 0, bTerminate is set to TRUE and mvTemporal is set equal to mvLXB0.

- When bTerminate is equal to FALSE, availableFlagB1 and predFlagLXB1 are equal to 1, DiffPicOrderCnt(ColPic, RefPicListX[refIdxLXB1]) is equal to 0, bTerminate is set to TRUE and mvTemporal is set equal to mvLXB1.

- When bTerminate is equal to FALSE, availableFlagA1 and predFlagLXB1 are equal to 1, DiffPicOrderCnt(ColPic, RefPicListX[refIdxLXA1]) is equal to 0, bTerminate is set to TRUE and mvTemporal is set to mvLXA1.

The location ( xColCb, yColCb ) of the collocated block inside ColPic is derived as follows.

xColCb = Clip3( xCtb, 
	Min( CurPicWidthInSamplesY − 1, xCtb + ( 1  <<  CtbLog2SizeY ) +3 ),	(8-163)
 	xColCtrCb + ( tempMv[0]  >>  4 ) )

yColCb = Clip3( yCtb, 
	Min( CurPicHeightInSamplesY − 1, yCtb + ( 1  <<  CtbLog2SizeY ) +3 ),	(8-164)
 	yColCtrCb + ( tempMv[1]  >>  4 ) )

The array colPredMode is set equal to the prediction mode array CuPredMode of the collocated picture specified by ColPic.

The motion vectors ctrMvL0 and ctrMvL1, the prediction list utilization flags ctrPredFlagL0 and ctrPredFlagL1, and the reference indices ctrRefIdxL0 and ctrRefIdxL1 are derived as follows:

- If colPredMode[xColCb][yColCb] is equal to MODE_INTER, the following applies:

- The variable currCb specifies the luma coding block covering ( xCtrCb ,yCtrCb ) inside the current picture.

- The variable colCb specifies the luma coding block covering the modified location given by ( ( xColCb >> 3 ) << 3, ( yColCb >> 3 ) << 3 ) inside the ColPic.

- The luma location ( xColCb, yColCb ) is set equal to the top-left sample of the collocated luma coding block specified by colCb relative to the top-left luma sample of the collocated picture specified by ColPic.

  - The derivation process for temporal motion vector prediction in subclause  8.3.2.12 is invoked with currCb, colCb,  (xColCb, yColCb), centerRefIdxL0, and sbFlag set equal to 1 as inputs and the output being assigned to centerMvL0 and centerPredFlagL0.

  - The derivation process for temporal motion vector prediction in subclause  8.3.2.12 is invoked with currCb, colCb,  (xColCb, yColCb), centerRefIdxL1, and sbFlag set equal to 1 as inputs and the output being assigned to centerMvL1 and centerPredFlagL1.

- Otherwise, the following applies:

ctrPredFlagL0 = 0		(8-165)

ctrPredFlagL1 = 0		(8-166)

