| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0515-v1 |
| --- | --- |

| Title: | CE3-related: Non-zero reference lines padding method on the top-line of CTU |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Po-Han Lin1, Chi-Chang Kuo, Ching-Chieh Lin2, Chun-Lung Lin3 | Email: | pohan@itri.com1 jacklin@itri.com2 chunlung@itri.com3 |
| Source: | ITRI International |  |  |

_____________________________

- Abstract

This contribution proposes a reference samples padding method in the top-line of CTU when multiple reference line intra prediction (MRL) is enabled.  According to the proposed method, MRL could be used even on the top-line of a CTU without accessing additional pixels outside the current CTU line. The results show that 0.44% and 0.20% gain observed in AI and RA test conditions respectively. The proposed method could retrieve some gain loss from disabling MRL of CU on the top-line of CTU without obvious encoding and decoding time increasing.

Introduction

Multiple reference line intra prediction (MRL) uses extended reference lines to predict a CU block in the intra prediction as shown in Fig. 1. A CU selects one of the available reference lines for generating the predictor, and a syntax would be signalled to indicate which reference line is used. CE3-1.1 was established in the last meeting to study MRL. In this sub-CE, only reference line 0, 1 and 3 are available. More detail could be found in JVET-L0283 [1].

In CE3-1.1, both test 1.1.3 and 1.1.4 proposed to exploit the MRL only in the MPM modes of a CU. Test 1.1.4 disabled the MRL if the current CU is on the top-line of a CTU to avoid using additional lines across the CTUs. This test may be friendlier for hardware implementation. However, the results show 0.06%/0.02% loss in AI/RA condition compared to test 1.1.3.

Figure 1. Concept of multiple reference line

Figure 2. Top-line buffer and left buffers

Figure 3. Example of CTU top boundary CU, N means an unavailable sample which is out of the line buffer

Proposed method

This contribution proposes a padding method for non-zero reference lines when the current CU is on the top-line of a CTU based on test 1.1.4. The padding method generates the non-zero reference lines from the nearest top-line buffer and the left buffers as illustrated in Fig. 2. If there is only one line buffer allowed for intra prediction, all samples above the line of X3 to Pp are not available for reference samples on the top of CTU boundary. When reference line 1 or 3 is used, the samples in the segment D, segment E and partial samples of the segment C are unavailable as shown in Fig. 3.

The proposed method will pad the segment D and E samples from the samples right below it on the top-line buffer. And for samples in the segment C, an interpolation filter is applied to derive the reference sample(s) as shown in Fig. 4 and Fig. 5. For reference line 1, there is only 1 sample Fa in the segment C. Fa could be derived by the following equation.

Fa=X1+L1+1≫1

For reference line 3, there are 3 samples Fb, Fc and Fd in the segment C. They could be derived by the following equations.

Fb=3*X3+L3+2≫2

Fc=X3+L3+1≫1

Fd=X3+3*L3+2≫2

The samples in the segment F are padded from the nearest sample in segment E which is the same as the method described in L0283.

Figure 4. Padding in reference line 1

Figure 5. Padding in reference line 3

Experimental results

The implementation is on top of the CE3-1.1.4 software. And the simulation is launched under common test condition [2]. Table 1 and Table 2 show the experimental results of the proposed method. It shows 0.44% gain in AI condition and 0.20% gain in RA condition. No encoding and decoding time impact is observed compared to test 1.1.4.

Table 1: Summary results in AI condition

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | 0.01% | 0.03% | 102% | 100% |
| Class A2 | -0.26% | -0.09% | -0.11% | 102% | 100% |
| Class B | -0.46% | -0.30% | -0.21% | 102% | 100% |
| Class C | -0.79% | -0.60% | -0.36% | 102% | 100% |
| Class E | -0.44% | -0.22% | -0.23% | 103% | 100% |
| Overall | -0.44% | -0.27% | -0.19% | 102% | 100% |
| Class D | -0.22% | 0.04% | -0.17% | 101% | 100% |
| Class F (optional) | -1.35% | -1.13% | -0.96% | 103% | 99% |

Table 2: Summary results in RA condition

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.02% | 0.04% | 102% | 101% |
| Class A2 | -0.19% | 0.12% | 0.00% | 102% | 101% |
| Class B | -0.20% | -0.02% | 0.02% | 102% | 100% |
| Class C | -0.27% | 0.11% | 0.05% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.20% | 0.05% | 0.03% | 102% | 101% |
| Class D | -0.02% | -0.11% | 0.39% | 102% | 100% |
| Class F (optional) | -0.91% | -0.73% | -0.38% | 101% | 99% |

Conclusions

This contribution proposes a simple modification which could retrieve some gain from the top-line CTU restriction. It shows 0.44% and 0.20 gain in AI and RA test conditions. It is proposed to adopt the modification based on CE3 test 1.1.4.

References

- B. Bross, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand, L. Zhao, X. Zhao, X. Li, S. Liu, Y.-J. Chang, H.-Y. Jiang, P.-H. Lin, C.-C. Lin “CE3: Multiple reference line intra prediction (Test 1.1.1, 1.1.2, 1.1.3 and 1.1.4), ” JVET-L0283, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, 11th meeting, Ljubljana, SI, 10–18 July 2018.

Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).