| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0515-v3 |
| --- | --- |

| Title: | CE3-related: Non-zero reference lines padding method on the top-line of CTU |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Po-Han Lin1, Chi-Chang Kuo, Ching-Chieh Lin2, Chun-Lung Lin3 | Email: | pohan@itri.com1 jacklin@itri.com2 chunlung@itri.com3 |
| Source: | ITRI International |  |  |

_____________________________

- Abstract

This contribution proposes two reference samples padding methods on the top-line of CTU when multiple reference line intra prediction (MRL) is enabled. According to the proposed method, MRL could be used even on the top-line of a CTU without accessing additional pixels outside the current CTU. The results of proposed method 2 show 0.44% and 0.21% gain in AI and RA test conditions respectively. The proposed method could retrieve some gain loss from disabling MRL of CU on the top-line of CTU without obvious encoding and decoding time increasing and unify the parsing process of MRL.

Introduction

Multiple reference line intra prediction (MRL) uses extended reference lines to predict a CU block in the intra prediction as shown in Fig. 1. A CU selects one of the available reference lines for generating the predictor, and a syntax would be signalled to indicate which reference line is used. CE3-1.1 was established in the last meeting to study MRL. In this sub-CE, only reference line 0, 1 and 3 are available. More detail could be found in JVET-L0283 [1].

In CE3-1.1, both test 1.1.3 and 1.1.4 proposed to exploit the MRL only in the MPM modes of a CU. Test 1.1.4 disabled the MRL if the current CU is on the top-line of a CTU to avoid using additional lines across the CTUs. This test may be friendlier for hardware implementation. However, the results show 0.06%/0.02% loss in AI/RA condition compared to the test 1.1.3.

Figure 1. Concept of multiple reference line

Figure 2. Top-line buffer and left buffers

Figure 3. Example of CTU top boundary CU, N means an unavailable sample which is out of the line buffer

Proposed method

This contribution proposes two padding methods for non-zero reference lines when the current CU is on the top-line of a CTU based on CE3 test 1.1.4. The padding methods generates the non-zero reference lines from the nearest top-line buffer and the left buffers as illustrated in Fig. 2. If there is only one line buffer allowed for intra prediction, all samples above the line of X3 to Pp are not available for reference samples on the top of CTU boundary. When reference line 1 or 3 is used, the samples in the segment D, segment E and partial samples of the segment C are unavailable as shown in Fig. 3.

Method 1

The proposed method 1 will pad the segment D and E samples from the samples right below it on the top-line buffer. And for samples in the segment C, an interpolation filter is applied to derive the reference sample(s) as shown in Fig. 4 and Fig. 5. For reference line 1, there is only 1 sample Fa in the segment C. Fa could be derived by the following equation.

Fa=X1+L1+1≫1

For reference line 3, there are 3 samples Fb, Fc and Fd in the segment C. They could be derived by the following equations.

Fb=3*X3+L3+2≫2

Fc=X3+L3+1≫1

Fd=X3+3*L3+2≫2

The samples in the segment F are padded from the nearest sample in segment E which is the same as the method described in JVET-L0283.

Figure 4. Padding in reference line 1

Figure 5. Padding in reference line 3

Method 2

The method 2 is quite similar to method 1, but it pads the samples in the segment C from the nearest sample in segment B without any interpolation filter. When the target sample is unavailable, it pads the samples from segment D. If the both are not available, the reference padding method in VTM is used. Method 2 is simpler than method 1 in both implementation and text. Fig. 6 describes the concept of this method.

Figure 6. Padding method 2

Experimental results

The implementation is on top of the CE3-1.1.4 software. And the simulation is launched under common test condition [2]. Table 1 and Table 2 show the experimental results of the proposed method 1 and method 2. It shows 0.44% gain in AI condition and 0.19% gain in RA condition for method 1, 0.44% gain in AI condition and 0.21% gain in RA condition for method 2 respectively. Table 3 and Table 4 show the results of the two methods over the CE3 test 1.1.4. No encoding and decoding time impact is observed compared to the test 1.1.4. Note that a small implementation bug was found; therefore, the experimental results were updated from the version 1.

Table 1: Summary results of proposed method 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.02% | 0.03% | 103% | 100% |
| Class A2 | -0.26% | -0.13% | -0.16% | 104% | 100% |
| Class B | -0.47% | -0.28% | -0.27% | 102% | 100% |
| Class C | -0.78% | -0.53% | -0.39% | 103% | 100% |
| Class E | -0.44% | -0.27% | -0.17% | 105% | 100% |
| Overall | -0.44% | -0.26% | -0.21% | 103% | 100% |
| Class D | -0.22% | 0.06% | -0.17% | 103% | 100% |
| Class F (optional) | -1.36% | -1.08% | -0.93% | 103% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.13% | 0.16% | 102% | 100% |
| Class A2 | -0.19% | 0.08% | -0.08% | 102% | 100% |
| Class B | -0.18% | -0.12% | 0.01% | 102% | 100% |
| Class C | -0.26% | 0.05% | -0.01% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | 0.02% | 0.02% | 102% | 100% |
| Class D | -0.02% | -0.15% | 0.22% | 102% | 100% |
| Class F (optional) | -0.90% | -0.64% | -0.53% | 100% | 99% |

Table 2: Summary results of proposed method 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | 0.02% | -0.01% | 102% | 100% |
| Class A2 | -0.25% | -0.13% | -0.14% | 103% | 100% |
| Class B | -0.46% | -0.27% | -0.23% | 102% | 100% |
| Class C | -0.78% | -0.55% | -0.39% | 102% | 100% |
| Class E | -0.44% | -0.27% | -0.14% | 104% | 99% |
| Overall | -0.44% | -0.26% | -0.20% | 102% | 100% |
| Class D | -0.22% | 0.07% | -0.15% | 102% | 99% |
| Class F (optional) | -1.35% | -1.06% | -1.05% | 102% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.14% | 0.09% | 102% | 100% |
| Class A2 | -0.20% | 0.13% | -0.04% | 101% | 101% |
| Class B | -0.20% | -0.02% | -0.10% | 102% | 100% |
| Class C | -0.29% | -0.04% | 0.03% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.21% | 0.04% | -0.02% | 102% | 100% |
| Class D | -0.03% | -0.14% | 0.13% | 102% | 100% |
| Class F (optional) | -0.89% | -0.63% | -0.43% | 100% | 99% |

Table 3: Summary results of method 1 over CE3-1.1.4

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.02% | 101% | 100% |
| Class A2 | -0.04% | -0.02% | -0.02% | 101% | 100% |
| Class B | -0.03% | -0.05% | -0.02% | 101% | 100% |
| Class C | -0.04% | -0.04% | -0.10% | 101% | 100% |
| Class E | -0.07% | -0.07% | 0.11% | 101% | 100% |
| Overall | -0.04% | -0.04% | -0.01% | 101% | 100% |
| Class D | -0.02% | -0.02% | 0.07% | 102% | 100% |
| Class F (optional) | -0.07% | -0.05% | -0.02% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.04% | 100% | 100% |
| Class A2 | 0.00% | 0.05% | -0.02% | 100% | 100% |
| Class B | -0.01% | -0.02% | -0.06% | 100% | 100% |
| Class C | -0.02% | 0.11% | 0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.04% | 0.00% | 100% | 100% |
| Class D | 0.03% | 0.08% | -0.01% | 100% | 100% |
| Class F (optional) | -0.04% | -0.04% | -0.03% | 100% | 100% |

Table 4: Summary results of method 2 over CE3-1.1.4

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.04% | -0.02% | 100% | 100% |
| Class A2 | -0.03% | -0.02% | 0.00% | 100% | 99% |
| Class B | -0.03% | -0.04% | 0.02% | 101% | 100% |
| Class C | -0.04% | -0.06% | -0.10% | 101% | 100% |
| Class E | -0.07% | -0.06% | 0.13% | 100% | 100% |
| Overall | -0.04% | -0.03% | 0.00% | 101% | 100% |
| Class D | -0.02% | 0.00% | 0.09% | 101% | 99% |
| Class F (optional) | -0.06% | -0.03% | -0.14% | 98% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.06% | -0.04% | 100% | 100% |
| Class A2 | 0.00% | 0.10% | 0.02% | 100% | 100% |
| Class B | -0.02% | 0.07% | -0.18% | 100% | 100% |
| Class C | -0.04% | 0.03% | 0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.06% | -0.04% | 100% | 100% |
| Class D | 0.01% | 0.09% | -0.10% | 100% | 100% |
| Class F (optional) | -0.04% | -0.02% | 0.08% | 100% | 100% |

Conclusions

This contribution proposes two simple modifications which could retrieve some gain from the top-line CTU restriction. Method 2 shows 0.44% and 0.21% gain in AI and RA under the common test conditions. Compared to CE3 test 1.1.4, the results show 0.04% and 0.02% gain in AI and RA respectively. It is proposed to adopt the method 2 based on CE3 test 1.1.4.

References

- B. Bross, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand, L. Zhao, X. Zhao, X. Li, S. Liu, Y.-J. Chang, H.-Y. Jiang, P.-H. Lin, C.-C. Lin “CE3: Multiple reference line intra prediction (Test 1.1.1, 1.1.2, 1.1.3 and 1.1.4), ” JVET-L0283, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, 11th meeting, Ljubljana, SI, 10–18 July 2018.

Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).