| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0567-v1 |
| --- | --- |

| Title: | Crosscheck for JVET-L0203 (CE4-related: LIC with reduced memory buffer) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Anish Tamse Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | tamse.anish@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the cross check results of “CE4-related: LIC with reduced memory buffer (JVET-L0203)”. The simulation was carried out under CTC. The results provided by the proponent were verified.

# Cross Check Results

The proposal addresses the memory requirement for storage of LIC parameters. CU neighbours lying outside the CTU are not used, hence reducing the memory required for parameter storage.

The simulation was carried out under the JVET common test condition [1]. The software was compiled with GCC version 7.3 and the AVX SIMD instructions. Huawei provided the source code of their proposal based on the BMS-2.0.1 software and results. The results match with those provided by the proponent.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.68% | -0.14% | -0.71% | 177% | 119% | -0.62% | -0.18% | -0.46% | 147% | 121% |
| Class A2 | -0.23% | -0.24% | -0.35% | 142% | 119% | -0.24% | -0.19% | -0.24% | 127% | 123% |
| Class B | -0.48% | -0.28% | -0.12% | 150% | 124% | -0.54% | -0.22% | -0.13% | 130% | 125% |
| Class C | -0.22% | -0.19% | -0.31% | 161% | 129% | -0.23% | -0.15% | -0.16% | 134% | 128% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.40% | -0.22% | -0.33% | 156% | 123% | -0.41% | -0.19% | -0.22% | 134% | 125% |
| Class D | 0.02% | -0.17% | -0.16% | 156% | 129% | 0.03% | 0.22% | 0.13% | 131% | 129% |
| Class F (optional) | -0.70% | -0.48% | -0.46% | 150% | 131% | -0.77% | -0.49% | -0.49% | 126% | 130% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.18% | -0.20% | -0.51% | 190% | 129% | -0.27% | -0.15% | -0.35% | 157% | 125% |
| Class E | 0.02% | 0.01% | 0.17% | 111% | 130% | -0.02% | -0.25% | -0.39% | 109% | 128% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.06% | 0.31% | -0.25% | 175% | 130% | 0.01% | -0.32% | -0.03% | 148% | 126% |
| Class F (optional) | -2.18% | -1.30% | -1.37% | 179% | 128% | -2.14% | -2.13% | -2.28% | 149% | 126% |

# Reference

[1] “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.