| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0406r1 |
| --- | --- |

| Title: | CE14.2-related: Extended applicability of bilateral filter (CE14.2.c) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dmytro Rusanovskyy Nikolay Shlyakhov Ted Hsieh Marta Karczewicz | Tel: Email: | +1 858 651 4225 dmytror@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution report additional results of CE14.2 test on parametrized in-loop Bilateral Filtering (BIF) presented in JVET-L0406. The applicability of the bilateral filter parametrized with PWL2 was extended to large block sizes. Test CE14.2c reports extension to 16x64 and 64x16 block sizes, to align applicability, In this contribution, which is aligned with methods tested in CE14.1 and CE14.3. Complexity estimates provided in JVET-L0406 for the test CE14.2b are also applicable for the current test.

The following BD rate results comparing to the VTM2.0.1 CTC are reported in this document:

CE14.2.c: AI : -0.42% 0.13% 0.16%; RA: -0.71% -0.21% -0.17%; LDB: -0.XX% 0.XX%	0.XX%

# Introduction

Document JVET-0406 presented a parametrization of bilateral filter earlier presented in JVET-J0021. It was proposed that family of weighting functions Wkx, QP, mode can be parameterized by a piece-wise linear model from a basis function W0(x), which is coarsely represented with a small number of pivot points W0 and derivation scaler for QP Sqp and mode Sm.

Below are provided an example of PWL model parameters being tested in CE14.2.a, with full family of W(x) being compactly represented with 120 bytes:

W0[10] = {42950, 37903, 26050, 13944, 5813, 1887, 477, 94, 14, 2}

Sm[5] = { 30195, 19316, 26068, 19316, 1470 };

Sqp[45] = { 65535, 43690, 32768, 26214, 21845, 18724, 16384, 14563, 13107, 11915, 10922, 10082, 9362, 8738, 8192, 7710, 7281, 6898, 6553, 6241, 5957, 5698, 5461, 5242, 5041, 4854, 4681, 4519, 4369, 4228, 4096, 3971, 3855, 3744, 3640, 3542, 3449, 3360, 3276, 3196, 3120, 3048, 2978, 2912, 2849};

The process of W(x) derivation utilizes a piece-wise linear interpolation between j pivot points specified by W0 as following:

idxQP = QP-18;sqp = Sqp[idxQP];
R = idxQP;

sm = Sm[mode];
Wm = W0 *sm >> 15;
R[0] = 0;

For j = 1..M-1{
 	idx1 = j;
	d = (Wm[idx1] - Wm[idx1-1])*sqp >> 17							(1)

R[idx1] = R[idx1-1] + R;

For x within j-range:
	{
   		idx2 = x – R[idx1-1];												    W(x) = max(Wm[idx1], Wm[idx1-1] + idx2 *d); 
	}
}

The W(x) are represented with 16 bits accuracy, thus the size of the LUT W(x,Qp, mode) entry is 2 bytes.

This approach allows to achieve a high accuracy approximation of the weight function W(x,Qp, mode), as shown in Figure 4 below:

Figure 4. Results of approximation of a weight function W(x,Qp,mode) with proposed 10-ranges PWL model

In the test CE14.2.b, a further reduced complexity PWL is utilized to reconstruct the family functions W(x,Qp, mode). Total number of two ranges are defied for PWL, with R[0] value separating two ranges of the PWL are selected to be power of 2, thus can be represented with R[0] = 1<< rShift. Under these constrains a reduced complexity derivation process can be employed.

The variable idxRQP[6] = 18+(1<<i) with i=0..5 establish the dependency between QP value and topShift value employed by the PWL model.

Reduced parameters set provided to the CU level for w(x) derivation consists of rShift, W[2], d[2], totally of 5 16 bits entries is derived as following:

sqp = Sqp[QP];sm = Sm[mode];
rShift = deriveR0 (idxRQP_minus18, QP); 
R[0] = 1 << rShift;
W[0] = W0[0]*sm >> 15;
W[1] = deriveW1(W0[10],sqp) * sm >> 15;							(2)

d[0] = (W[1] - W[0]))*sqp >> 17

d[1] = (-W[1]))*sqp >> 17

The function deriveRS() utilizes 5 comparators for QP against the idxRPQ entries and returns idx of idxRPQ. The function deriveW1() employs 9 parallel comparators to interpolation W1 value from W0[10]

Describe above derivation are invoked once per sequence for each CU type and results in 5 bytes of data for each. Due to this, complexity of this derivation is not included in the complexity analysis provided below.

During the filtering process at CU elvel, for each x value, with x > 0 representing absolute difference of intensity values x=Pk, 0-P0, 0, the w(x) can be derived as following:

R[1] = R[0];

R[0] = 0;

idx1 = (bool) (x >> rShift);

idx2 = x – R[idx1];												(3)

w(x)= max(0, W[idx1] + d[idx1] * idx2 );

Proposed PWL model with adaptive range sizes allows to achieve more coarse, however still sufficiently accurate approximation of the weighting function W(x), as shown in Figure 5 below:

Figure 4. Results of approximation of a weight function W(x,Qp,mode) with proposed 2-ranges PWL model

# Proposal

Proposed in JVET-J406 parametrized bilateral filter was implemented with a certain constrains for inter-coded pictures, namely CU sizes of 4x4, and blocks with minimal side of being large than 8 were omitted from filtering. This effectively limited applicability of the filter to inter coded CUs sizes of smaller than 8xN, where 4<N<64. It is proposed to modify latter constrain and extend applicability of the parametrized BIF to larger CU sizes in inter coded pictures.

Namely, the following change was tested:

#if TEST14_2_c

if( cs.sps->getSpsNext().getUseBIF() && isLuma(compID) && (currTU.cu->qp > 17) && (32 > std::min(currTU.lumaSize().width, currTU.lumaSize().height) ) )

#else

if( cs.sps->getSpsNext().getUseBIF() && isLuma(compID) && (currTU.cu->qp > 17) && (16 > std::min(currTU.lumaSize().width, currTU.lumaSize().height) ) )

#endif

#if !TEST14_2_c

const uint16_t sc[5] = { 30195, 19316, 26068, 19316, 1470 }; // Default table of CE14.2.b

#else

const uint16_t sc[5] = { 30195, 19316, 26068, 19316, 19316}; // Table close to CE14.1.b

#endif

# Experimental Results

The CE14.2.c test was implemented on top of PWL2 design of CE14.b that was presented in JVET-L0406. Simulation results for a short length test shown in Tables 1 and 2 demonstrates that CE14.2.c provide a noticeable compression efficiency improvement over CE14.2.b for higher resolution test sequence.

Complete simulation results over VTM2.0.1 anchor is provided in Table 3 and 4.

## Sub test CE14.2.c over CE14.2.b

Table 1. CE14.2.c simulation results over short test set comparing to CE14.2.b

## Sub test CE14.2.c over VTM2.0.1 CTC anchor

Table 2. CE14.2.c simulation results comparing to VTM2.01 CTC

| CE14.2.c | Over VTM2.01 CTC |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  |  | U |  |  | V |  | EncT |  |  | DecT |  |  |  | Y |  | U |  | V |  | EncT |  |  |  |  |  | DecT |
| Class A1 |  | -0.16% |  |  | 0.12% |  | 0.11% |  | 109% |  |  | 114% |  |  | -0.42% |  |  |  | -0.21% |  | -0.29% |  | 107% |  |  | 109% |  |  |  |
| Class A2 |  | -0.28% |  |  | 0.14% |  |  | 0.15% |  | 106% |  |  | 103% |  |  |  | -0.75% |  | -0.50% |  | -0.36% |  | 104% |  |  | 105% |  |  |  |
| Class B |  | -0.35% |  | 0.18% |  | 0.23% |  |  |  |  | 108% |  |  | 108% |  |  |  | -0.80% |  | -0.22% |  | -0.12% |  | 107% |  |  | 110% |  |  |
| Class C |  | -0.77% |  | 0.16% |  | 0.22% |  |  |  |  | 109% |  |  | 110% |  |  |  | -0.80% |  | 0.03% |  | 0.02% |  | 106% |  |  | 114% |  |  |
| Class E |  | -0.51% |  | -0.02% |  | 0.01% |  |  |  |  | 108% |  |  | 111% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | -0.42% |  |  | 0.13% |  |  | 0.16% |  | 108% |  |  | 109% |  |  |  | -0.71% |  | -0.21% |  | -0.17% |  | 106% |  |  | 110% |  |  |  |
| Class D |  | -0.57% |  | 0.33% |  | -0.09% |  |  |  |  | 107% |  |  | 109% |  |  |  | -0.73% |  | 0.05% |  | 0.18% |  | 106% |  |  | 122% |  |  |

| CE14.2.c | Over VTM2.01 CTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Low delay B Main10 |  |  |  |  | Low delay P Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  | -0.88% |  | 0.66% |  | -0.32% |  | 105% |  | 112% |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.88% | 1.08% | -0.01% | 107% | 126% |  |  |  |  |  |

# Conclusion

This contribution report additional results of CE14.2 test on parametrized in-loop Bilateral Filtering (BIF) presented in JVET-L0406. The applicability of the bilateral filter parametrized with PWL2 was extended to large block sizes for inter coded blocks. Test CE14.2c reports extension to 16x64 and 64x16 block sizes, which is aligned with methods tested in CE14.1 and CE14.3. Complexity estimates provided in JVET-L0406 for the test CE14.2b are also applicable for the current test.

It is proposed to consider provided results for evaluation of coding tools tested in CE14.

# Patent rights declaration(s)

Qualcomm Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “Description of Core Experiment 11 (CE14): Deblocking”, JVET-K1034.

- “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions”, 10th JVET Meeting, San Diego, California, USA, April 2018, JVET-J0021.

- “CE2.1.3: In-loop bilateral filter”, 11th JVET Meeting, Ljubljana, Slovenia, July 2018, JVET-K0384.

- “JVET common test conditions and software reference configurations for SDR video”, JVET-K1010.