| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0627 |
| --- | --- |

| Title: | Crosscheck of JVET-L0291 (CE3-Related: Extended reference sample construction for longer interpolation filter in intra prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vasily Rufitskiy, Alexey Filippov, Jianle Chen | Email: | {vasily.rufitskiy, alexey.filippov, jianle.chen} @huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This document reports the cross-check results for the proposal reported in input document JVET-L0291 “CE3-Related : Extended reference sample construction for longer interpolation filter in intra prediction”. The source code was provided by LGE. Reported Bd-rate results match closely with the ones provided by the proponents.

# Introduction

The tests were performed with BMS-2.0.1 in VTM configuration, following JVET common test condition [1].

# Experimental results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-2.0.1 (tag) with vtm config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.18% | -0.16% | 103% | 98% |
| Class A2 | -0.14% | -0.03% | -0.12% | 103% | 100% |
| Class B | -0.40% | -0.42% | -0.36% | 103% | 101% |
| Class C | -0.64% | -0.82% | -0.78% | 101% | 102% |
| Class E | -0.57% | -0.66% | -0.69% | 102% | 100% |
| Overall | -0.40% | -0.44% | -0.43% | 102% | 100% |
| Class D | -0.52% | -0.39% | -0.60% | 101% | 103% |
| Class F (optional) | -0.19% | -0.50% | -0.38% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-2.0.1 (tag) with vtm config |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.19% | -0.08% | 100% | 102% |
| Class A2 | -0.07% | -0.01% | -0.04% | 101% | 103% |
| Class B | -0.20% | -0.16% | -0.03% | 100% | 100% |
| Class C | -0.26% | -0.19% | -0.18% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.17% | -0.14% | -0.08% | 101% | 102% |
| Class D | -0.27% | -0.40% | -0.16% | 101% | 103% |
| Class F (optional) | 0.06% | 0.07% | 0.05% | 101% | 101% |

# Conclusion

The results obtained in the cross-check match the results presented in JVET-L0291 (VTM-2.0.1 anchor).

# Reference

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-K1010 Ljubljana, SI, July 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd does not have any current or pending patent rights relating to the technology described in this contribution.