| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0677_v3 |
| --- | --- |

| Title: | CE14 related: Decoder run time analysis non-SIMD and SIMD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jacob Ström Per Wennersten Jack Enhorn Du Liu Kenneth Andersson Rickard Sjöberg Torshamnsgatan 23 164 80 Stockholm Sweden | Tel: Email: | +46702672192 {jacob.strom, per.wennersten, jack.enhorn, du.liu, kenneth.r.andersson, rickard.sjoberg}@ ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This proposal compares decoder run times for AI for CE14.1a and CE14.3b for both non-SIMD and SIMD implementations. It is stated that non-SIMD run times are 105% for CE14.1a and 110% for CE14.3b. It is further stated that SIMD run times are 102% for CE14.1a and 106% for CE14.3b. Later, the proponents of CE14.3b have updated their AI decoder run time to 104%. In version 3 of this contribution, the AI run time for CE14.3b has been cross-checked, and it is stated that this results in a figure (105%) similar to that given by the proponents of CE14.3b. The authors claim that the difference in SIMD run times for AI is a factor of 2 between 14.1a and 14.3b. The third version of this contribution also includes random access run time measurements. For CE14.1a, the SIMD run time is stated to be 100.3% using ten runs. For CE14.3b, the SIMD run time is stated to be 101.3%. using nine runs.

# Introduction

At the Ljubljana meeting it was discussed that it was a major problem for the bilateral filter JVET-K0274 that the LUT was too big to be implemented in software. The scalar implementation was not deemed fast enough. CE14.1a solves this problem by removing the need for a LUT.

At this meeting, CE14.3b is presented, and it has a reduced LUT of size 16. While this in its own right does not preclude a fast LUT operation, there are other problems with CE14.3b that makes it hard to make a fast SIMD implementation, namely:

- A check needs to be done to handle the fact that LUT is bypassed for x>threshold

- A rounding and shift operation needs to be done before the LUT itself

- If the value is negative, the output of the LUT should be negated

In addition, three such look-up operations need to be done per pixel. In summary, we believe it is very hard to make a fast SIMD implementation of CE14.3b. This is also carried out in practice; the proponents of CE14.3b report that the decoder run time for their AI implementation is 106%. This is higher than the scalar implementation of 14.1c (105%) which was deemed too slow at the previous meeting.

In this contribution we also present decoder run times for a SIMD version of CE14.1a for AI. The result is 102% decoder run time. This is significantly faster than CE14.b by a factor of two, even after the updated SIMD figure of 104% from the proponents of CE14.b.

# Experiment AI

We carried out 10 decoder runs with our SIMD implementation for the all intra case. The result is shown below:

| try | 1 | 102.54% |
| --- | --- | --- |
| try | 2 | 102.43% |
| try | 3 | 102.17% |
| try | 4 | 101.74% |
| try | 5 | 101.73% |
| try | 6 | 101.66% |
| try | 7 | 101.68% |
| try | 8 | 101.89% |
| try | 9 | 101.87% |
| try | 10 | 101.59% |
|  | average | 101.93% |

The xls files for the different runs along with the SIMD software is included in the contribution. In the first version of this document we erroneously compared against a BMS anchor. In the second version this has been corrected. This has changed the estimated decoder run time from 101.4% to 101.9%. This is the reason for the updated SIMD run times.

We also conducted 10 experiments with the SIMD code for 14.3b:

|  |  | AI |
| --- | --- | --- |
| try | 1 | 104.9% |
| try | 2 | 104.2% |
| try | 3 | 104.9% |
| try | 4 | 104.6% |
| try | 5 | 104.6% |
| try | 6 | 104.7% |
| try | 7 | 104.4% |
| try | 8 | 105.3% |
| try | 9 | 104.8% |
| try | 10 | 105.2% |
|  | average | 104.8% |

This result shows that there is a clear difference between the two run times, at around a factor of two. It also demonstrates that the SIMD implementation of 14.3b is about as fast as the scalar implementation of 14.1a, which had a reported to run of 105%.

# Experiment Random Access

We conducted 10 experiments with our SIMD implementation for the random access case. The result is shown below:

|  |  | 14.1a | 14.3b |
| --- | --- | --- | --- |
|  |  | RA | RA |
| try | 1 | 101.0% | 101.46% |
| try | 2 | 100.9% | 100.19% |
| try | 3 | 100.1% | 102.77% |
| try | 4 | 99.2% |  |
| try | 5 | 100.5% | 100.74% |
| try | 6 | 99.7% | 102.81% |
| try | 7 | 100.7% | 101.62% |
| try | 8 | 101.0% | 101.09% |
| try | 9 | 100.4% | 100.23% |
| try | 10 | 99.8% | 100.74% |
|  | average | 100.33% | 101.29% |

The fourth run for 14.3b was never started by mistake. As can be seen in the image, there is a substantial difference in run time also here. It should be noted that a run times of under 100% are not impossible, even though it involves more filtering than the anchor. The reason is that the reduced ringing can make the RD optimization choose a larger block size. This effect has been noticed for class D even for scalar versions of the code, as seen in the result section of JVET-L0172.

# Methodology

Both the anchor and test sequence are run on the same machine. Each try involves decoding of all the binaries in the test set. For each sequence in a try, a pseudo random number determines if the anchor sequence or test sequence should be run first.

# Conclusion

The run times can be summarized in the following table:

|  |  | decoder run time |  |  |
| --- | --- | --- | --- | --- |
|  |  | AI | AI |  |
|  |  | not SIMD | SIMD |  |
| CE14.1a |  | 105% | 102% |  |
| CE14.1b |  | 103% |  |  |
| CE14.1c |  | 104% |  | <--- was rejected since not possible to speed up with SIMD |
| CE14.3a |  | 110% |  |  |
| CE14.3b |  | 110% | 104% |  |

In conclusion we claim that the decoder run time is prohibitive for CE14.3b but satisfactory for CE14.1a. We also note that there is a factor of four in run time increase between the two SIMD versions.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).