| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0682-v2 |
| --- | --- |

| Title: | CE6-related: Low-complexity approximations with 8-bit Transform Adjustment Filters (TAF) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Yung-Hsuan Chao, Amir Said, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports results for an 8-bit version of the unified transformation approach approximating the DCT-8 and DST-7 in CE6.1.6, where each transform is approximated with an adjustment filter as pre-processing and inverse of DCT-2.

The results show less than 0.xx% coding gain loss in BD rate as against the VTM-2.0.1 anchor.

# Introduction

In VTM 2.0.1, pairs of DCT-2, DST-7 or DCT-8 are used for separable transformation of residual block, where DCT-2 is used up to size 64x64 and DST-7 and DCT-8 are allowed up to size 32x32. Supporting multiple transforms has several consequences related to complexity and computational costs. For instance, not all transforms can be implemented efficiently for all dimensions, and they may require different types of hardware designs. Specifically, DCT-2 has efficient hardware implementations (partial-butterfly structures, etc.), so that it is feasible to use DCT-2 for large blocks. However, DST-7 and DCT-8 do not have such efficient implementations and supporting DCT-8 and DST-7 for large sizes (e.g., 64x64) may be quite costly. This proposal addresses these problems by using a unified (i.e., a single type of) transform implementation and additional processing filters, called transform adjustment filters (TAFs).

# 2 Proposed method

The proposed method is an 8-bit extension of the TAF method tested in CE6.1.6 where the following changes are made.

- All existing transforms are switched to the 8-bit precision as proposed in JVET-L0285.

- New TAF tables with 8-bit precision is used instead of the 9-bit TAF parameters in JVET-L0386 (CE6.1.6).

# Experimental Results

The proposed method is implemented based on BMS-2.0.1 software and tested under CTC [3] (i.e., VTM 2.0.1 configurations). In addition, it is tested by enabling inter MTS (Inter AMT) via EMT=3 and EMTFast =3 on top of CTC (VTM 2.0.1 + EMT=3).

Table 4 presents results of proposed method used to implement DCT-8 and DST-7 for 16x16 and 32x32. Note that the anchor is using 10bit transforms and the tested method use only 8-bit transforms.

Table 4: Results of the proposed method (8-bit) used to implement DST-7 and DCT-8 for sizes 16x16 and 32x32.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test: L0682 (8-bit) Over VTM-2.0.1 (10-bit) |  |  |  |  | Test: L0682+ EMT=3 (8-bit) Over VTM-2.0.1+ EMT=3 (10-bit) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.06% | 0.03% | 98% | 86% | 0.07% | 0.06% | 0.03% | 98% | 86% |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.11% | 0.09% | 0.14% | 98% | 88% | 0.11% | 0.09% | 0.14% | 98% | 88% |
| Class C | 0.05% | 0.00% | 0.21% | 99% | 97% | 0.05% | 0.00% | 0.21% | 99% | 97% |
| Class E | 0.08% | 0.12% | -0.06% | 98% | 93% | 0.08% | 0.12% | -0.06% | 98% | 93% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.05% | 0.11% | -0.15% | 99% | 96% | 0.05% | 0.11% | -0.15% | 99% | 96% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Test: L0682 (8-bit) Over VTM-2.0.1 (10-bit) |  |  |  |  | Test: L0682+ EMT=3 (8-bit) Over VTM-2.0.1+ EMT=3 (10-bit) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.09% | 0.04% | 0.08% | 100% | 99% | 0.10% | -0.12% | 0.09% | 96% | 94% |
| Class C | 0.07% | 0.21% | 0.12% | 99% | 98% | 0.03% | 0.11% | 0.04% | 98% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.05% | -0.14% | 0.49% | 100% | 101% | 0.00% | -0.21% | 0.47% | 99% | 99% |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |
|  | Test: L0682 (8-bit) Over VTM-2.0.1 (10-bit) |  |  |  |  | Test: L0682+ EMT=3 (8-bit) Over VTM-2.0.1+ EMT=3 (10-bit) |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | 0.08% | 0.14% | -0.17% | 101% | 101% | 0.00% | -0.02% | 0.16% | 98% | 100% |
| Class E | -0.05% | 0.64% | 0.40% | 101% | 100% | -0.05% | 0.26% | 0.47% | 98% | 100% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.05% | 0.41% | -0.37% | 100% | 100% | 0.01% | 0.39% | 0.26% | 99% | 102% |

# References

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said and X. Zhao, “Core Experiment 6: Arithmetic Coding Engine,” JVET-J1026, 9th meeting, San Diego, CA, USA, April 2018.

- F. Bossen, J.Boyce, X. Li, V. Seregin and K. Suehring,  “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, 11th meeting, Ljubljana, Slovenia, July 2018.

- A. Said, H. Egilmez, V. Seregin, and M. Karczewicz, “Complexity Reduction for Adaptive Multiple Transforms (AMT) using Adjustment Stages” 9th meeting, San Diego, CA, USA, April 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).