| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0692-v1 |
| --- | --- |

| Title: | BoG report on CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 647 745 1691 frank@bossentech.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG on CABAC met on Sunday Oct 7, 2018 between 4:30pm and 6pm.

The topic of discussion was defined as follows in the JVET-L meeting notes:

Considering the fact that the total memory even in worst case is less than one line buffer of a video, memory is asserted to be not a critical issue here.

Throughput (pipelining, number of cycles) could be a more critical issue. The probability estimate is probably OK, but potentially multiple context models, and customized window could cause problems. More analysis on this is needed. BoG (F. Bossen, M. Zhou) to look into this.

Suggestions from the BoG include:

- Further study the issue of CABAC complexity until the 13th JVET meeting

- For HW, do in-depth analysis on paper (two companies volunteered)

- For SW, set up a test framework that can be used to measure throughput (two companies volunteered)

# Notes

Subrange computation (from CE5.2/5.3)

AVC/HEVC: lookup table 64x4x8 bit

Options:

- [Add/xor] + table lookup 32x8x8 bit (or other)

- [Add/xor] + multiplier 5x4 (and upwards)

- [Add/xor] + table lookup (32x7 or 16x7 bit) + multiplier (7x4)

- [Add/xor] + lzcnt + multiplier 5x5 (and upwards)

- [Add/xor] + lzcnt + lookup 8x8x8 bit

All solutions can be implemented using lookup table, except CE5.2.4 (also needs lzcnt).

Each solution may require a specific implementation when using a multiplier (except CE5.2.3 for which multiplication-based implementation may not be possible)

Probability estimation (from CE5.1)

AVC/HEVC: lookup table 64x2x7 bits

Options:

- Add + fixed shift (x2)

- Add + fixed shift (x2) + counter

- Add + variable shift (x1)

- Add + variable shift (x2)

- Table lookup (32x8 bits) + fixed shift + add (x2)

- Table lookup (32x8 bits) + variable shift + add (x1)

- Table lookup (32x8 bits) + variable shift + add (x2)

Note: for case with table lookup, table encodes a piece-wise linear function.

How to determine throughput?

HW

Some analysis in JVET-L0094 (TSMC 12nm)

All CE5.2 experiments (range computation) were synthesized.

Multiplication requires smaller area, but LUT has shorter critical path.

Note: it was mentioned that critical path contains initial xor operation.

No results for HEVC (but could be provided).

Since most (or all) CE5.2 and CE5.3 proposals can be implemented using a lookup table, all proposals seem adequate for HW implementation.

Is it realistic to do synthesis exercise for entire engine? Probably not.

Should do paper analysis by next meeting to determine throughput estimate. Volunteers: HHI, Qualcomm

SW

More realistic (than for HW) to implement a fast version of entire engine.

Suggestion:

Measure run time in a test framework using artificial bit streams. May encode/decode either using single context, or using multiple contexts.

Looking at decoder should be sufficient, as there are fewer feedback loops in encoder.

Open question: how to cross-check optimized implementations?

Volunteers: Sharp, HHI