| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0694-v3 |
| --- | --- |

| Title: | CE4-related: Combination of affine mode clean up and line buffer reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen Han Huang Wei-Jung Chien | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com hanhuang@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Huawei Technologies. Co., Ltd, Qualcomm |  |  |

_____________________________

- Abstract

In this contribution, combination of affine mode clean up (JVET-L0047 [2] method 1) and line buffer reduction (JVET-L0045 [3]) with the modification of using the sub-block vectors in the line buffer to do affine inheritance is proposed. It’s proposed to use the same sub-block motion vector field for the motion compensation, merge/skip/AMVP list derivation, de-blocking, and storage of the temporal motion vector predictors (TMVPs). The control point motion vectors (CPMVs) are used for the affine motion data inheritance (i.e. for merge mode and affine AMVP mode) if the candidate block is in the current CTU. However, the bottom-left and bottom-right sub-block motion vectors are used for the affine motion data inheritance if the candidate block is in the above CTU. Therefore, no additional line buffer is required to store the CPMVs. It’s also proposed to use sub-block MV at bottom-left and bottom-right location for affine data inheritance from above CTU without division by setting the coordinates of bottom-left and bottom-right corner are set to (xNb, yNb+neiH) and (xNb+neiW, yNb+neiH) for inheritance.

The BD-rate differences are 0.01% in RA, -0.0x% in LB on top of VTM-2.0.1 plus already adopted affine proposals.

Problem statements

During JVET L meeting, 2 proposals for affine mode were adopted:

- Line buffer reduction for affine mode (JVET-L0045). The basic idea in JVET-L0045 is to use the 4-parameter affine motion model, the bottom-left and bottom-right control point motion vectors (CPMVs) to do affine motion data inheritance for the current CU, if the (source) neighboring CU is located at the top CTU boundary regardless whether it uses the 4- or 6-parameter model. The bottom-left and bottom-right CPMVs are stored in the regular motion data line buffer and are shared for the merge/skip and AMVP list derivations, to avoid the need of a separate line buffer to store the control point motion vectors.

- Affine mode clean-up (method 1 of JVET-L0047). The CPMVs are stored in a separate buffer. The CPMVs are only used in the affine motion data inheritance. The sub-block MVs are used for motion compensation, merge/skip/AMVP list derivation, de-blocking, and storage of the temporal motion vector predictors. And the sub-block based motion compensation for ATMVP and affine mode are aligned.

The cleanup principle in JVET-L0047 cannot be achieved if these two proposals are straightforwardly combined. In such case, the MVs in regular motion data line buffer will be unaligned with sub-block MVs in bottom CTU boundary, because the bottom-left and bottom-right CPMVs (derived at the bottom-left and bottom-right locations of the CU) are stored in the regular motion data line buffer. It requires conditional check in the specification to select different motion field according to the position of neighboring block in merge/skip/AMVP list derivations.

Figure 1: Illustration of motion vector usage for directory combined method

As shown in Figure 1, along the top CTU boundary, the bottom-left and bottom right corner control point motion vectors of a CU are not used for motion compensation. Instead, the sub-block vectors are generated (but not stored in the line buffer) and used for motion compensation. For example in CU E of Figure 1,  the bottom-left and bottom right corner CPMVs VLE0 and VLE1 (marked in green) of CU E are stored in the line buffer, and used for the affine inheritance, merge/skip/AMVP list derivation of neighboring CUs in bottom CTUs, but not used for MC/TMVP/de-blocking. Instead, the separate sub-block vectors (marked in brown in Figure 1) are generated for the corner sub-blocks and used for MC/TMVP/de-blocking and merge/skip/AMVP list derivation of neighboring CUs except CUs in bottom CTUs.

Proposal combination

In this contribution, the following are proposed:

- Affine mode clean-up (method 1 of JVET-L0047). The CPMVs are stored in a separate buffer. The CPMVs are only used in the affine motion data inheritance. The sub-block MVs are used for motion compensation, merge/skip/AMVP list derivation, de-blocking, and storage of the temporal motion vector predictors. And the sub-block based motion compensation for ATMVP and affine mode are aligned.

- Affine motion data inheritance from above CTU. If the candidate CU for affine motion data inheritance is in the above CTU line, the bottom-left and bottom-right sub-block MVs in the line buffer instead of the CPMVs are used for the affine MVP derivation. In this way, the CPMVs are only stored in local buffer. If the candidate CU is 6-parameter affine coded, the affine model is degraded to 4-parameter model.

- Since the sub-block MV represent the motion at the center of the sub-block, the distance of two corner sub-block MVs at the bottom of a candidate PU is neiW-4, wherein neiW is the width of the candidate PU. To avoid division by (neiW-4), which may not be power of 2, the rough distance neiW is used for inheritance. The coordinates of bottom-left and bottom-right corner are set to (xNb, yNb+neiH) and (xNb+neiW, yNb+neiH) for inheritance.

As shown in Figure 2, along the top CTU boundary, the bottom-left and bottom right sub-block motion vectors of a CU are used for everything and stored in line buffer. These sub-block MVs are also used for affine inheritance of neighboring affine CUs in bottom CTUs. For example in CU E of Figure 1,  the bottom-left and bottom right corner sub-block MV VLE0 and VLE1 (marked in brown) of CU E are stored in the line buffer, and used for the affine inheritance, merge/skip/AMVP/TMVP list derivation of neighboring CUs in bottom CTUs and de-blocking.

Figure 2: Illustration of motion vector usage for proposed combined method

The control point vectors v0 and v1 of the current CU are derived by using the 4-parameter model, and by

v0x=vLE1x-vLE0x(xLE1-xLE0)(x0-xLE0)-vLE1y-vLE0y(xLE1-xLE0)(y0-yLE0)+vLE0xv0y=vLE1y-vLE0y(xLE1-xLE0)(x0-xLE0)+vLE1x-vLE0x(xLE1-xLE0)(y0-yLE0)+vLE0y               Equation 1

v1x=vLE1x-vLE0x(xLE1-xLE0)(x1-xLE0)-vLE1y-vLE0y(xLE1-xLE0)(y1-yLE0)+vLE0xv1y=vLE1y-vLE0y(xLE1-xLE0)(x1-xLE0)+vLE1x-vLE0x(xLE1-xLE0)(y1-yLE0)+vLE0y               Equation 2

And if the current CU uses the 6-parameter affine motion model, the control point vectors v2 is derived by

v2x=vLE1x-vLE0x(xLE1-xLE0)(x2-xLE0)-vLE1y-vLE0y(xLE1-xLE0)(y2-yLE0)+vLE0xv2y=vLE1y-vLE0y(xLE1-xLE0)(x2-xLE0)+vLE1x-vLE0x(xLE1-xLE0)(y2-yLE0)+vLE0y               Equation 3

The coordinates of the bottom-left and bottom-right sub-blocks satisfy the following conditions:

xLE0=xE0xLE1=xE1yLE0=y0yLE1=y1                                     Equation 4

Simulation results

The contribution has been implemented on top of VTM-2.0.1 plus already adopted affine proposals, including JVET-L0045 (CE4: Test results of CE4.1.11 on line buffer reduction for affine mode), JVET-L0271 (CE4.1.6: Simplification of affine AMVP candidate list construction), JVET-L0142 (CE4: Simplification of the common base for affine merge (Test 4.2.6)), JVET-L0632 (Crosscheck of JVET-L0142 (CE4: Simplification of the common base for affine merge (Test 4.2.6))) and JVET-L0369 (CE4: Separate list for sub-block merge candidates (Test 4.2.8)). The test condition descripted in JVET-K1024 [1] is used for verifying the contribution. The summary results are provided in Table 1. On average, the BD-rate differences are 0.01% in RA, and 0.0x% in LB, respectively.

Table 1 Coding performance over VTM-2.0.1 plus already adopted affine proposals

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | -0.06% | 101% | 102% |
| Class A2 | 0.01% | 0.06% | 0.01% | 101% | 106% |
| Class B | 0.01% | 0.02% | -0.01% | 100% | 103% |
| Class C | 0.00% | -0.07% | -0.17% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | -0.06% | 100% | 103% |
| Class D | 0.02% | -0.14% | 0.10% | 100% | 105% |
| Class F (optional) | 0.00% | -0.03% | -0.06% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.03% | -0.26% | 0.02% | 101% | 104% |
| Class E | -0.17% | -0.25% | -0.23% | 101% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.06% | 0.15% | -0.31% | 100% | 102% |
| Class F (optional) | 0.11% | -0.22% | 0.40% | 101% | 100% |

References

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-K1024, the 11th JVET meeting, July 2018, Ljubljana, SI.

- M. Zhou, B. Heng, “CE4-related: A clean up for affine mode”, JVET-L0047, the 12th JVET meeting, Macao, CN, 3–12 Oct. 2018.

- M. Zhou, “CE4: Test results of CE4.1.11 on line buffer reduction for affine mode”, JVET-L0045, the 12th JVET meeting, Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).