| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0696-v1 |
| --- | --- |

| Title: | Proposed starting point for interoperability point syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill.boyce@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

This contribution proposes a simple starting point for the interoperability point syntax for VTM-2, based upon the agreements of design principles during the review of contributions JVET-L0042, JVET-L0043, JVET-L0044, and JVET-L0270.

# Background

During the review of contributions JVET-L0042, JVET-L0043, JVET-L0044, and JVET-L0270, the following design principles were agreed to, as captured in JVET-L_notes-d6.doc

Decisions on agreements in principle:

- It was suggested to agree in principle, as a starting point, to have something at the start of the SPS that indicates properties that cannot be violated in the entire bitstream. Agreed.

- It was suggested to start with presumption that there would be a list of disable flags. Agreed.

- It was suggest to agree in principal that there would a separation between things that affect the syntax or decoding process and things that merely express constraints. Agreed.

# Proposal

This proposal is based on contribution JVET-L0044, but modified based on the agreements listed above. The proposed constraint flags/syntax elements are placed at the beginning of the SPS. All of constrain syntax elements are fixed length coded.

The HLS sections of the VTM-2 document do not currently match the VTM-2 software. Unlike in JVET-L0044, in this proposal, the existing HLS tool enable flags are not conditioned based on these proposed constraint syntax element values, but are unmodified from the existing design (which is not well documented), where there are tool enable flags later in the SPS or in the slice header.

It was suggested in the design principles to separate constraints that would affect the decoding process and things that merely express constraints. In this proposal, there are no constraint syntax elements that would directly affect the decoding process. However, the existing HLS enable flags that correspond to the constraint syntax elements would impact the decoding process. The constraint syntax elements would merely restrict allowable values of the existing HLS enable flags.

18 constraint syntax elements are proposed, using 22 bits.

## Proposed syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| intra_only_constraint_flag | u(1) |
| max_bitdepth_constraint_idc | u(4) |
| max_chroma_format_constraint_idc | u(2) |
| frame_only_constraint_flag | u(1) |
| no_adaptive_loop_filter_constraint_flag | u(1) |
| no_sample_adaptive_offest_constraint_flag | u(1) |
| no_chroma_separate_tree_constraint_flag | u(1) |
| no_dep_quant_constraint_flag | u(1) |
| no_sign_data_hiding_constraint_flag | u(1) |
| no_cclm_constraint_flag | u(1) |
| no_extra_intra_modes_constraint_flag | u(1) |
| no_wide_intra_angles_constraint_flag | u(1) |
| no_multi_trans_constraint_flag | u(1) |
| no_affine_motion_constraint_flag | u(1) |
| no_high_precision_mvs_constraint_flag | u(1) |
| no_temporal_mvp_constraint_flag | u(1) |
| no_sbtmvp_constraint_flag | u(1) |
| no_adaptive_mv_res_constraint_flag | u(1) |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| qtbtt_dual_tree_intra_flag | ue(v) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_qt_size_intra_slices_minus2 | ue(v) |
| log2_min_qt_size_inter_slices_minus2 | ue(v) |
| max_mtt_hierarchy_depth_inter_slices | ue(v) |
| max_mtt_hierarchy_depth_intra_slices | ue(v) |
| sps_cclm_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |
| if( sps_sbtmvp_enabled_flag ) |  |
| log2_sbtmvp_default_size_minus2 | u(1) |
| sps_amvr_enabled_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) |  |
| sps_affine_type_flag | u(1) |
| sps_mts_intra_enabled_flag | u(1) |
| sps_mts_inter_enabled_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Patent rights declaration(s)

Intel does not have any current or pending patent rights relating to the technology described in this contribution.