| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0702 |
| --- | --- |

| Title: | Twitch Class F test sequence |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tarek AMARA | Tel: Email: | N/A amatarek@twitch.tv |
| Source: | Twitch |  |  |

_____________________________

Abstract

A new test sequence is presented to JVET for inclusion into the Class F test set. The content is a screen content (eSport game) uncompressed in yuv420 8 bits mode, with a 1080p60 resolution.

The sequence is offered free of charge and under a copyright license suitable for use in standardization projects.

A set of eSport clips were looked at when BoG met on 14 July 2018 (JVET-K0541v1) and the outcome is the current contribution.

- Introduction

Current Class F test sequences is outdated ( low resolution, low complexity) and misses representation for a growing video segment, namely eSport / Gaming.

The clips proposed is of 10 seconds length without scene changes and with good level of details and a characteristics

- Sequence description

Twitch is proposing the addition of a new e-sport sequence to the Class F test set.  The sequence is  1080p60 YUV 4:2:0 and BT.709 color primaries.  The sequence have a wide set of characteristics including camera panning, high texture as well as sharp objects and edges.

| Sequence | Frame rate | Number of frames |
| --- | --- | --- |
| Twitch_EurotruckSimulator2_1920x1080_60_8bit_420.yuv | 60 fps | 600 |

- Test sequence analysis

The proposed clip is encoded using HM16.6 , Random Access configuration  at the following QP points: 25, 35, 42 and 47. Intra period was set to 64. The table below summarizes the QP/Bitrate/PSNR results. the Rate-distortion curve shows no anomalies or  outliers in the rate / distortion relationship.

| QP | Bitrate | Y-PSNR | U-PSNR | V-PSNR | YUV-PSNR |
| --- | --- | --- | --- | --- | --- |
| 25 | 35776.0984 | 35.2235 | 41.4845 | 44.2223 | 36.3128 |
| 35 | 4736.1296 | 29.9845 | 38.1546 | 41.0697 | 31.4249 |
| 42 | 1190.8984 | 27.2587 | 36.9181 | 39.8065 | 28.8148 |
| 47 | 428.8016 | 25.5007 | 35.3988 | 38.0947 | 27.0744 |