| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0025-v3 |
| --- | --- |

| Title: | CE5: Summary report on the Arithmetic Coding Engine |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Heiner Kirchhoffer Fraunhofer HHI Amir Said Qualcomm Inc. | Email: | heiner.kirchhoffer@ hhi.fraunhofer.de asaid@qti.qualcomm.com |
| Source: | CE5 coordinators |  |  |

_____________________________

# Abstract

This is the summary report of Core Experiment 5 (CE5) on the Arithmetic Coding Engine. Subtest 1 (experiments CE5.1.1 – 5.1.13) addresses the effect on the compression efficiency of training parameters of the probability estimation stage of the proposed arithmetic coding engines. Subtest 2 addresses the achievable throughput of the different coding engines of subtest 1 in an optimized software implementation. Subtest 3 provides a summary of the analysis of hardware implementation aspects.

# Test conditions and guidelines

The experiments were implemented in separate branches in the following git repository:

https://vcgit.hhi.fraunhofer.de/JVET-L-CE5/VVCSoftware_VTM.git

For subtest 2, a testbed is available in branch CE5.2, which contains throughput-optimized software implementations of the coding engines of subtest 1.

# Subtest 1

## Experiment list

The following table lists all tests that were performed in this CE and gives a summary of the tools.

Note on the naming convention of CE tests: ‘CE5.x.y.z’ means ‘CE5.x.y’ as specified in the CE5 description with configuration ‘z’ according to the following tables.

| CE5.1 – Probability estimation with retrained parameters |  |  |  |
| --- | --- | --- | --- |
| CE # (Proponent) | Related Docs. | Summary of the tool | Crosscheckers |
| 5.1.1 (HHI) | JVET-M0725 | 2 linear states (10+14 bits), fixed | MediaTek |
| 5.1.2 (HHI) | JVET-M0725 | 2 linear states (10+14 bits), variable | MediaTek |
| 5.1.3 (Qualcomm) | JVET-M0412 | 2 linear states (10+14 bits), variable: Configuration 1: Coding interval subdivision from JVET-L0335 is used Configuration 2: Coding interval subdivision from JVET-L0117 is used | HHI |
| 5.1.4 (Qualcomm) | JVET-M0413 | 1 linear state (14 bits), variable: Configuration 1: Coding interval subdivision from JVET-L0335 is used Configuration 2: Coding interval subdivision from JVET-L0117 is used | Sharp Labs |
| 5.1.5 (HHI) | JVET-M0727 | 2 log. states (8+12 bits), fixed | Sharp Labs |
| 5.1.6 (HHI) | JVET-M0727 | 2 log. states, variable Configuration 1: 8+12 bits Configuration 2: 10+14 bits | Sharp Labs |
| 5.1.7 (HHI) | JVET-M0727 | 1 log. state (12 bits), variable | Sharp Labs |
| 5.1.8 (Samsung) | JVET-M0199 | 2 linear states (15+15 bits), Only the short window size model is updated for the first 31 bins of a context model. | MediaTek |
| 5.1.9 (MediaTek) | JVET-M0172 | 2 linear states (10+14 bits), fixed Multiplier-based coding interval subdivision. Configuration 1: 6-bit by 5-bit multiplier. Configuration 2: 6-bit by 4-bit multiplier. Both configurations can be implemented as 32x8x8 bit LUT. | Samsung |
| 5.1.10 (Sharp Labs) | JVET-M0453 | 2 linear states, fixed Uses 2b−1 instead of 2b in probability update | HHI |
| 5.1.11 (Sharp Labs) | JVET-M0453 | 2 linear states, variable Uses 2b−1 instead of 2b in probability update | HHI |
| 5.1.12 (Sharp Labs) | JVET-M0453 | 1 linear state, variable Uses 2b−1 instead of 2b in probability update | MediaTek |
| 5.1.13 (Sharp Labs) | JVET-M0453 | 2 linear states, variable Uses 2b−1 instead of 2b in probability update Uses 5x4 multiplier for coding interval subdivision | HHI |

Contributions related to CE5.1:

JVET-M0196: CE5-related: Counter-based multi-CABAC for partial context models [Y. Piao, K. Choi (Samsung)]

JVET-M0389: CE5-related: Minor optimizations for increasing the throughput of CE5.1.5 and CE5.1.6 [H. Kirchhoffer, C. Bartnik, P. Haase, T. Hinz, S. Matlage, B. Stabernack, J. Stegemann, D. Marpe, H. Schwarz, T. Wiegand (HHI)]

JVET-M0395: CE5-related: Alternative implementation of CABAC range sub-interval derivation for CE5.1.5, CE5.1.6 and CE5.1.7 [P. Haase, H. Kirchhoffer, S. Matlage, H. Schwarz, D. Marpe, T. Wiegand (HHI)]
JVET-M0453: CE5 on arithmetic coding: experiments 5.1.1, 5.1.2, 5.1.3, 5.1.4, 5.1.5, 5.1.6, 5.1.7, 5.1.8, 5.1.10, 5.1.11, 5.1.12, 5.1.13, 5.2, and more [F. Bossen (Sharp)]

JVET-M0772: CE5-related: Clean up of the context model initialization process for CE5.1.5 and CE5.1.6 [J. Stegemann, H. Kirchhoffer, D. Marpe, H. Schwarz, T. Wiegand (HHI)]

## Summary of the results

Average test results for subtest 1 (relative to VTM-3.0).

|  | AI |  |  |  | RA |  |  |  | LB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE # | Y | U | V | EncT/ DecT (%) | Y | U | V | EncT/ DecT (%) | Y | U | V | EncT/ DecT (%) |
| 5.1.1 | -0,81% | -1,15% | -1,20% | 105/101 | -0,71% | -1,06% | -0,97% | 102/100 | -0,60% | -0,22% | -0,18% | 102/100 |
| 5.1.2 | -0,97% | -1,18% | -1,27% | 107/101 | -0,97% | -0,99% | -0,93% | 103/99 | -0,89% | -0,08% | -0,25% | 102/99 |
| 5.1.3.1 | -0,95% | -1,35% | -1,44% | 107/103 | -0,90% | -1,06% | -0,97% | 105/106 | -0,77% | -0,01% | -0,23% | 106/103 |
| 5.1.3.2 | -0,99% | -1,42% | -1,53% | 107/105 | -0,95% | -1,07% | -1,04% | 106/106 | -0,82% | -0,04% | -0,15% | 107/104 |
| 5.1.4.1 | -0,80% | -1,12% | -1,11% | 108/107 | -0,64% | -0,11% | -0,01% | 107/110 | -0,56% | 1,01% | 1,51% | 107/104 |
| 5.1.4.2 | -0,89% | -1,21% | -1,18% | 108/107 | -0,71% | -0,26% | -0,05% | 108/109 | -0,61% | 1,02% | 1,76% | 107/107 |
| 5.1.5 | -0,76% | -1,47% | -1,41% | 101/98 | -0,63% | -0,79% | -0,70% | 97/93 | -0,68% | -0,29% | 0,08% | 94/91 |
| 5.1.6.1 | -0,83% | -1,51% | -1,53% | 103/98 | -0,76% | -0,77% | -0,75% | 98/93 | -0,76% | -0,44% | -0,15% | 95/92 |
| 5.1.6.2 | -0,91% | -1,44% | -1,44% | 104/97 | -0,81% | -0,92% | -0,85% | 98/93 | -0,72% | -0,42% | -0,05% | 95/92 |
| 5.1.7 | -0,66% | -1,12% | -1,10% | 100/98 | -0,53% | -0,96% | -0,95% | 97/93 | -0,52% | -0,66% | -0,52% | 94/92 |
| 5.1.8 | -0,86% | -1,36% | -1,37% | 106/101 | -0,68% | -0,86% | -0,71% | 102/101 | -0,53% | -0,60% | 0,51% | 99/95 |
| 5.1.9.1 | -0,79% | -1,05% | -1,18% | 105/101 | -0,57% | -1,05% | -1,00% | 102/100 | -0,50% | -0,82% | -0,66% | 103/101 |
| 5.1.9.2 | -0,76% | -1,02% | -1,15% | 105/102 | -0,55% | -1,03% | -0,98% | 102/100 | -0,49% | -0,81% | -0,65% | 102/101 |
| 5.1.10 | -0,79% | -1,10% | -1,16% | 108/104 | -0,67% | -1,06% | -1,02% | 104/102 | -0,54% | -0,38% | -0,78% | 103/101 |
| 5.1.11 | -0,94% | -1,14% | -1,23% | 108/103 | -0,93% | -0,95% | -1,05% | 103/101 | -0,80% | -0,27% | -0,38% | 102/101 |
| 5.1.12 | -0,66% | -0,74% | -0,50% | 108/103 | -0,55% | 0,18% | 0,94% | 102/100 | -0,54% | 0,81% | 2,33% | 101/99 |
| 5.1.13 | -0,93% | -1,13% | -1,21% | 109/103 | -0,91% | -0,93% | -1,03% | 104/101 | -0,78% | -0,24% | -0,35% | 101/98 |
| 2 states variable, 2 states fixed, 1 state variable |  |  |  |  |  |  |  |  |  |  |  |  |

# Subtest 2

Throughput-optimized software implementations of the configurations in CE5.1 are tested in terms of the achievable throughput in the decoder. Two types of bin sequences are used for the evaluation:

- VTM3: Bin sequences extracted from VTM-3.0 CTC bitstreams

- RND:  Randomly generated bin sequences

Three contributions report results for CE5.2:

- JVET-M0453 (Sharp)

- JVET-M0463 (Qualcomm)

- JVET-M0762 (HHI)

Contributions related to CE5.2:

JVET-M0389: CE5-related: Minor optimizations for increasing the throughput of CE5.1.5 and CE5.1.6 [H. Kirchhoffer, C. Bartnik, P. Haase, T. Hinz, S. Matlage, B. Stabernack, J. Stegemann, D. Marpe, H. Schwarz, T. Wiegand (HHI)]

JVET-M0395: CE5-related: Alternative implementation of CABAC range sub-interval derivation for CE5.1.5, CE5.1.6 and CE5.1.7 [P. Haase, H. Kirchhoffer, S. Matlage, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

# Subtest 3: In-depth analysis of hardware implementation aspects

In this subtest, complexity and throughput aspects of a hypothetical hardware implementation of the proposed arithmetic coding engines is investigated.

The test report can be found in JVET-M0759.

Basically, the hardware implementations of the various probability estimators studied in subtest 3 are very simple, so the completion of the operation of decoding one bin should be finished within one clock cycle at a reasonable clock frequency.

## Summary for subtest 3

| JVET Contribution | BIN/Cycl | Impact on throughput compared to ideal reference implementation |
| --- | --- | --- |
| HHI JVET-K0430 | 1 | No impact on throughput. |
| HHI JVET-L0426 | 1 | No impact on throughput. |
| HHI JVET-L0429 | 1 | No impact on throughput. |
| Qualcomm JVET-L0115 | 1 | No impact on throughput. |
| Qualcomm JVET-L0116 | 1 | No impact on throughput. |

# Crosscheck reports

The following table lists the reported crosschecks.

| CE # | Crosschecker | Simulation results match | Implementation matches the description | Comments by the crosscheckers |
| --- | --- | --- | --- | --- |
| CE5.1.1 | MediaTek | Yes, runtime is also similar | Yes | None |
| CE5.1.2 | MediaTek | Yes, runtime is also similar | Yes | None |
| CE5.1.3 | HHI | Yes | Yes | None |
| CE5.1.4 | Sharp Labs | Yes | Yes |  |
| CE5.1.5 | Sharp Labs | Yes for BD rate / No for runtime | Yes | Average 8% difference in encoder run time, 10% difference in decoder run time |
| CE5.1.6 | Sharp Labs | Yes for BD rate / No for runtime (see comment) | Yes (see comment) | Has additional test with 10+14 bit state representation which isn’t described in JVET-L1025 Average 8% difference in encoder run time, 9% difference in decoder run time |
| CE5.1.7 | Sharp Labs | Yes for BD rate / No for runtime (see comment) | Yes | Average 8% difference in encoder run time, 10% difference in decoder run time |
| CE5.1.8 | MediaTek | Yes, runtime is also similar | Yes | None |
| CE5.1.9 | Samsung | Yes |  |  |
| CE5.1.10 | HHI | Yes | Yes | None |
| CE5.1.11 | HHI | Yes | Yes | None |
| CE5.1.12 | MediaTek | Yes, runtime is also similar | Yes | None |
| CE5.1.13 | HHI | Yes | Yes | None |
| CE5.2 | Qualcomm | N/A | Yes | Engines 1 and 3 were crosschecked. |
|  | HHI | N/A | Yes | Engines 2 and 4 were crosschecked. |