| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0060 |
| --- | --- |

| Title: | CE4: Enhanced Merge with MVD (Test 4.4.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonori Hashimoto Eiichi Sasaki Tomohiro Ikai | Email: | +81-43-299-8526 tomonori.hashimoto@sharp.co.jp eiichi.sasaki@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution reports a result of CE4.4.4 enhanced merge with MVD (MMVD). In CE4.4.4, four tests are conducted. Test 4.4.4.a uses diagonal candidate, test 4.4.4.b uses mutiple distance lists, test 4.4.4.c is a combination of test a and test b, and test 4.4.4.d is test c with full-pel position for large distance. It is reported that the bd-rate is -0.28% with 106% encoding time under RA condition and 0.00% with 103% encoding time under LDB condition for test 4.4.4.d. It is proposed to adopt test 4.4.4.d method into the next VVC draft / VTM.

Proposed Methods

Directional candidates

Directional candidates are used in addition to horizontal / vertical candidates of VTM-3.0. The directional candidates are shown as below.

Table 1. Motion direction (proposal)

| Direction IDX | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-axis | +2 | –2 | 0 | 0 | +1 | -1 | -1 | +1 |
| y-axis | 0 | 0 | +2 | –2 | +1 | -1 | +1 | -1 |

Note: the value of x-axis and y-axis of diagonal direction are half of that of horizontal and vertical direction respectively.

Multiple distance lists

Considering the amount of distance (motion difference) would different from areas to pictures to sequences, we propose multiple (i.e. two) set of distance lists. The selection flag is context coded so that it applies in an optimal way.

The distance list is shown as follows.

Table 2. First distance list

| Distance IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 3/4-pel | 5/4-pel |

Table 3. Second distance list

| Distance IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Pixel distance | 1-pel | 2-pel | 4-pel | 8-pel |

Full-pel position for large range

In test d, motion vectors are converted into an integer position if the distance is large. Since integer position doesn’t need additional pixels for interpolation, necessary area size for MMVD encoding / decoding decreases, which would reduce memory band requirement. When the second distance list is selected and distance index is larger than 1, the motion vectors are converted into an integer position.

Experimental Results

The simulations were conducted according to JVET-L1024 [2]. Following Table 4 shows the results.

4.4.4.a : using diagonal candidates

4.4.4.b : using multiple distance lists

4.4.4.c : Combination of directional candidates and multiple distance lists

4.4.4.d : Combination of directional candidates and multiple distance list with full-pel position for large range

Table 4. All results CE4.4.4

4.4.4.a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM30-SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.07% | -0.03% | 103% | 100% |
| Class A2 | -0.12% | -0.04% | -0.06% | 106% | 100% |
| Class B | -0.15% | -0.04% | -0.15% | 107% | 100% |
| Class C | -0.17% | -0.12% | -0.06% | 109% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | -0.07% | -0.08% | 106% | 100% |
| Class D | -0.27% | -0.19% | -0.19% | 111% | 100% |
| Class F | -0.09% | -0.12% | -0.16% | 109% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM30-SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.14% | -0.21% | 103% | 100% |
| Class C | -0.05% | 0.21% | 0.16% | 104% | 100% |
| Class E | -0.06% | 0.08% | 0.14% | 103% | 100% |
| Overall | -0.05% | 0.03% | 0.00% | 104% | 100% |
| Class D | -0.15% | 0.09% | -0.07% | 105% | 100% |
| Class F | 0.14% | -0.07% | 0.06% | 104% | 100% |

4.4.4.b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM30-SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.04% | -0.11% | 102% | 99% |
| Class A2 | -0.07% | 0.00% | 0.05% | 103% | 100% |
| Class B | -0.03% | 0.08% | -0.03% | 103% | 100% |
| Class C | 0.05% | 0.04% | 0.12% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | 0.03% | 0.01% | 102% | 100% |
| Class D | 0.01% | -0.08% | 0.25% | 102% | 100% |
| Class F | 0.01% | 0.02% | 0.00% | 102% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM30-SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.12% | -0.07% | 101% | 100% |
| Class C | 0.07% | 0.25% | 0.35% | 101% | 100% |
| Class E | 0.15% | 0.22% | 0.08% | 101% | 101% |
| Overall | 0.09% | 0.09% | 0.11% | 101% | 100% |
| Class D | 0.10% | 0.16% | 0.67% | 101% | 100% |
| Class F | 0.18% | 0.08% | 0.06% | 101% | 99% |

4.4.4.c

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM30-SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.32% | -0.31% | -0.26% | 102% | 100% |
| Class A2 | -0.35% | -0.19% | -0.19% | 105% | 100% |
| Class B | -0.26% | -0.07% | -0.16% | 106% | 100% |
| Class C | -0.22% | -0.14% | -0.16% | 109% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.16% | -0.19% | 106% | 100% |
| Class D | -0.29% | -0.34% | -0.14% | 111% | 100% |
| Class F | -0.10% | -0.11% | -0.10% | 109% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM30-SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.03% | 0.28% | 103% | 100% |
| Class C | 0.01% | 0.22% | 0.26% | 105% | 100% |
| Class E | 0.12% | 0.31% | 0.20% | 103% | 100% |
| Overall | 0.01% | 0.14% | 0.25% | 104% | 100% |
| Class D | -0.10% | 0.16% | 0.34% | 105% | 100% |
| Class F | 0.03% | -0.13% | 0.06% | 104% | 100% |

4.4.4.d

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM30-SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.26% | -0.28% | 102% | 100% |
| Class A2 | -0.32% | -0.21% | -0.15% | 105% | 100% |
| Class B | -0.27% | -0.11% | -0.21% | 106% | 100% |
| Class C | -0.22% | -0.14% | -0.15% | 109% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.17% | -0.20% | 106% | 100% |
| Class D | -0.30% | -0.31% | -0.21% | 111% | 100% |
| Class F | -0.09% | -0.07% | -0.08% | 109% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM30-SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | 0.08% | 0.15% | 103% | 100% |
| Class C | -0.02% | 0.45% | 0.21% | 104% | 100% |
| Class E | 0.14% | 0.40% | -0.26% | 103% | 100% |
| Overall | 0.00% | 0.28% | 0.07% | 103% | 100% |
| Class D | -0.06% | 0.15% | -0.28% | 105% | 100% |
| Class F | 0.11% | -0.24% | -0.02% | 104% | 100% |

Conclusion

This contribution reports the results of CE4.4.4 enhanced MMVD. It is proposed to adopt test 4.4.4.d method into the next VVC draft / VTM.

References

[1] Tomonori Hashimoto, Eiichi Sasaki, Tomohiro Ikai, “Non-CE4: Enhanced ultimate motion vector expression,” JVET-L0355, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macau, CN, 3–12 October 2018.

[2] Haitao Yang, Shan Liu, Kai Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-L1024, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).