| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0066 |
| --- | --- |

| Title: | AHG12: Flexible Tile Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Tomohiro Ikai | Tel: Email: | +81-43-299-8526 yasugi.yukinobu@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

An updated version of Flexible Tile Partitioning for VTM-3.0. It is ported from JVET-L0359 version implemented on VTM-2.0 and added partial supports for new features in VTM-3.0. The experimental results with flexible tile partitioning which tile size unit is set to 16 reportedly show that luma BD-rate coding losses on average are 0.69%, 1.64% and 2.36% for All Intra, Random Access and Low Delay configuration respectively under the common test condition (CTC) for SDR sequences.

Introduction

In the previous documents (JVET-K0155[1] and JVET-L0359[2]), we introduced a tile splitting method with fine granularity unit, i.e. the smaller unit than CTU. The conventional tile segmentation by CTU sometimes has difficulty to fit frame packing boundaries or to do good load balancing because the unit of tile splitting is coarse. This difficulty may be more severe in VVC’s 128x128 (potentially 256x256) CTU size. In this contribution, an updated version of Flexible Tile Partitioning for VTM-3.0 is proposed.

Technical description

We propose a syntax, tile_size_unit_idc, which controls the unit of tiles. If tile_size_unit_idc is 0, 1, 2, .. the tile height and width can be 4, 8, 16, 32….

This tile_size_unit_idc decides the unit of column_width_minus1 and column_height_minus1 if uniform_spacing_flag is equal to 0), if uniform_spacing_flag is equal to 1, the width and height is decided depending on picture size and tile_size_unit_idc.

Figure 1 shows an example of the conventional (HEVC) 2x2 tiles where equal size splitting cannot be possible. Figure 2 shows an example of the proposed tile segmentation where the picture can be split into tiles of the same size. In the right and bottom edges of picture, there’s no CTU size  tile, which are effectively encoded and decoded as the same as in picture boundary with boundary splitting technique.

Figure 1: Example of 2x2 segmentation (conventional method)

Figure 2: Example of 2x2 segmentation (Proposal)

A modified working draft and source code are attached to the package of this contribution.

Experimental results

Experimental results for 2x2 tile partitioning with unit size = 16 are as follows:

Conclusion

An updated version of Flexible Tile Partitioning for VTM-3.0 is proposed. It is recommended to adopt the proposed method into the next version of WD and VTM.

References

[1] Y. Yasugi, T. Ikai, “AHG12: Flexible Tile Partitioning” JVET-K0155, Jul. 2018, Ljubljana, SI.

[2] Y. Yasugi, T. Ikai, “AHG12: Flexible Tile Partitioning” JVET-L0359, Oct. 2018, Macau, CN.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).