| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0069 |
| --- | --- |

| Title: | Non-CE4: Syntax change of MMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Eiichi Sasaki Takeshi Chujoh Tomohiro Ikai | Tel: Email: | +81-43-299-8526 eiichi.sasaki@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

In the current VTM-3.0, merge with MVD (MMVD)[1] shows gains in common test conditions (CTC)[2]. However the authors found that MMVD causes luma BD-rate loss of 0.21% / 0.95% in RA and LB conditions in high QP (42-57) range. The authors concluded this issue was caused by the current syntax structure of MMVD, which needs relatively large number of bins (5-bins) for the skip mode and merge mode.

This contribution proposes alternative syntax structure where the syntax elements of MMVD are signaled when skip_flag is equal to 0 and merge_flag is equal to 0, before the conventional AMVP motion vector difference. Experimental results shows -0.36%/-0.74% luma BD-rate change in RA and LB under high QP condition and -0.11%/-0.23% luma BD-rate change in RA and LB under CTC.

Problem Statement

Merge with MVD (MMVD)[1] has been adopted in the previous meeting. On current syntax structure, MMVD causes loss in high QP. It is necessary to signal over 5-bins to express MVD of MMVD, hence MMVD is not effective in high QP and in this case MMVD off flag may reduce the efficiency of the other skip and merge modes because the syntax elements of MMVD are signaled in skip or merge mode. The simulation results of VTM-3.0 without MMVD under high QP (42, 47, 52, 57) condition is shown in Table 1.

Table 1. Results of VTM-3.0 without MMVD under high QP condition

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.60% | -1.73% | -1.77% | 83% | 99% |
| Class A2 | -0.13% | -1.16% | -1.02% | 80% | 96% |
| Class B | -0.28% | -1.32% | -1.28% | 80% | 95% |
| Class C | 0.10% | -0.19% | -0.01% | 80% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.21% | -1.07% | -0.99% | 81% | 96% |
| Class D | 0.05% | -0.39% | -0.10% | 78% | 96% |
| Class F | -0.14% | -0.33% | -0.30% | 82% | 95% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.20% | -2.01% | -2.69% | 94% | 91% |
| Class C | -0.32% | -0.51% | -1.96% | 95% | 95% |
| Class E | -1.35% | -1.66% | -1.90% | 97% | 93% |
| Overall | -0.95% | -1.42% | -2.25% | 95% | 93% |
| Class D | 0.26% | -0.71% | -1.45% | 97% | 94% |
| Class F | -0.79% | -0.94% | -0.75% | 97% | 87% |

Proposed Syntax Structure

To suit MMVD to high QP as well as normal QP, we change the syntax structure. Proposed text is shown in the following.

Table 2. Proposed coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { /* MODE_INTER */ |  |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) { |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| merge_data( x0, y0, cbWidth, cbHeight ) |  |
| } else { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 ) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ] = 0 |  |
| } else { |  |
| mvd_coding( x0, y0, 1, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 1, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 1, 2 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( sps_amvr_enabled_flag && inter_affine_flag = = 0 && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 1 ] != 0 ) ) |  |
| amvr_mode[ x0 ][ y0 ] | ae(v) |
| if( sps_gbi_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && cbWidth * cbHeight >= 256 ) |  |
| gbi_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |
| } |  |

Table 3. Proposed merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mh_intra_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| mh_intra_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| mh_intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| mh_intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 16 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |

Experimental Results

The proposal is implemented on VTM-3.0 and simulated under high QP condition and CTC[2]. The results are shown in Table 4 and Table 5.

Table 4. Results of Proposed Syntax Structure under high QP condition

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.51% | -0.58% | -0.74% | 100% | 101% |
| Class A2 | -0.49% | -0.48% | -0.65% | 101% | 100% |
| Class B | -0.39% | -0.73% | -0.73% | 101% | 99% |
| Class C | -0.10% | 0.12% | -0.12% | 101% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | -0.42% | -0.55% | 101% | 99% |
| Class D | -0.14% | -0.51% | 0.02% | 101% | 96% |
| Class F | -0.22% | -0.27% | -0.31% | 100% | 95% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.73% | -0.91% | -1.43% | 102% | 93% |
| Class C | -0.56% | -0.25% | -0.54% | 102% | 95% |
| Class E | -0.99% | -1.21% | -1.68% | 102% | 92% |
| Overall | -0.74% | -0.77% | -1.19% | 102% | 94% |
| Class D | 0.30% | -0.31% | -0.31% | 104% | 95% |
| Class F | -0.53% | -0.64% | -1.58% | 101% | 89% |

Table 5. Results of Proposed Syntax Structure under CTC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.07% | 0.04% | 101% | 100% |
| Class A2 | -0.08% | 0.14% | 0.24% | 103% | 100% |
| Class B | -0.13% | 0.15% | 0.02% | 102% | 100% |
| Class C | -0.13% | 0.03% | 0.17% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | 0.07% | 0.11% | 102% | 100% |
| Class D | -0.12% | 0.19% | 0.16% | 103% | 101% |
| Class F | -0.09% | -0.03% | -0.02% | 105% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | -0.11% | -0.08% | 102% | 101% |
| Class C | -0.21% | 0.05% | -0.14% | 101% | 101% |
| Class E | -0.32% | -0.01% | -0.36% | 102% | 100% |
| Overall | -0.23% | -0.03% | -0.17% | 102% | 100% |
| Class D | -0.28% | 0.23% | 0.21% | 102% | 101% |
| Class F | -0.16% | 0.12% | -0.07% | 100% | 100% |

Conclusions

In this contribution, a new syntax structure of MMVD is proposed to suit high QP condition. Experimental results show that the proposed syntax structure improve coding efficiency under high QP condition and also under CTC. It is suggested to adopt proposed syntax structure into the next VVC working draft.

References

- M. W. Park, Y. Piao, M. Park, K. Choi, “CE4 Ultimate motion vector expression,” JVET-L0054, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).