| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0095-v1 |
| --- | --- |

| Title: | Non-CE3: Intra simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Adarsh K. Ramasubramonian Vadim Seregin Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121 | Email: | geertv@qti.qualcomm.com aramasub@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Firstly, it is proposed to reduce the number of interpolation filter phases from 32 to 16 for interpolation of the neighbouring reference samples used in intra prediction. It is asserted that this change halves the number of coefficients to be stored for the 4-tap filters. The reported BD-rate increase is 0.02% and 0.00% for the AI and RA configurations, respectively, while runtimes are unaffected. Secondly, two VVC draft text harmonizations are proposed with reported BD-rate changes of 0.00% and −0.01% for the AI and RA configurations, respectively, while runtimes are unaffected.

# Proposals

## 16-phase reference sample interpolation

In the third version of the VVC draft text, two 4-tap 32-phase interpolation filters are adopted for interpolation of the neighbouring reference samples used in intra prediction of luma blocks. Interpolation of chroma blocks uses 32-phase linear interpolation. It is proposed to reduce the number of interpolation phases from 32 to 16 for both the luma and chroma interpolation filters. This way, the number of coefficients to be stored for each 4-tap interpolation filter is reduced by half from 128 to 64. The proposed revision of the VVC draft 3 text is attached together with this document.

## Harmonization of directional intra modes

In the VVC draft 3 text, the directional intra modes 2, 34, 66 are treated separately from the other directional intra modes. The reason is that before adoption of new directional intra modes from proposal JVET-L0279, the square block diagonal modes 2, 34, and 66, were the only modes besides horizontal and vertical prediction that could be predicted using simple reference sample copying and did not require interpolation. However, JVET-L0279 introduced several additional directional modes that do not require interpolation, which resulted in the inconsistency in the VVC draft text. Therefore, it is proposed to process each directional mode consistently and employ the zeroth phase of the 4-tap interpolation filters. One implication is that PDPC is applied using unfiltered reference samples only. As reported in the results section, the BD-rate change is a negligible 0.00% and −0.01% for the AI and RA configurations, respectively.

The proposed change of paragraph 8.2.4.2.4 of the VVC draft 3 text is as follows:

“The variable filterFlag is derived as follows:

- If all of the following conditions are true, filterFlag is set equal to 1:

- refIdx is equal to 0

- nTbW * nTbH is greater than 32

- cIdx is equal to 0

- one or more of the following conditions is true:

- predModeIntra is equal to INTRA_PLANAR

- predModeIntra is equal to INTRA_ANGULAR34

- predModeIntra is equal to INTRA_ANGULAR2 and nTbH is greater than or equal to nTbW

- predModeIntra is equal to INTRA_ANGULAR66 and nTbW is greater than or equal to nTbH

- Otherwise, filterFlag is set equal to 0.”

And paragraph 8.2.4.2.7:

“The variable filterFlag is derived as follows:

If predModeIntra is equal to INTRA_ANGULAR2, INTRA_ANGULAR34 or INTRA_ANGULAR66 or refIdx is not equal to 0, filterFlag is set equal to 0.”

## Bit-exact harmonization of reference sample filtering decision for angular modes

It is proposed to modify one entry in Table 8-4 of the VVC draft 3 as follows:

|  | nTbS = 2 | nTbS = 3 | nTbS = 4 | nTbS = 5 | nTbS = 6 | nTbS = 7 |
| --- | --- | --- | --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 20 → 16 | 14 | 2 | 0 | 0 | 0 |

This change, which is bit-exact, harmonizes the filtering decision for the reference samples for all directional intra modes. The proposed change of paragraph 8.2.4.2.7 of the VVC draft 3 text is as follows:

“The variable nTbS is set equal to ( Log2 ( nTbW ) + Log2 ( nTbH ) )  >>  1.

The variable whRatio is set equal to Abs( Log2( nTbW / nTbH ) ).

The variable wideAngle is set equal to 0.

For non-square blocks (nTbW is not equal to nTbH), the intra prediction mode predModeIntra is modified as follows:

- If all of the following conditions are true, wideAngle is set equal to 1 and predModeIntra is set equal to ( predModeIntra + 65 ).

- nTbW is greater than nTbH

- predModeIntra is greater than or equal to 2

- predModeIntra is less than ( whRatio > 1 )  ?  ( 8 + 2 * whRatio )  :  8

- Otherwise, if all of the following conditions are true, wideAngle is set equal to 1 and predModeIntra is set equal to ( predModeIntra − 67 ).

- nTbH is greater than nTbW

- predModeIntra is less than or equal to 66

- predModeIntra is greater than ( whRatio > 1 )  ?  ( 60 − 2 * whRatio )  :  60

The variable filterFlag is derived as follows:

- If predModeIntra is equal to INTRA_ANGULAR2, INTRA_ANGULAR34 or INTRA_ANGULAR66 or refIdx is not equal to 0, filterFlag is set equal to 0.

- Otherwise, the following applies:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 8-4.

- The variable filterFlag is derived as follows:

- If minDistVerHor is greater than intraHorVerDistThres[ nTbS ] or wideAngle is equal to 1, filterFlag is set equal to 1.

- Otherwise, filterFlag is set equal to 0.”

Table 8-4 – Specification of intraHorVerDistThres[ nTbS ] for various transform block sizes nTbS

|  | nTbS = 2 | nTbS = 3 | nTbS = 4 | nTbS = 5 | nTbS = 6 | nTbS = 7 |
| --- | --- | --- | --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 20 16 | 14 | 2 | 0 | 0 | 0 |

# Results

The common test conditions specified in JVET-L1010 for SDR video and the guidelines described in the CE3 test description document JVET-L1023 are followed. Test results are reported for the AI and RA test configurations. The VTM3.0 software is used for implementing the tests.

## 16-phase reference sample interpolation

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.12% | 0.06% | 100% | 100% |
| Class A2 | 0.01% | 0.01% | 0.04% | 100% | 102% |
| Class B | 0.03% | 0.00% | -0.01% | 99% | 99% |
| Class C | 0.03% | -0.01% | -0.12% | 100% | 100% |
| Class E | 0.04% | 0.03% | 0.05% | 99% | 98% |
| Overall | 0.02% | -0.01% | 0.00% | 100% | 100% |
| Class D | 0.01% | 0.04% | 0.04% | 100% | 98% |
| Class F | 0.03% | 0.04% | 0.03% | 99% | 96% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | 0.06% | 98% | 98% |
| Class A2 | -0.02% | 0.03% | 0.15% | 99% | 99% |
| Class B | 0.00% | 0.11% | 0.08% | 99% | 99% |
| Class C | 0.02% | -0.05% | 0.23% | 98% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.13% | 99% | 98% |
| Class D | -0.03% | -0.44% | 0.05% | 98% | 96% |
| Class F | 0.02% | 0.05% | -0.04% | 99% | 99% |

## Harmonization of directional intra modes

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.10% | 100% | 99% |
| Class A2 | 0.00% | -0.05% | 0.05% | 101% | 101% |
| Class B | 0.01% | -0.04% | -0.02% | 99% | 99% |
| Class C | 0.00% | 0.02% | -0.15% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.08% | 99% | 98% |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.04% | 0.12% | 99% | 95% |
| Class F | 0.00% | 0.07% | 0.07% | 99% | 97% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.18% | 0.01% | 99% | 98% |
| Class A2 | -0.02% | -0.02% | 0.09% | 100% | 100% |
| Class B | -0.01% | 0.23% | 0.08% | 98% | 98% |
| Class C | -0.01% | -0.19% | 0.16% | 98% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.06% | 0.09% | 99% | 98% |
| Class D | 0.00% | -0.29% | 0.01% | 99% | 97% |
| Class F | -0.02% | 0.01% | 0.05% | 100% | 100% |

## Bit-exact harmonization of reference sample filtering decision for angular modes

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.000000% | 0.000000% | 0.000000% | 100% | 98% |
| Class A2 | 0.000000% | 0.000000% | 0.000000% | 100% | 99% |
| Class B | 0.000000% | 0.000000% | 0.000000% | 100% | 100% |
| Class C | 0.000000% | 0.000000% | 0.000000% | 101% | 102% |
| Class E | 0.000000% | 0.000000% | 0.000000% | 100% | 100% |
| Overall | 0.000000% | 0.000000% | 0.000000% | 100% | 100% |
| Class D | 0.000000% | 0.000000% | 0.000000% | 100% | 99% |
| Class F | 0.000000% | 0.000000% | 0.000000% | 100% | 99% |

## Combination of proposed simplifications

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.09% | 0.00% | 99% | 103% |
| Class A2 | 0.02% | 0.00% | 0.00% | 101% | 96% |
| Class B | 0.03% | -0.02% | 0.02% | 98% | 102% |
| Class C | 0.00% | 0.05% | -0.03% | 100% | 102% |
| Class E | 0.04% | -0.05% | 0.20% | 98% | 99% |
| Overall | 0.02% | -0.02% | 0.03% | 99% | 101% |
| Class D | 0.02% | -0.02% | 0.23% | 99% | 99% |
| Class F | 0.03% | -0.05% | 0.13% | 100% | 101% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.04% | 0.01% | 99% | 99% |
| Class A2 | -0.03% | 0.01% | 0.18% | 100% | 100% |
| Class B | -0.01% | 0.16% | 0.11% | 98% | 98% |
| Class C | 0.03% | -0.10% | -0.06% | 98% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.06% | 99% | 99% |
| Class D | -0.01% | -0.41% | -0.12% | 99% | 98% |
| Class F | 0.05% | -0.02% | 0.02% | 100% | 100% |

# Conclusion

Firstly, it is proposed to reduce the number of interpolation filter phases from 32 to 16 for interpolation of the neighbouring reference samples used in intra prediction, which halves the number of coefficients to be stored for the 4-tap filters. Secondly, two VVC draft text harmonizations are proposed. The BD-rate and runtimes changes are reported to be negligible. It is requested to adopt the proposed simplifications into VTM4.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).