| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0097-v1 |
| --- | --- |

| Title: | CE3: On MMLM (Test 2.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Geert Van der Auwera Ted Hsieh Vadim Seregin Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Email: | aramasub@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document reports the test results of multi-model LM (MMLM), sub-test 2.1 of CE3 Intra Prediction and Mode Coding, as described in JVET-L1023. In MMLM, two linear models are used for predicting the chroma samples from luma; the reconstructed luma samples are classified into two groups, and each model is applied to the respective group. The performance over the VTM-3.0 anchor reported for the various tests are as follows: BD-rate performance of (−0.25%, −1.71%, −2.19%) for (Y, Cb, Cr) with runtimes of 100% (Enc) and 100% (Dec) for the AI configuration and (−0.12%, −1.36, −1.70%) for (Y, Cb, Cr) with runtimes of 99% (Enc) and 99% (Dec) for the RA configuration.

# Introduction

The VVC Draft 3 describes the luma-to-chroma prediction mode, also referred to as CCLM mode, which constructs a linear prediction model between the luma and the chroma samples, as follows:

predCi, j=α·recL'i, j+ β

The reconstructed luma (down-sampled) and chroma samples from the top and left neighbouring blocks are used to derive the parameters (α and β) of the model at the encoder and the decoder. The min-max method (JVET-L0191) is used for the derivation of the CCLM parameters. The parameters are then used to derive the chroma sample prediction of the current CU from the reconstructed luma samples of the current CU. VVC Draft 3 describes three CCLM modes – CCLM-TL, CCLM-T and CCLM-L; in CCLM-TL mode, both left and top neighbouring reference samples are used for parameter derivation, whereas in CCLM-L and CCLM-T only the left neighbouring samples and the top neighbouring samples, respectively, are used for the derivation of the parameters of the model.

This CE test studies the performance of multi-model LM (MMLM) prediction of luma to chroma, where the reconstructed luma samples are classified into two classes, and one model each is derived and applied for the respective class.

# Proposal

In MMLM, two linear models are used to derive the chroma prediction samples.

Neighbouring luma samples and neighbouring chroma samples of the current block (aligned with CCLM and MDLM) are classified into two groups. Each group is used as a training set to derive a linear model (i.e., a particular α and β are derived for a particular group). This test re-uses the min-max method for parameter estimation (as used in VTM-3.0) and the downsampling filters used for the derivation of the linear model parameters in CCLM. Furthermore, the samples of the current luma block are also classified based on the same rule for the classification of neighbouring luma samples. Figure 1 shows an example of classifying the neighbouring samples into two groups based on the value Threshold. Threshold is calculated as the average value of the neighbouring reconstructed luma samples. A neighbouring sample with Rec′L[x,y] <= Threshold is classified into group 1; while a neighbouring sample with Rec′L[x,y] > Threshold is classified into group 2. For each group, the minimum and the maximum values of the luma samples, and their corresponding chroma samples are identified. The parameters α and β are derived for each group in both the encoder and decoder.

The reconstructed luma values are partitioned into two categories using Threshold: one model is applied for each category. During the reconstruction phase, the chroma prediction samples are derived from the luma reconstructed samples as follows:

Figure 1: An example of classifying the neighbouring samples into two groups

Currently in VTM3, there are three modes related to CCLM. In this proposal, three multi-model equivalent modes are included in the chroma mode list. The number of full RD checks are kept aligned with the VTM3 anchor.

# Complexity analysis vis-à-vis CCLM

The following section provides a comparison of the complexity of MMLM at the decoder vis-à-vis CCLM. CCLM in VTM3 computes the minimum luma value (Min1) and the maximum luma values (Max2) from the neighbouring samples, and uses those to compute the parameters of the CCLM model. In MMLM, two minimum values and maximum values are computed: minimum and maximum values of the first group (Min1 and Max1) and the second group (Min2 and Max2). The values Min1 and Max2 are the same across the two groups. Figure 2 shows the steps involved in the computation. It may be noted that the minimum, the maximum and the average of the downsampled luma values (in CCLM and MMLM) may be computed as soon as the neighbouring luma samples are available. The neighbouring chroma samples are only required for computing the α and β parameters.

Figure 2: Comparison of decoder operation for derivation of MMLM parameters compared to CCLM; Yavg is the Threshold value used to classify the two groups in MMLM

Finally, in the reconstruction phase, the downsampled luma at the decoder is compared with the threshold Yavg (in CCLM, this comparison step is absent) and the appropriate model parameters are chosen for deriving the chroma prediction sample.

Table 1: Comparison of operations for computing the min, max and avg. values for CCLM and MMLM.

|  | Comparisons | Additions | Divisions |
| --- | --- | --- | --- |
| CCLM | 2N (Min1, Max2) | 0 | 0 |
| MMLM | 4N = 2N (Min1, Max2) + N (demux) + N (Min2, Max1) | N−1 (Yavg) | 1 (table) |

# Results

The results of the proposed method on top of VTM3 are shown in Table 1. The BD-rate for the proposed method is (−0.25%, −1.71%, −2.19%) for (Y, Cb, Cr) with runtimes of 100% EncT and 100% DecT for the AI configuration and (−0.12%, −1.36%, −1.70%) for (Y, Cb, Cr) with runtimes of 99% EncT and 99% DecT for the RA configuration.

Table 2: Results for MMLM

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.73% | -2.81% | -4.63% | 99% | 102% |
| Class A2 | -0.07% | -0.97% | -0.82% | 101% | 103% |
| Class B | -0.20% | -1.99% | -2.38% | 99% | 99% |
| Class C | -0.28% | -1.94% | -2.39% | 100% | 100% |
| Class E | 0.02% | -0.61% | -0.50% | 99% | 97% |
| Overall | -0.25% | -1.71% | -2.19% | 100% | 100% |
| Class D | -0.08% | -1.24% | -1.25% | 100% | 100% |
| Class F | -0.37% | -1.82% | -2.39% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.40% | -1.96% | -3.48% | 98% | 99% |
| Class A2 | -0.01% | -0.75% | -0.58% | 101% | 102% |
| Class B | -0.06% | -1.60% | -1.78% | 99% | 97% |
| Class C | -0.06% | -1.07% | -1.08% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -1.36% | -1.70% | 99% | 99% |
| Class D | -0.01% | -1.13% | -0.64% | 100% | 100% |
| Class F | -0.13% | -1.03% | -1.34% | 99% | 99% |

# Acknowledgements

We would like to thank Foxconn for conducting a cross-check for this test.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).